3GPP TSG-RAN WG2 #129bis	R2-2502148

Wuhan, China, Feb 7th– 11st, 2025

Agenda Item:	8.5.2

Source:	Ericsson

Title:	Discussion on on-demand SSB for NES

Document for:	Discussion, Decision

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised [1]. The first objective concerns the on-demand SSB SCell operation, as shown below.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, SCell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

# 2	Configuration and signalling aspects

This contribution addresses the aspects concerning the activation/deactivation of on-demand SSB transmissions on SCells, configuration, signaling, L3 measurements, and SCell activation procedure.

## 2.1	MAC CE for on-demand SSB

RAN2 has discussed the MAC CE design in RAN2#127 and RAN2#127bis meetings and reached the following agreements:

RAN2#127 Agreements

OD-SSB transmission indication:

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

RAN2#127bis Agreements

OD-SSB transmission indication:

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

It is important to observe that in RAN1, the functional details on the MAC CE activation of the on-demand SSB are still open. Based on RAN1#120 agreements, it seems RAN1 has agreed on higher level field od-ssb-config which includes the OD-SSB parameters defining the OD-SSB to be activated. Note that also periodicity I included there, while earlier periodicity was considered by RAN1 to be separately controlled by the MAC CE.

Agreement

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2.

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2.

  - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency)

    - FFS: Whether this parameter can be absent for Case #2

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst)

    - FFS: Whether this parameter can be absent for Case #2

  - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity)

  - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing)

    - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB.

  - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId)

    - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB.

  - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex)

  - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower)

    - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx)

  - FFS: Which of the above parameters are a list

It is currently unclear which kind of functionality RAN1 is considering. The latest agreement on od-ss-Config field points to a design where there are N on-demand SSB configurations, configured via RRC signalling, with different parameter values and the MAC CE points out one of the configurations to be activated/deactivated. Yet another option is to have M “base configurations” configured by RRC and a few separate parameters to be selected/indicated directly by the MAC CE. Which way to design the MAC CE depends on the functionality. E.g. is it necessary to change a parameter value of already activated OD-SSB instead of simply reactivating a different od-ssb-Config – if a change of OD-SSB is needed.

It is assumed that when the UE receives a new MAC CE that activates a OD-SSB configuration which has e.g. N=8 repetitions, then it should replace any ongoing OD-SSB and the counter should be set to zero. In case there is a functional need to update e.g. periodicity but to keep the counter, then a separate field in the MAC CE(or even separate MAC CE is needed). Otherwise, it is more efficient and flexible to have all the degrees of freedom in the RRC. For example, if periodicity field is added separately but there is only three codepoints used, one codepoint is wasted. It also fixes those MAC CE bits to be used for periodicity only.

We suggest to ask RAN1 questions related to the OD-SSB details so RAN2 could receive the necessary details to progress the work on the OD-SSB MAC CE.

Questions to RAN1:

- What is the maximum number of different OD-SSB configs that can be configured per one SCell?

- Is there a functional need that a value of a parameter, like periodicity, needs to be explicitly indicated in the OD-SSB indication MAC CE? For example need to change a periodicity while one OD-SSB(and any counter related) is active?

There are many open questions in RAN1 that impact the MAC CE design for on-demand SSB.

Send LS to RAN1 about questions related to the MAC CE design.

# 3	L3 measurements

Another aspect that was discussed during RAN2 #127 meeting concerns the measurements on on-demand SSBs, and the conclusions are summarized in the following list of agreements:

RAN2#127 Agreements

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

R2-2407414: Proposal 3: RAN2 WG to discuss the issue of false measurement report       		 triggering due to no SSB transmission when on-demand SSB is deactivated.

- RAN2 does not handle this issue in OD-SSB

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Via its serving cell configuration, the UE has the configuration for on-demand SSB and receives the MAC CE for activation of the on-demand SSB reception. Since the UE has this information, it is not evident that any enhancement is needed for the L3 RRM configuration. In what follows, we summarize the status of RAN1 and RAN4 discussions that are relevant from the perspective of the measurements

RAN2#129 agreements related to SSB:

Agreements on OD-SSB

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

## 3.1	MAC CE based triggering for OD-SSB measurement

RAN4 has defined fast measurement framework for the OD-SSB for deactivated SCell. It means UE should measure the OD-SSB in a fast mode during a time window and after that UE may revert back to slow mode. Related agreements are as follows:

RAN4 #112bis

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1)

Agreements:

Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window.

- After the time window, UE can perform deactivated SCell measurement with a slow mode.

  - Option 2-1: fallback legacy requirement

  - Option 2-2: stop measurement

  - Option 2-3: No requirement

- Time window means the time duration starting from receiving OD-SSB activation indication

  - FFS the time window configuration or pre-defined

- FFS the fast L3 measurement requirement

- FFS the relation between time window and OD-SSB deactivation command in the time domain

When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

RAN4 #113

Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure

Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation?

Agreement:

- After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective.

- The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism

RAN4 #114

- Sub-issue 1-1 – The length of FMW

Agreement:

- The time window is defined with Tuncertain plus actual measurement duration.

  - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows.

  - The actual measurement duration equals min(Treport  , Tidentify) from the end of uncertain time.

    - Treport is the first OD-SSB based L3 measurement reporting

    - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow.

      - A detected cell: OD-SSB based measurement period

      - A newly detectable cell: OD-SSB based cell identification period

No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify).

Since UE is measuring efficiently the OD-SSB for a deactivated SCell, the measurement is useful only if also reported to the network. In legacy, a measurement for deactivated sCell can be reported as per SCell activation based on MAC CE. The deactivated SCell measurement requirement is defined in TS 38.133 v18.5.0 as follow.

Table 3: Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Table 4: Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(Mmeas_period_w/o_gaps x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

The deactivated SCell measurement cycle is configured in MeasObjectNR TS 38.331 v18.0.0 as follow.

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}

measCycleSCell The parameter is used only when an SCell is configured on the frequency indicated by the measObjectNR and is in deactivated state, see TS 38.133. gNB configures the parameter whenever an SCell is configured on the frequency indicated by the measObjectNR, but the field may also be signalled when an SCell is not configured. Value sf160 corresponds to 160 sub-frames, value sf256 corresponds to 256 sub-frames and so on.

To follow the existing framework and make use of the fast measurement framework designed by RAN4 we propose to support MAC CE based trigger for serving cell OD-SSB measurement report

.

RAN2 to support MAC CE based trigger of serving cell OD-SSB measurement report.

## 3.2	Measurement object configuration for OD-SSB

Furthermore, RAN1 agreement from RAN1#120 reveals that OD-SSB and AO-SSB can be on different frequencies but in same BWP:

Agreement

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

We assume RAN4 will continue to work on the measurement rules on which measurements can be combined and which measurements can be considered as intrafrequency and which measurements are considered as interfrequency[2]. Since the OA-SSB and OD-SSB are on same BWP although on different raster points, it seems natural to support the measurements by one measurement object.

RAN2 to agree to use the existing serving cell measurement object to support the serving cell measurements of OA-SSB and OD-SSB when these are within the same BWP

# 4	SCell activation procedure

SCell activation procedure has been discussed in RAN4 and the following agreements were reached during the RAN4#112bis meeting:

RAN4 #112bis Agreement

Issue 1-4-5: Which type of SSB is used during SCell activation

Agreement:

- In Case 1, UE is required to always use OD-SSB for Scell activation.

- In Case 2, when both OD-SSB and always-on SSB are transmitted,

  - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline

  - The agreement can be revisited if different with other WG

On this topic, RAN2 can discuss whether to confirm the RAN4 assumption that only on-demand SSB is used for SCell activation procedure when both always-on and on-demand SSBs are transmitted. In our opinion, there appear to be no RAN2 issues for UEs to combine always-on SSB and OD-SSB or to use only on-demand SSB for SCell activation procedures when the network transmits both always-on SSB and on-demand SSB. Since RAN4 had the disclaimer that their agreement is pending on the other WGs, we think that it would be beneficial to conclude in RAN2 whether the RAN4 assumption can be confirmed.

RAN2 to confirm that when network transmits both always-on SSB and on-demand SSB there appears to be no RAN2 issues for UEs to combine always-on SSB and OD-SSB or to use only on-demand SSB for SCell activation procedure.

# 5	Conclusion

In this contribution we discuss the second objective highlighted in the WID. Based on the discussion in the previous sections we made the following observations:

Observation 1	There are many open questions in RAN1 that impact the MAC CE design for on-demand SSB.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Send LS to RAN1 about questions related to the MAC CE design.

Proposal 2	RAN2 to support MAC CE based trigger of serving cell OD-SSB measurement report.

Proposal 3	RAN2 to agree to use the existing serving cell measurement object to support the serving cell measurements of OA-SSB and OD-SSB when these are within the same BWP

Proposal 4	RAN2 to confirm that when network transmits both always-on SSB and on-demand SSB there appears to be no RAN2 issues for UEs to combine always-on SSB and OD-SSB or to use only on-demand SSB for SCell activation procedure.

# References

RP-242354, “Revised WID: Enhancements of network energy savings for NR”, RAN#105, October 2024.

R4-xxxxx, “Discussion on OD-SSB in Rel-19 NES”, RAN4#114 Wuhan, China Ericsson

# Appendix

3GPP TSG RAN WG2 #129bis		R2-24xxxx

Wuhan, China, April 7th – 11th, 2025

Title:	LS on MAC CE on OD SSB

Response to:

Release:	Rel-19

Source:	RAN2, Ericsson

To:	RAN1

Cc:

Contact person:	Helka-Liina Määttänen

Helka-liina.maattanen@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN2 has been discussing the OD-SSB MAC CE and would like to ask RAN1 details related to the design in order to progress the MAC CE design in RAN2..

Questions to RAN1:

- What is the maximum number of different OD-SSB configs that can be configured per one SCell?

- Is there a functional need that a value of a parameter, like periodicity, needs to be explicitly indicated in the OD-SSB indication MAC CE? For example need to change a periodicity while one OD-SSB(and any counter related) is active?

# 2	Actions

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to respond the above questions.

# 3	Dates of next TSG RAN WG 2 meetings

TSG RAN WG2 Meeting #130                     	19 to 23, May 2025			Malta, MT

TSG RAN WG2 Meeting #131                          25 to 29, Aug. 2025			Bangalore, India