3GPP TSG-RAN WG2 Meeting #129bis	R2-2502194

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.13.3

Source:	Nokia

Title:	Control plane and SRAP procedures for Multi-hop realy

WID/SID:	NR_SL_relay_multihop – Release 19

Document for:	Discussion and Decision

# 1	Introduction

This Tdoc discuss the open issues on control plane and SRAP for Multi-hop relay

# 2	Control plane and SRAP

When it comes to SI acquisition there are two semi-related issues for the intermediate relay UE(s) being whether an intermediate relay UE;

- can acquire the SI directly from the gNB instead of the parent intermediate relay UE/last UE,

- may autonomously forward the SI information when requested by PC5-RRC

The intermediate UE, when it receives SI from a parent relay, forwards the SI message only to the child UEs which requested the SI (i.e., the intermediate UE keeps track of the required SI for each child node).  FFS on the need for including a “remote UE ID” in the request.

The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list.  FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

Agreements:

When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain the SI required by it or requested by the remote UE by requesting SI from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink).  FFS if it can also receive it directly from SIB broadcast by the last relay UE’s serving cell on Uu (when the intermediate relay UE is in coverage of the same cell that serves the last relay UE).

As a consequence of approach 1, the intermediate UE can be viewed as both a relay and a remote UE, thus it may be allowed to utilise both specified procedures i.e. either obtain directly from serving cell, or through parent relay UE. This would also have the advantage that we don’t assert artificial requirements towards the intermediate relay UE which may cause additional overhead.

Proposal 1: An intermediate relay UE can acquire SI from both the last relay UEs serving cell, if applicable, as well as through PC5-RRC from the parent relay UE.

In legacy relay, the SIB the relay UE acquires the requested SIB and forwards it to the remote UE. The question is how the intermediate UE is allowed to acquire i.e. which method the intermediate UE can acquire the SI for the remote UE. In order to reduce overhead, it would be reasonable to allow an

Proposal 2: The intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: An intermediate relay UE can acquire SI from both the last relay UEs serving cell, if applicable, as well as through PC5-RRC from the parent relay UE.

Proposal 2: The intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.