3GPP TSG-RAN WG2 Meeting #129bis	R2-2502200

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.13.2

Source:	Nokia

Title:	Open issues on Relay discovery and (re)selection

WID/SID:	NR_SL_relay_multihop - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution discuss open aspects of Relay discovery and (re)selection

# 2	Relay discovery and (re)selection

In RAN2 it was agreed that at least for the baseline procedure i.e. approach 1, the last relay UE cannot be operated as an intermediate relay UE towards a different cell. The reason behind this decision is that it would require the UE from camping on camping on two different cells i.e. its own serving cell and the serving cell for the potentially other intermediate remote UE. Furthermore, the operation may be similar to that of CA which is not included within the scope of the WID

The last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell.  FFS if there can be exceptions for control plane approach 2 when the intermediate relay UEs are in idle/inactive.

In terms of the FFS on approach 2, the reasoning is that the UE is not camping on a cell, and thus it need not to be concerned about any serving cell. However, even when being in RRC_IDLE/RRC_INACTIVE, the UE is still required to comply to the configuration of both cells and thus the operation does not come for free, as the UE will need to potentially find at least the intersecting configuration of the two different cells and apply that configuration.

Additionally, when forwarding the SI to the child UE, there will be no ambiguity w.r.t. which configuration needs to be forwarded.

Observation 1: It is not trivial to specify the UE behaviour even in the case a UE is in RRC_IDLE/RRC_INACTIVE on one cell and needs to provide relay service of another cell.

Proposal 2: Even if approach 2 is agreed, The last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell.

# 3	Conclusion

This document has made the following observations and proposals:

Observation x: It is not trivial to specify the UE behaviour even in the case a UE is in RRC_IDLE/RRC_INACTIVE on one cell and needs to provide relay service of another cell.

Proposal x: Even if approach 2 is agreed, The last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell.