3GPP TSG-RAN WG2 Meeting #129bis	R2-2502217

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:		8.2.4

Source:		Futurewei

Title:		Further discussions on A-IoT data transmission and other aspects

Document for:		Discussion and Decision

# Introduction

In RAN2#129, the following was agreed [1]:

- Agreements on segmentation

- To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

- R2D segmentation is not supported for R19 A-IoT.

In this contribution, we further discuss some detailed design aspects to support segmentation.

# Discussions

## Comparison of 1-bit Segmentation Indication (SI) vs. 2-bit SI

Should a 2-bit SI need to be considered, using the definition of the SI field in the RLC header as a reference, we can define the 2-bit SI for Ambient IoT as in Table 1 below.

Table 1. Definition of 2-bit SI

| Binary Value | Description |
| --- | --- |
| 00 | Current transmission contains all bits of an SDU |
| 01 | Current transmission contains the first segment of an SDU |
| 10 | Current transmission contains the last segment of an SDU |
| 11 | Current transmission contains neither the first nor the last segment of an SDU |

Using similar language, we can define the 1-bit SI for Ambient IoT as in Table 2 below, which complies with the current agreement on the 1-bit indication.

Table 2. Definition of 1-bit SI

| Binary Value | Description |
| --- | --- |
| 0 | Current transmission contains the last segment of an SDU |
| 1 | Current transmission does not contain the last segment of an SDU |

In meeting #127bis, RAN2 also agreed on the following [2]:

- Do not support sequence number for A-IoT segmentation functionalities.

- Do not support segment number and the number of segments for A-IoT segmentation functionalities.

These two agreements were reached based on the understanding that (re)transmissions from a same Ambient IoT device use a stop-and-wait protocol with N=1, i.e., the maximal number of pending (re)transmission is 1, or in other words, parallel transmission processes, as in LTE and NR, are not supported for Ambient IoT devices. Therefore, from a receiver’s perspective, the reader can determine whether a current D2R transmission contains all bits of an SDU, the first segment of a (segmented) SDU, the last segment of a (segmented) SDU, or neither the first nor the last segment of a (segmented) SDU, even if the 1-bit SI, as described in Table 1, is used, e.g., based on the SI in the D2R transmission currently received and another SI in the prior D2R transmission received. The complexity involved in checking the 1-bit SIs in two consecutive D2R transmissions is not significant to a reader, whether the reader being a gNB reader or a UE reader. Therefore, there is no significant advantage of using 2-bit SI over using 1-bit SI, even if the CN cannot provide an estimated expected D2R message size to the reader. Hence, we propose that RAN2 no longer consider the 2-bit SI design, even if the CN cannot provide an estimated expected D2R message size to the reader.

### Observation 1. There is no significant advantage of using 2-bit SI over using 1-bit SI, even if the CN cannot provide an estimated expected D2R message size to the reader.

### Proposal 1. The 1-bit SI is to be introduced, no matter whether the CN can provide an estimated expected D2R message size or not. RAN2 no longer considers the 2-bit SI design.

## Presence of the SI field

In meeting #127bis, RAN2 also agreed on the following [2]:

- RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

This assumption means that segmentation can be supported on a D2R transmission only if the D2R transmission is a D2R data transmission that carries an AIoT NAS PDU, because according to SA2, AIoT NAS layer is the upper layer immediately above the AIoT MAC layer. D2R data transmissions should include D2R transmissions that carry Msg3 because the device ID in Msg3 is carried as a part of the Inventory Response, which is an AIoT NAS PDU according to SA2. Otherwise, if a D2R transmission carries only AS information without carrying an AIoT NAS PSU, segmentation cannot be supported on it, because the AS information, such as RN16, AS ID, ACK/NACK, are not stored in the AIoT NAS layer. Besides, the chance that these AS information will be oversized and hence requiring segmentation is very low. Therefore, we propose that segmentation is supported only on D2R data transmissions (including Msg3) that carry an AIoT NAS PDU.

Observation 2. No AS layer buffering for A-IoT segmentation functionalities means that segmentation can be supported on a D2R transmission only if the D2R transmission is a D2R data transmission (including MSg3) that carries an AIoT NAS PDU.

Proposal 2. Segmentation is supported only on D2R data transmissions (including Msg3) that carry an AIoT NAS PDU.

Therefore, the SI field is not present in a D2R transmission not carrying an AIoT NAS PSU and may be optionally present in a D2R data transmission (including Msg3) that carries an AIoT NAS PSU. Furthermore, if Proposal 1 is agreeable, the SI is only 1 bit. If the 1-bit SI field is optionally present in a D2R data transmission, a 1-bit flag is still needed to indicate the presence or absence of the SI field, resulting in at least the same number of bit(s) in total, if not more, compared to making the 1-bit SI field a mandatory field in all D2R data transmissions without a need of the flag bit.

Observation 3. If the 1-bit SI field in optionally present in a D2R data transmission, a 1-bit flag is still needed to indicate the presence or absence of the SI field, resulting in at least the same number of bit(s) in total, if not more, compared to making the 1-bit SI field a mandatory field in all D2R data transmissions without a need of the flag bit.

Proposal 3. The SI field is a mandatory field in D2R data transmissions (including Msg3) that carry an AIoT NAS PDU.

## Location of the SI field

From the viewpoints of PDU construction and signaling overhead, it may be advantageous to have all mandatory and fixed-length fields at the beginning of a MAC PDU to make it easy for bit(s) retrieval. If Proposals 1 to 3 are agreeable, the SI field is such a mandatory and fixed-length field in all D2R data transmissions. Therefore, it may be advantageous to place the SI field in the early portion of the MAC PDU, e.g., as the first field after the Message Type field when the Message Type field indicating a message type corresponding to D2R data transmission.

In addition, RAN1 has agreed on the following [3]:

Agreement RAN1 concludes that the generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC are reused for Ambient IoT. Agreement When CRC is attached to a PRDCH or PDRCH transmission, - When the number of information bits is ≤ X bits, CRC-6 is used. Otherwise, when the number of information bits is > X bits, CRC-16 is used. Down-selection by RAN1#120bis from the following for X considering the balance of overhead and probability of undetected error: - Alt. 1: 24 - Alt. 2: 56 - FFS impact of segmentation, if any - Note: impact may not be in RAN1

Due to a shorter length of the check value, CRC-6 has worse (1000x worse) undetected error rate performance than CRC-16. For example, if a D2R data message consists of 100 bits and is segmented during the D2R transmission, 80 bits may be transmitted in the first segment. Hence, a CRC-16 is used for the first segment. The remaining 20-bit is transmitted as the last segment and hence a CRC-6 is used for the second segment. The reassembled message will have the 1000x worse error detection performance of CRC-6 despite CRC-16 being used on the other segment. A solution to avoid this performance degradation is that CRC-16 is always used on a D2R data transmission containing the last segment of a segmented SDU no matter how many bits are included in that D2R data transmission. Having the SI field in a fixed location in the MAC PDU can help the physical layer processing to easily retrieve the SI bit without a need to understand the MAC PDU format or other contents except the SI bit.

Proposal 4. CRC-16 is used on a D2R data transmission containing the last segment of a segmented SDU no matter how many bits are included in that D2R data transmission.

Proposal 5. The SI field is placed in the early portion of the MAC PDU of D2R data transmissions (including Msg3) that carry an AIoT NAS PDU, e.g., being the first field after the Message Type field when the Message Type field indicating a message type corresponding to D2R data transmission.

RAN2 could send an LS to RAN1, informing them additional progress, if any, made by RAN2 on segmentation in meeting#129bis, to assist them in their study on the impact of segmentation.

Proposal 6. RAN2 send an LS to RAN1, informing them additional progress, if any, made by RAN2 on segmentation.

# Conclusions

### Observation 1. There is no significant advantage of using 2-bit SI over using 1-bit SI, even if the CN cannot provide an estimated expected D2R message size to the reader.

Observation 2. No AS layer buffering for A-IoT segmentation functionalities means that segmentation can be supported on a D2R transmission only if the D2R transmission is a D2R data transmission (including MSg3) that carries an AIoT NAS PDU.

Observation 3. If the 1-bit SI field in optionally present in a D2R data transmission, a 1-bit flag is still needed to indicate the presence or absence of the SI field, resulting in at least the same number of bit(s) in total, if not more, compared to making the 1-bit SI field a mandatory field in all D2R data transmissions without a need of the flag bit.

Based on these observations, we propose the following:

### Proposal 1. The a-bit SI is to be introduced, no matter whether the CN can provide an estimated expected D2R message size or not. RAN2 no longer considers the 2-bit SI design.

Proposal 2. Segmentation is supported only on D2R data transmissions (including Msg3) that carry an AIoT NAS PDU.

Proposal 3. The SI field is a mandatory field in D2R data transmissions (including Msg3) that carry an AIoT NAS PDU.

Proposal 4. CRC-16 is always used on a D2R data transmission containing the last segment of a segmented SDU no matter how many bits are included in that D2R data transmission.

Proposal 5. The SI field is placed in the early portion of the MAC PDU of D2R data transmissions (including Msg3) that carry an AIoT NAS PDU, e.g., being the first field after the Message Type field when the Message Type field indicating a message type corresponding to D2R data transmission.

Proposal 6. RAN2 send an LS to RAN1, informing them additional progress, if any, made by RAN2 on segmentation.

# References

R2_129_ChairNotes_25-02-21_final.

R2_127bis_ChairNotes_24-10-18_13-00.

R1-250xxxx Chair notes RAN1#120 (9.4 R19 Ambient IoT) v08.