3GPP TSG-RAN WG2#129-bis	R2-2502226

Wuhan, China, April. 7th – 11th, 2025

Source: 	Fujitsu

Agenda item:	8.5.3

Title:	Remaining issues on on-demand SIB1 procedure

Document for:	Discussion and decision

# Background

In RAN2#129 meeting, RAN2 made some agreements as follows [1].

Agreements on OD-SIB1

- There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- SIB-x can be cell specific or area specific, as legacy.

- Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

In this contribution, we would like to discuss and provide our views on the remaining on-demand SIB1 issues.

# Discussion

## UL WUS procedure

### UL WUS configuration

UL WUS configuration by RRC release message

In RAN2#126 meeting, the following issue was discussed and RAN2 agreed whether or not to adopt this as an option in stage 3.

2.	RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3

In RAN2#128 meeting, RAN2 discussed and reached “UL-WUS configuration in RRC release is not supported” as a WA. Because some companies think it is beneficial in terms of reducing broadcast signalling. But if the NW does not broadcast the UL WUS configuration, the NW cannot update UL WUS configuration after the NES UE is in the idle/inactive mode. In addition, the NES UE re-selected from a neighboring cell to Cell A does not have a valid UL WUS configuration. So, it imposes another mechanism to provide the UL WUS configuration, e.g., periodical SI update. Therefore, it is suspicious this option can reduce overhead of broadcast signalling for UL WUS configuration.

Proposal 1:	RAN2 to confirm the working assumption, i.e. UL-WUS configuration in RRC release is not supported.

In RAN1#120 meeting, RAN1 concluded it is up to RAN2 whether the value in OD-SIB1 IE can be different from the value of the corresponding IE in the UL WUS configuration as follows.

Conclusion

It’s up to RAN2 to discuss whether the value in OD-SIB1 IE can be different from the value of the corresponding IE in the WUS configuration, and how to define the UE behavior if the values are different.

According to the RAN1 agreements, the UL WUS configuration could include different values for the same IEs provided in SIB1 of the NES cell. However, in the current specification, the UE simply applies or forwards the received IEs in SIB1 whenever it is acquired. Hence, there is no impact on the UE behaviour even if the value in OD-SIB1 IE can be different from the value of the corresponding IE in the UL WUS configuration.

Proposal 2:	RAN2 to confirm there is no impact on the UE behaviour even if the value in OD-SIB1 IE can be different from the value of the corresponding IE in the UL WUS configuration.

### UL WUS transmission

RRC connected mode

In RAN#106 meeting, the following way forward was endorsed for RRC connected mode [2].

On RRC Connected mode - The scope of WID will not be updated to include RRC Connected. - However, RAN2 will address the specific case of RLF, to use the same procedure as idle/inactive to request OD-SIB1 transmission. No need to change the scope of WI to indicate this.

Based on the above RAN plenary guideline, SIB1 request by a dedicated signaling in connected mode (i.e., RRCSystemInfoRequest message) is not pursued.

Observation 1:	SIB1 request by a dedicated signaling in connected mode is not pursued.

In RAN2#129 meeting, RAN2 made some agreements to support SIB1 request from RRC connected mode. However, if the searchSpaceSIB1 is configured and SIB1 is not broadcasting in RRC connected mode, it was FFS whether it relies on the NW implementation to deliver SIB1 or not. In our understanding, such a configuration can be avoided by NW implementation. For example, the NES UE may initiate RRC establishment procedure to enter RRC connected mode after on-demand SIB1 reception. Or the NES UE in connected mode may be moved to the NES cell by handover. In both cases, as the NW can provide dedicated SIB1 via RRCReconfiguration message, it could be completely handled by NW implementation and no optimization is needed (e.g., enhancement of RRCSystemInfoRequest message to request SIB1).

Proposal 3:	For Rel-19 NES UE in RRC connected mode, RAN2 does not optimize the case where the searchSpaceSIB1 is configured but SIB1 is not broadcasting.

Cell selection criterion S

During timer T311 is running, the NES UE can send UL WUS to the candidate NES cell if the NES cell is satisfied the legacy cell selection criteria. One remaining issue on this procedure, the NES UE may not have the cell selection information before checking cell selection criterion S. For example, the cell selection criterion S is calculated by the following formula [3]:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset ) – Pcompensation – Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) – Qoffsettemp

In a case of cell reselection, i.e., to evaluate the neighbour cells, the above parameters can be provided by SIB2, SIB3 and SIB4 in the serving cell. For example, Qqualmin and Qrxlevmin are defined in SIB2 in TS 38.331 as follows:

q-QualMin

Parameter "Qqualmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin.

q-RxLevMin

Parameter "Qrxlevmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells.

Whereas in a case of cell selection, because it is not neighbour cells, the UE will need to acquire these parameters directly from SIB1 therefore the UE cannot evaluate the cell selection criterion S for the OD-SIB1 cell. In addition, RAN1 agreed: “Cell selection information is not included in the UL-WUS configuration”, thereby there is no mechanism to provide cell selection information to the NES UE before RLF is detected. To resolve the issue, the simplest way is reusing the existing cell reselection parameters included in Cell A for OD-SIB1 cell selection during timer T311 is running.

Proposal 4:	When timer T311 is running, the cell selection criterion S is evaluated by the existing cell reselection parameters provided from Cell A.

A cell supporting OD-SIB1

In the POST e-mail discussion on the MAC CR, it is FFS whether there is a need to clarify that a preamble can be transmitted and RAR received on “SpCell or a cell supporting SIB1 request” instead of just “SpCell”.

In the current MAC running CR:

1) In Section 5.1.1:

-	ra-ResponseWindow: the time window to monitor RA response(s) (SpCell only);

2) In Section 5.1.3:

2>	else:

3>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

3>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

4>	if the Random Access Preamble is transmitted on the SpCell:

5>	indicate a Random Access problem to upper layers;

5>	if this Random Access procedure was triggered for SI request or SIB1 request:

6>	consider the Random Access procedure unsuccessfully completed.

3) In Section 5.1.4:

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

…

2>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

3>	if the Random Access Preamble is transmitted on the SpCell:

4>	indicate a Random Access problem to upper layers;

4>	if this Random Access procedure was triggered for SI request or SIB1 request:

5>	consider the Random Access procedure unsuccessfully completed.

In our understanding, when the UE is camping on Cell A and requesting OD-SIB1 of an NES cell, the UE’s SpCell is Cell A but not the NES cell. In this case, UL WUS (preamble) is transmitted on the NES cell and Random access response is received on the NES cell, but not on the UE’s SpCell. Therefore, for more accurate procedure, it is suggested to change “SpCell” to “SpCell or a cell supporting OD-SIB1”.

Proposal 5:	A preamble can be transmitted and RAR received on “SpCell or a cell supporting SIB1 request” instead of just “SpCell”.

### UL WUS acknowledgement reception

RAR MAC PDU for SIB1 request ACK

When an NES UE camping on Cell A is requesting OD-SIB1 from an NES cell, the NES UE receives SIB1 request ACK in the Random Access Response MAC PDU transmitted by the NES cell.

In RAN1#118 meeting, RAN1 has agreed shared RO or separated RO related to UL WUS transmission as follows:

Agreement For further work on on-demand SIB1 in idle/inactive mode related to dedicated PRACH resource usage, RAN1 to study the following options: - Option 1 (shared RO): The dedicated WUS resource shares the same PRACH resource pool with PRACH resource for other usages. - Option 2 (separated RO): The dedicated WUS resource uses an independent RACH resource pool with PRACH resource for other usages.

In RAN1#118-bis meeting, RAN1 has further agreed it is up to gNB whether RO for UL WUS is shared or separated for other usages as follows:

Agreement It is up to gNB to configure whether RACH occasions for UL WUS are shared or separated from the RACH occasions for other usages.

In case of shared RO among UL WUS and other RA preambles, the Random Access Response MAC PDU transmitted by the NES cell may include SIB1 request ACK for the NES UE and SI request ACK and/or RAR for other UEs in the NES cell. The agreed UL WUS configuration does not include preamble configuration for SI request in the NES cell, so the UE might not know the configured preambles for SI request or RAR other than SIB1 request in NES cell. If the SIB1 request ACK for the NES UE is after an SI request ACK (1-byte long) or RAR (1+7=8 byte long) for other UEs in the Random Access Response MAC PDU, the NES UE might not know the start position of its SIB1 request ACK in the MAC PDU, as shown in Figure 1.

Figure 1. Example of RAR MAC PDU for SIB1 request ACK

In Figure 1, if the NES UE detects the RAPID in a MAC subPDU is not for SIB1 request ACK, the NES UE does not know whether the MAC subPDU is for SI request ACK or for RAR. Then the NES UE has problem to find the start position of the next MAC subPDU and could not find whether the SIB1 request ACK is present in the MAC PDU.

To resolve this issue, in our view, the MAC subPDU with SIB1 request ACK can be placed after the MAC subPDU with BI (if present) and before the MAC subPDU with SI request ACK and the MAC subPDU with RAR, so the NES UE camped on Cell A can stop decoding Msg2 when the NES UE detects the SIB1 request ACK without knowing the preambles used for SI request in the NES cell.

Proposal 6: The MAC subPDU with SIB1 request ACK is placed right after the MAC subPDU with BI (if present) and before the MAC subPDU with SI request ACK and the MAC subPDU with RAR in the MAC PDU for Random Access Response.

In the POST e-mail discussion on the MAC CR, whether to change a handling preamble backoff during SIB1 request procedure was raised. Because the NES UE uses a dedicated preamble for SIB1 request, there is no contention between preambles. So, the NES UE can ignore the backoff indicator provided by the RAR MAC subPDU. However, in general, it happens in the legacy SI request procedure and RAN2 has agreed such a handling is not needed in RAN2 NR-AH#2 meeting [4].

Agreements for Msg1 based SI request method:

7:	Back off is applicable for Msg1 based SI requests but no special Back off subheader/ procedure is required.

Proposal 7:	No need to change the preamble backoff handling in the RAR MAC PDU during SIB1 request procedure.

### On-demand SIB1 reception

On-demand SIB1 update in NES cell

In RAN2#127 meeting, it was agreed that UE keeps SIB1 updated while on the NES cell via regular SI modification procedure. An issue that can be further discussed is how long the network should transmit the on-demand SIB1 from the network modification period, as shown in the following figure.

Figure 2. On-demand SIB1 update in the NES cell

In our view, the network can transmit the on-demand SIB1 during the whole next modification period (i.e., legacy SI update), and the UE can assume the updated SIB1 is transmitted from the next modification period. If the UE is unable to acquire the updated SIB1, the UE should consider the NES cell as barred, because the UL WUS configuration may be obsoleted due to SI update.

Proposal 8:	The updated SIB1 is transmitted during whole next modification period.

Proposal 9:	The UE considers the NES cell as barred if the UE is unable to acquire the updated SIB1.

On-demand SIB1 transmission status

In RAN1#120 meeting, it was discussed how/whether to check the on-demand SIB1 transmission status of the NES cell before the UL WUS transmission, based on the following agreement in RAN2#127-bis meeting.

If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Currently there are five options for monitoring type-0 PDCCH for SIB1 and it seems that RAN1 would try to select one out of these options [5]. It is FFS in the running RRC CR but RAN2 impacts will depend on the RAN1 conclusion. It is difficult to discuss this aspect by RAN2 only thus it is preferred to wait for further RAN1 progress.

Proposal 10:	RAN2 to wait for RAN1 progress how to check the on-demand SIB1 is already transmitted if the UE has a valid UL WUS configuration.

# Conclusion

In this contribution, the following proposals and observations were made:

UL WUS configuration by RRC release message

Proposal 1:	RAN2 to confirm the working assumption, i.e. UL-WUS configuration in RRC release is not supported.

Proposal 2:	RAN2 to confirm there is no impact on the UE behaviour even if the value in OD-SIB1 IE can be different from the value of the corresponding IE in the UL WUS configuration.

RRC connected mode

Proposal 3:	For Rel-19 NES UE in RRC connected mode, RAN2 does not optimize the case where the searchSpaceSIB1 is configured but SIB1 is not broadcasting.

Cell selection criterion S

Proposal 4:	When timer T311 is running, the cell selection criterion S is evaluated by the existing cell reselection parameters provided from Cell A.

A cell supporting OD-SIB1

Proposal 5:	A preamble can be transmitted and RAR received on “SpCell or a cell supporting SIB1 request” instead of just “SpCell”.

RAR MAC PDU for SIB1 request ACK

Proposal 6: The MAC subPDU with SIB1 request ACK is placed right after the MAC subPDU with BI (if present) and before the MAC subPDU with SI request ACK and the MAC subPDU with RAR in the MAC PDU for Random Access Response.

Proposal 7:	No need to change the preamble backoff handling in the RAR MAC PDU during SIB1 request procedure.

On-demand SIB1 update in NES cell

Proposal 8:	The updated SIB1 is transmitted during whole next modification period.

Proposal 9:	The UE considers the NES cell as barred if the UE is unable to acquire the updated SIB1.

On-demand SIB1 transmission status

Proposal 10:	RAN2 to wait for RAN1 progress how to check the on-demand SIB1 is already transmitted if the UE has a valid UL WUS configuration.

# References

- R2-2501331, Report from session on V2X/SL, R19 NES and MOB

- RP-243297

- TS 38.304

- R2-1707602, Report of 3GPP TSG RAN WG2 NR AdHoc#2 meeting

- R1-2501381