3GPP TSG-RAN WG2 Meeting #129bis			         R2-2502228

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.4.3

Source: 	LG Electronics Inc.

Title:         	Remaining issues on measurement offloading and relaxation

Document for: 	Discussion and Decision

Introduction

In this contribution we discuss the remaining issues on the RRM measurement offloading and relaxation.

Discussion

- Case#1. Measurement offloading

RAN2#127 Agreements:

- For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

  - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

  - The exit condition is based on the LR measurement results.

According to RAN2 and RAN4 agreements so far, if the entry condition for RRM offloading is met, the MR can sleep more by skipping the serving cell measurements. However, while MR stays awake for performing SDT or receiving MBS session, the UE can perform the serving cell measurements using MR without additional power consumption. On the contrary, performing LR measurements while MR is awake would results in additional power consumption. Therefore, UE should not perform the RRM measurement offloading to LR while performing SDT or receiving MBS session, even if the entry condition for RRM measurement offloading is met.

Proposal 1	[For case #1] Even if the entry condition for RRM offloading is met, UE does not perform the measurement offloading to LR while receiving MBS or performing SDT.

Currently, the serving cell measurements using MR is used for the following purposes in RRC_IDLE/INACTIVE:

- Determining whether the cell (re-)selection criterion is met

- Determining whether to perform the neighbor cell measurements based on the measurement rules

- Deciding whether to perform RA on NUL or SUL

- Determining whether to trigger RRC resume while receiving multicast session in RRC_INACTIVE

- Determining whether the SDT condition is met

RAN2 needs to discuss and determine which of these purposes the LR measurements can or cannot be used for. For the purposes where the LR measurements are deemed applicable, new LR measurement-based condition/threshold may be introduced.

The intra-frequency and equal priority inter-frequency cell reselection is performed based on comparing the ranking of serving cell and neighbour cells. If the cell reselection criterion based on the LR measurement is permitted, UE will select the highest ranked cell by comparing the LR measurements and neighbor cell measurements based on MR, and the UE may select incorrect cell due to the relatively inaccurate LR measurement.

The mobility performance should not be compromised for power saving, and the mobility decision should be made based on reliable MR measurements. Therefore, the LR measurements based cell reselection criterion should not be allowed under the assumption that the threshold for measurement offloading is always higher than the threshold for skipping intra-frequency measurement.

Proposal 2	[For case #1] Do not support the LR measurements based cell reselection criterion.

If the proposal 2 is accepted, the measurement offloading to LR will be allowed only for UEs located closer to the cell centre, where the probability of cell reselection is low. Meanwhile, deciding whether to use SUL and whether to measure neighbor cells are needed only near the cell boundary. Therefore, the separate condition based on the LR measurements seems not needed for them.

- Case#3. Measurement relaxation

RAN2#129 Agreements:

- The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements.

- If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

While performing the measurement relaxation, both MR measurements and LR measurements are available, so a separate exit condition is not needed. As in R16 RRM relaxation, the UE can stop the measurement relaxation if the entry condition is no longer met.

Proposal 3	[For case #3] No separate exit condition for measurement relaxation. If the entry condition for measurement relaxation is not met, UE stops the measurement relaxation.

Conclusion

Proposal 1	[For case #1] Even if the entry condition for RRM offloading is met, UE does not perform the measurement offloading to LR while receiving MBS or performing SDT.

Proposal 2	[For case #1] Do not support the LR measurements based cell reselection criterion.

Proposal 3	[For case #3] No separate exit condition for measurement relaxation. If the entry condition for measurement relaxation is not met, UE stops the measurement relaxation.