3GPP TSG-RAN WG2 Meeting #129bis	           R2-2502242

Wuhan, China, April 7th – April 11th, 2025

Agenda item:	8.10.4

Source:	China Telecom

Title:	SON of intra-NTN mobility

Document for:	Discussion

- Introduction

RAN3 have made some progress of SON/MDT for intra-NTN mobility scenarios. In this paper, we mainly discuss MRO for intra-NTN mobility aspects.

- Discussion

RAN3 have the following agreements related to MRO for intra-NTN mobility:

RAN3#124: MRO for intra-NTN mobility shall consider the following trigger conditions: - Measurement-based trigger condition; - A time-based trigger condition; - A location-based trigger condition; - A time-based and Measurement-based combined trigger condition; - A location-based and Measurement-based combined trigger condition; Do not consider MRO for feeder link switch. MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover. RAN3#125: MRO for CHO with a time-based trigger condition: Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell. Distinguish if the RLF happens before or after T1 Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell. RAN3#125bis: For location based trigger condition: Too late CHO Execution: RLF happens in the source cell before CHO execution. The UE re-establishes in a cell other than the source cell. Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell. For earth fixed case, there is no need to report the measured distance from the UE. FFS on the earth moving case. For joint trigger condition: Case 1: Too Late CHO Execution: RLF happens in the source cell. The UE re-establishes in the cell other than the source cell. Case 2: Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens in the target cell short after successful HO. The UE re-establishes in the source cell or a third cell. UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not. RAN3#126: WA: There is no need to report any other info in the RLF report for the location based CHO. RAN3#127: Time based CHO: Existing timers can be reused to identify when RLF happens with 100ms accuracy. FFS whether more accuracy is needed. No need to identify whether RLF happens before or after T1 for time-based CHO. Location based CHO: Revert the WA: There is no need to report any other info in the RLF report for the location based CHO. For earth moving case, the accuracy and complexity issue of the network-based solution is acknowledged. - Option 1: to introduce the absolute UTC time of when RLF/HOF happens in the RLF report. - Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell (when RLF occurred). FFS on option 1 or option 2. MRO mechanism for intra-NTN mobility shall be enhanced to enable handling of local and temporary propagation changes (e.g. due to clouds, rain etc.)?

Regarding MRO for CHO with a time-based trigger condition, RAN3 agreed that existing timers can be reused to identify when RLF happens with 100ms accuracy while whether more accuracy is needed is FFS. The existing RLF report includes timeSinceFailure, timeConnFailure and timeSinceCHO-Reconfig. As pointed out by companies, the issue with reusing these timers for MRO for intra-NTN mobility is that the t1-Threshold defined in TS 38.331 is in 10ms units while the granularity of the existing timers in RLF report is in seconds or 100ms. If timeSinceCHO-Reconfig is used to distinguish if the RLF happens before or after t1-Threshold, it would result in a wrong conclusion of comparing t1-Threshold and (the time CHO config is received + timeSinceCHO-Reconfig) in the network, because the network can’t know the true time of timeSinceCHO-Reconfig in 10ms, i.e. the reported value is 200ms, but the true time may be 150ms, 160ms, 170ms, 180ms or 190ms.

On the other hand, the network need deduce the time when CHO config is received by the UE based on the absolute time when the network sent CHO config and propagation delay, which introduce additional uncertainty. It is beneficial to have the absolute time or the relative time between t1-Threshold and the time RLF occurs in the RLF report, which provides additional information, i.e. the reference for the adjustment amount of t1-Threshold if the failure is caused by improper T1 setting. Furthermore, reporting the relative time between t1-Threshold and the time RLF occurs has less overhead than the absolute time. We suggest to introduce the relative time between t1-Threshold in the RLF report.

Proposal 1: Regarding MRO for CHO with a time-based trigger condition, include relative time between the time RLF occurs and t1-Threshold in the RLF report.

As for case 2 in case of CHO with a time-based trigger condition, if the UEs report the absolute time when CHO execution failed or relative time between CHO execution failure and t1-Threshold, the network can figure out the adjustment amount of t1-Threshold if the failure is caused by improper T1 setting. Considering the overhead, we suggest to introduce the relative time in the RLF report.

Proposal 2: Regarding MRO for CHO with a time-based trigger condition, include relative time between CHO execution failure and t1-Threshold in the RLF report.

RAN3 agreed to select one from option 1 and option 2 to resolve the accuracy and complexity issue of the network-based solution for earth moving case. From our point of view, for the UE configured with condEvent D2 (i.e. Distance between UE and the serving cell moving reference location is above threshold1 and distance between UE and a moving reference location is below threshold2), it is natural that the UE need to measure the distance from the UE to moving reference location of serving cell and the distance from the UE to moving reference location of candidate cell. Thus it is straightforward to include measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF occurred in the RLF report (i.e. option 2), without additional effort.

Proposal 3: Regarding MRO for CHO with a location-based trigger condition, include measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF occurred in the RLF report.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: Regarding MRO for CHO with a time-based trigger condition, include relative time between the time RLF occurs and t1-Threshold in the RLF report.

Proposal 2: Regarding MRO for CHO with a time-based trigger condition, include relative time between CHO execution failure and t1-Threshold in the RLF report.

Proposal 3: Regarding MRO for CHO with a location-based trigger condition, include measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF occurred in the RLF report.