3GPP TSG-RAN WG2 Meeting #129bis	R2-2502251

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.1.3

Source:	InterDigital Inc.

Title:	NW side data collection

Document for:	Discussion

# 1. Introduction

Several agreements regarding network side data collection were made in previous RAN2 meetings (e.g., RAN2-126bis [1], RAN2-127 [2], RAN2-17bis [3], RAN2-128 [4], and RAN2-129 [5]). The Annex lists the agreements regarding network side data collection during these meetings.

In this contribution, we address some of the open issues.

# 2. Discussion

## 2.1 Availability Indication

In RAN2-127bis [3], it was agreed that:

The UE can indicate the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication. and FFS how data availability indication is sent to the network.

In RAN2-129, it was further agreed that [5]:

- Data availability indication is supported.  FFS when this would be triggered

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

Currently, the UE-MeasurementsAvailable IE is used to indicate all relevant indicators for UE measurements (e.g.., logged MDT measurements, RLF reports, etc.,) and can be included in different RRC Complete messages (e.g., RRC Reconfiguration/Setup/Resume). It is rather straightforward to extend the same IE to indicate also the availability of logged measurements for a training of a network sided model.

Proposal 1: The UE can (opportunistically) indicate the availability of logged data using an RRC Reconfiguration Complete message (e.g., via a new indication within the UE-MeasurementsAvailable-r19 IE).

In RAN2-127 [2], it was agreed that:

- When UE reaches its buffer limitation, the UE stops measurement for data collection purposes and logging.

In RAN2-128 [4], it was further agreed that:

- The UE reports to the network when buffer is or may become full.  FFS when it reports (before and/or after).

Based on the above agreements shown above, it naturally follows that the UE stopping the data collection/logging due to buffer limitation should be one trigger condition for availability indication, instead of waiting for the next opportunity to indicate the availability in an RRC reconfiguration complete message, as the UE will not be able to collect more data during this time. A more proactive approach is to make it possible to configure the UE to send the availability indication even before the buffer is full, so that the collected data could be retrieved before the buffer is full and data collection will not be interrupted.

Proposal 2: The UE can be configured with a buffer level threshold (as a percentage of the total buffer level) and it will send an availability indication when the collected data reaches this buffer size (e.g., via a UAI message).

If the UE is configured to perform data collection for different use cases simultaneously, it is important for the UE to indicate in the availability indication information regarding the data collection configuration that the availability indication is corresponding to.

Proposal 3: If the UE is configured to perform data collection for different use cases simultaneously, the UE will include, in the availability indication, information about the corresponding data collection configuration or use case. How the data collection configuration is indicated (e.g., configuration ID, functionality/sub-functionality ID, etc.,) is FFS.

## 2.2 Sending the collected data

It was agreed in RAN2-127bis [3] that UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. Currently, the network sends to the UE a UEInformationRequest message indicating the type of logged measurements that it wants the UE to send, and the UE includes the requested data/measurements using corresponding fields, as shown below:

UEInformationRequest-r16-IEs ::= SEQUENCE {

idleModeMeasurementReq-r16       ENUMERATED{true}                         OPTIONAL, -- Need N

logMeasReportReq-r16             ENUMERATED {true}                        OPTIONAL, -- Need N

connEstFailReportReq-r16         ENUMERATED {true}                        OPTIONAL, -- Need N

ra-ReportReq-r16                 ENUMERATED {true}                        OPTIONAL, -- Need N

rlf-ReportReq-r16                ENUMERATED {true}                        OPTIONAL, -- Need N

mobilityHistoryReportReq-r16       ENUMERATED {true}                        OPTIONAL, -- Need N

lateNonCriticalExtension         OCTET STRING                             OPTIONAL,

nonCriticalExtension             UEInformationRequest-v1700-IEs           OPTIONAL

}

UEInformationRequest-v1700-IEs ::= SEQUENCE {

successHO-ReportReq-r17          ENUMERATED {true}                        OPTIONAL, -- Need N

coarseLocationRequest-r17        ENUMERATED {true}                        OPTIONAL, -- Need N

nonCriticalExtension             UEInformationRequest-v1800-IEs           OPTIONAL

}

UEInformationRequest-v1800-IEs ::= SEQUENCE {

flightPathInfoReq-r18            FlightPathInfoReportConfig-r18           OPTIONAL, -- Need N

successPSCell-ReportReq-r18      ENUMERATED {true}                        OPTIONAL, -- Need N

reselectionMeasurementReq-r18    ENUMERATED {true}                        OPTIONAL, -- Need N

nonCriticalExtension             SEQUENCE {}                              OPTIONAL

}

UEInformationResponse-r16-IEs ::=    SEQUENCE {

measResultIdleEUTRA-r16              MeasResultIdleEUTRA-r16             OPTIONAL,

measResultIdleNR-r16                 MeasResultIdleNR-r16                OPTIONAL,

logMeasReport-r16                    LogMeasReport-r16                   OPTIONAL,

connEstFailReport-r16                ConnEstFailReport-r16               OPTIONAL,

ra-ReportList-r16                    RA-ReportList-r16                   OPTIONAL,

rlf-Report-r16                       RLF-Report-r16                      OPTIONAL,

mobilityHistoryReport-r16            MobilityHistoryReport-r16           OPTIONAL,

lateNonCriticalExtension             OCTET STRING                        OPTIONAL,

nonCriticalExtension                 UEInformationResponse-v1700-IEs     OPTIONAL

}

UEInformationResponse-v1700-IEs ::=  SEQUENCE {

successHO-Report-r17                 SuccessHO-Report-r17                OPTIONAL,

connEstFailReportList-r17            ConnEstFailReportList-r17           OPTIONAL,

coarseLocationInfo-r17               OCTET STRING                        OPTIONAL,

nonCriticalExtension                 UEInformationResponse-v1800-IEs     OPTIONAL

}

UEInformationResponse-v1800-IEs ::=  SEQUENCE {

flightPathInfoReport-r18             FlightPathInfoReport-r18            OPTIONAL,

successPSCell-Report-r18             SuccessPSCell-Report-r18            OPTIONAL,

measResultReselectionNR-r18          MeasResultIdleNR-r16                OPTIONAL,

nonCriticalExtension                 SEQUENCE {}                         OPTIONAL

}

Thus, there is a need to introduce a new IE/field to indicate the network is requesting a logged data for network side model training. As discussed in section 2.1, if the UE is configured to perform data collection for different use cases simultaneously, it is important for the network to also indicate which collected data is to be reported.

Proposal 4: A new IE/field is introduced to the UEInformationRequest message to indicate the network is requesting the UE to send logged data for network sided model training.

Proposal 5: If the UE is configured to perform data collection for different use cases simultaneously, the network includes, in the UEInformationRequest message, information about the corresponding data collection configuration or use case. How the data collection configuration is indicated (e.g., configuration ID, functionality/sub-functionality ID, etc.,) is FFS.

Similarly, a new IE/field needs to be introduced in the UEInformationResponse that will be used to include the relevant collected data.

Proposal 6: A new IE/field is introduced to the UEInformationResponse message that will be used to contain the logged data.

## 2.3 Data storage duration and handling during state transitions

In legacy NR, the UE is allowed to release any logged measurement (e.g., RLF report, logged MDT report, etc.,) after 48 hours. We believe it is necessary to specify a similar maximum duration of time that the UE is mandated to keep data logged for network side model training. The exact time duration can be decided later in the normative phase once we get more idea about the expected size of the logged data (e.g., if the logged data is large then the time duration needs to be short).

Proposal 7: The UE keeps the logged data that is not sent to the network for at least a specified time duration. The value of the time duration (e.g., 48 hours as in the case of logged MDT or a smaller/larger value) is FFS.

In relation to the time duration for keeping the logged data, it is also important to discuss whether the UE keeps the logged data during transition back and forth between CONNECTED and IDLE/INACTIVE.

In RAN2-129, it was agreed [5]:

- UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

Considering that the inactivity timer for deciding the transition to IDLE/INACTIVE is up to network implementation, and it can be very short (e.g. in few minutes or even seconds), it is reasonable to assume that the duration for keeping the log is expected to be longer than that. Also, the UE may experience an RLF (and trigger re-establishment) while collecting the data or after the data collection is finished but before the data was sent to the network.

Thus, we propose to keep the same approach as for legacy logged measurements such as logged MDT and RLF reports, i.e., UE keeps the logged data across state transitions or re-establishment, at least until the time duration for keeping the logs has expired.

Proposal 8: The UE maintains the logged data upon transitioning to IDLE/INACTIVE or during RLF detection/recovery.

Proposal 9: The UE indicates availability of collected data upon transitioning back to CONNECTED state in RRCSetupComplete/RRCResumeComplete/RRCReestablishmentComplete messages.

## 2.4 Content of the collected data

In RAN2#127bis, the following was agreed regarding the content of the collected data [3]:

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

NR supports logging of time information in the logged MDT case, as shown below:

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

LogMeasInfoList-r16 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16    OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16        OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL,

...,

[[

inDeviceCoexDetected-r17             ENUMERATED {true}                   OPTIONAL

]]

}

As can be seen above, there is one absolute time information corresponding to the absolute time when the logged measurement configuration was provided, and for each collected sample, there is a relative timestamp from this absolute timestamp. A similar approach can be adopted for the logged measurements for network side data collection

Proposal 10: Time information for the collected data is kept in the same way as logged MDT:

- an absolute time corresponding to the time the data collection configuration was received, and

- for each collected sample, a relative time from the absolute time

As discussed in section 2.3, it has been agreed that the UE retains logged data during HO. It is still not clear how the UE behaves if the UE is configured to also log data in the target cell, before it has sent the collected data in the previous cell to the network. Even though there could be a way for the network to figure out the collected data in one cell from the sample timestamps (e.g., based on when the UE was handed over to the target cell), it is a rather complicated solution. A more scalable approach, that will also allow a flexible deployment of the model training entity anywhere in the network, is include the associated ID and cell ID information in the logged data. It is not essential to include this information in every sample, but rather within a given log, the UE may have collected data partitioned for each cell, with the corresponding cell ID and associated ID for that cell during the data collection.

Proposal 11a: If data collection across multiple cells in a given data collection session is to be supported (i.e., data collected from multiple cells is to be sent in the same report), the UE will include the following information in the collected data:

- serving cell ID (FFS if PCI is sufficient or CGI is required)

- network side additional condition for the serving cell (e.g., associated ID)

Proposal 11b: The collected data from multiple cells is partitioned by the serving cell (i.e., no need to repeat cell ID/associated ID information with each sample).

# 4. Conclusion

In this contribution, we discussed the open issues regarding logging and reporting of data for the training of a network-sided model for the beam management use case, and the following proposals were made:

Proposal 1: The UE can (opportunistically) indicate the availability of logged data using an RRC Reconfiguration Complete message (e.g., via a new indication within the UE-MeasurementsAvailable-r19 IE).

Proposal 2: The UE can be configured with a buffer level threshold (as a percentage of the total buffer level) and it will send an availability indication when the collected data reaches this buffer size (e.g., via a UAI message).

Proposal 3: If the UE is configured to perform data collection for different use cases simultaneously, the UE will include, in the availability indication, information about the corresponding data collection configuration or use case. How the data collection configuration is indicated (e.g., configuration ID, functionality/sub-functionality ID, etc.,) is FFS.

Proposal 4: A new IE/field is introduced to the UEInformationRequest message to indicate the network is requesting the UE to send logged data for network sided model training.

Proposal 5: If the UE is configured to perform data collection for different use cases simultaneously, the network includes, in the UEInformationRequest message, information about the corresponding data collection configuration or use case. How the data collection configuration is indicated (e.g., configuration ID, functionality/sub-functionality ID, etc.,) is FFS.

Proposal 6: A new IE/field is introduced to the UEInformationResponse message that will be used to contain the logged data.

Proposal 7: The UE keeps the logged data that is not sent to the network for at least a specified time duration. The value of the time duration (e.g., 48 hours as in the case of logged MDT or a smaller/larger value) is FFS.

Proposal 8: The UE maintains the logged data upon transitioning to IDLE/INACTIVE or during RLF detection/recovery.

Proposal 9: The UE indicates availability of collected data upon transitioning back to CONNECTED state in RRCSetupComplete/RRCResumeComplete/RRCReestablishmentComplete messages.

Proposal 10: Time information for the collected data is kept in the same way as logged MDT:

- an absolute time corresponding to the time the data collection configuration was received, and

- for each collected sample, a relative time from the absolute time

Proposal 11a: If data collection across multiple cells in a given data collection session is to be supported (i.e., data collected from multiple cells is to be sent in the same report), the UE will include the following information in the collected data:

- serving cell ID (FFS if PCI is sufficient or CGI is required)

- network side additional condition for the serving cell (e.g., associated ID)

Proposal 11b: The collected data from multiple cells is partitioned by the serving cell (i.e., no need to repeat cell ID/associated ID information with each sample).

# 5. References

[1]	RAN2-126 Chairman Notes, Fukuoka, Japan, May 2024

[2]	RAN2-127 Chairman Notes, Maastricht, The Netherlands, August 2024

[3]	RAN2-127bis Chairman Notes, Hefei, China, October 2024

[4]	RAN2-128 Chairman Notes, Orlando, USA, November 2024

[5]	RAN2-129 Chairman Notes, Athens, Greece, February 2025

# 6. Annex (Agreements regarding network side data collection)

RAN2-126 agreements [1]:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting.

RAN2-127 agreements [2]:

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network.

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event-based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

RAN2-127bis agreements [3]:

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicate the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

RAN2-128 agreements [4]:

Agreements on NW side data collection: - Focus on the following three radio condition event-based logging: - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2) - Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value) - L1 beam level measurement - Measurements on aperiodic CSI resources are not reported for NW sided data collection. - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

RAN2-129 agreements [5]:

Agreements on NW side data collection: - Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported.FFS what to log - Low power bit indication is supported - Data availability indication is supported. FFS when this would be triggered - As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT. - UE retains logged data during handover (HO). FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed - UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE). - FFS how to handle idle/inactive and RLF cases and whether we have a unified.