3GPP TSG-RAN WG2 Meeting #129bis	R2-2502252

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.1.4

Source:	InterDigital Inc.

Title:	Data Collection for UE sided Model Training

Document for:	Discussion

# 1. Introduction

The issue of data collection for UE sided model training was discussed in RAN-106 and a way forward was agreed in [1]:

- During Rel-19 RAN2 continues studies on AIML UE sided data collection (excluding 1b)

  - Study RAN aspects related to data transfer over UP

  - Discuss level of NG-RAN involvement in the control and configuration of UE side data collection.

  - Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion).

  - Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain.

  - Discuss on the scalability aspects of CP

  - RAN2 can Liaise with other WGs if/when needed

In this contribution, we address some of the issues that are still open.

# 2. Discussion

## 2.1 Data collection

As extensively discussed in [3] and [4], the data collection configuration for the training of a UE sided model can be provided by the RAN for some use cases (e.g., beam management), while for other uses, it may come from outside the RAN (e.g., LMF for the positioning use case). Our understanding is that even in the latter cases where the configuration is coming from outside the RAN (e.g. LMF), some level of RAN awareness and control is required to make this that the performance of the UE will not be impacted due to measurements due to data collection.

The content/format of the collected data and data visibility has been discussed in previous RAN2 meetings, and we have stated in our response to SA2 the following [4]:

- Standardized data refers to data whose format/content is explicitly defined in 3GPP specifications, allowing the network to understand its content and meaning.

- As stated in the LS sent from RAN, visibility of data content signifies that the MNO will be able to be aware of, access, and comprehend the content of the collected/reported data without the need of SLA. Thus, full visibility allows the MNO to verify/match the data specified/configured to be collected and the data that is being reported.

Thus, our understanding is that for each use case, the content/format of the data/measurements to be collected will be specified. Any UE vendor specific data can be transferred as well (e.g., in a transparent container) along with the specified data.

Proposal 1: The format of the data/measurement to be collected will be specified per use case basis. UE vendor-specific/proprietary data can be included in a transparent container along with the standardized data.

The topic of data collection for network side model training has been extensively discussed in the previous RAN2 meetings (e.g., [5][6]) and our understanding is that we can adopt some of the agreements also for the UE sided data collection. For example, it has been agreed that both periodic and event based logging will be supported. We do not see any reason why the same principle cannot be applied to the collection of the data for the UE side model training. That is, the entity that is providing the data collection configuration could also configure the UE to log the data periodically and/or based on the fulfillment of some events. These aspects are transparent to the RAN.

Another issue we have addressed in the network sided data collection discussion is buffer management: when the UE reaches its buffer limitation, it stops the logging of the measurements [4]. Our understanding is that the same principle applies for the collection of data for UE sided models (i.e., UE has a limited buffer capacity for collecting the data and will stop the logging when the buffer capacity is reached).

Proposal 2: When the UE reaches its buffer limitation, it stops the logging of the measurements (as in the case of network side data collection)

## 2.2 Data transfer

For transferring the collected data via user plane, some consideration should be given regarding the QoS requirements for such a bearer. In the context of UE sided data collection for AIML model training, our understanding is that there are no strict latency or bit rate requirements as the training is expected to be performed offline. However, since the UE will have a limited space for logging the data, the requirements may change when the UE reaches its buffer limitation. For example, if proposal 3 above is agreed and the UE stops the data collection upon reaching its buffer limitation, the UE will not be able to collect any more data until the collected data is sent out and the buffer space is reclaimed (or the UE is configured to discard older data to accommodate for newer data samples). That is, when the buffer limitation is reached, the bearer may need to be considered at a higher priority than when the buffer utilization is very low.

Thus, to accommodate this dynamic requirements of the transfer of the collected data and for future proofness of data collection needs for other use cases other than AIML model training, having a dedicated bearer that may have dynamic characteristics is recommendable. Whether there is a need to have dedicated bearers for each data collection configuration/use case requires further discussion. Other aspects that need discussion are on how the dynamic requirements of such a bearer will impact user plane procedures such as scheduling and buffer status reporting. Similar mechanisms as the availability indication that was agreed for network side data collection could also be considered.

Proposal 3: For the transfer of collected data via user plane, the UE can be configured with a dedicated bearer.

Proposal 4: RAN2 to discuss the following aspects in relation to the dedicated bearer for the transfer of collected data for UE side model training:

- QoS requirements/profile of the bearer

- if one bearer can be used for all data collection transfer or if there is a need for a dedicated bearer for each data collection configuration/use case

- impacts on user plane procedures such as buffer status reporting and scheduling

Current NR UP bearers are terminated at the UPF. If the total amount of collected data that is going to be transferred over the network is considerable, routing all the data via the UPF may overload the UPF and direct routing to the destination server/entity may be considered. Aspects such as the PDU session for data collection (e.g., whether a different PDU session is required for data collection, or the same PDU session as other UP data can be used) also need to be discussed. However, all these are outside the scope of RAN2 and should be studied in SA groups.

Proposal 5: Aspects such as the end-to-end PDU session and (dedicated)bearer setup for the transfer of collected data over UP and the routing of the collected data from UE/gNB to the data collection server at CN/OAM should be discussed first in SA groups.

# 3. Conclusion

In this contribution, we discussed the open issues regarding data collection for UE sided model training and transfer of the collected data over the UP, and the following proposals were made:

Proposal 1: The format of the data/measurement to be collected will be specified per use case basis. UE vendor-specific/proprietary data can be included in a transparent container along with the standardized data.

Proposal 2: When the UE reaches its buffer limitation, it stops the logging of the measurements (as in the case of network side data collection)

Proposal 3: For the transfer of collected data via user plane, the UE can be configured with a dedicated bearer.

Proposal 4: RAN2 to discuss the following aspects in relation to the dedicated bearer for the transfer of collected data for UE side model training:

- QoS requirements/profile of the bearer

- if one bearer can be used for all data collection transfer or if there is a need for a dedicated bearer for each data collection configuration/use case

- impacts on user plane procedures such as buffer status reporting and scheduling

Proposal 5: Aspects such as the end-to-end PDU session and (dedicated)bearer setup for the transfer of collected data over UP and the routing of the collected data from UE/gNB to the data collection server at CN/OAM should be discussed first in SA groups.

# 4. References

[1]	RP-243307, Way forward on AIML UE sided data collection, RAN2 Chair (Interdigital), RAN-106, Madrid, Spain, Dec 2024

[2] 	R2-1279 Chairman Notes, Athens, Greece, Feb 2025

[3]	R2-2410504, [POST127bis][020][AI PHY] Reply LS to SA2/SA5 (InterDigital/Nokia), Orlando, USA, November 2024

[4]	R2-2411152, Reply LS to SA2 on AIML data collection, Interdigital, RAN2-128, Orlando, USA, November 2024

[5]	R2-127bis Chairman Notes, Hefei, China, October 2024

[6]	R2-128 Chairman Notes, Orlando, USA, November 2024