3GPP TSG-RAN WG2 Meeting #129bis	R2-2502253

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.3.2.2.

Source:	InterDigital Inc.

Title:	Generalization of AIML models for RRM measurement prediction

Document for:	Discussion

# 1. Introduction

In RAN2-129, model generalization for speed and frequency were discussed in [1]. As can be seen the agreements in Annex A.1, model generalization was shown to work across all speeds (slightly better for GC#2 than GC#1) and also performs well for generalization in frequency (for GC#2, if knowledge about the input/output frequency is employed).

In this contribution, we provide further results on generalization across these two sets of network deployment configurations [2] as shown in Table 1, where priority is given to the FR1 case.

Table 1: Network Deployment Configurations for Generalization Study

| Parameter | Configuration #1 (UMi) | Configuration #2 (UMa) |
| --- | --- | --- |
| Deployment Scenario | UMi | UMa |
| ISD | 200 m | 500 m |
| BS Antenna height | 10 m | 25 m |
| BS Tx Power | 40 dBm | 44 dBm |

# 2. Discussion

### 2.1 Intra-cell intra-frequency FR1 to FR1 temporal domain case B

To generate data, we consider a sampling period of 40 ms for L1 beam-level RSRPs. Then, non-sliding L1 filtering is applied to obtain L1 filtered beam-level RSRPs. With L1 filtering window length that is equal to 5, the sampling period of non-sliding L1 filtered beam-level RSRPs, and consequently L3 cell-level RSRPs is 200 ms. It is worth noting that L1 filtering is a simple averaging process of the input L1 beam-level RSRPs.

Figure 1: Non-sliding L1/L3 Filtering

We consider 50% measurement reduction rate as shown in Figure 2. For predicting cell-level RSRPs at time instance T, we use input from 3 previous time instances, which are T-5, T-3, and T-1. It is necessary to use input from multiple time instances to enable the AI/ML model to learn information about UE trajectory. The RSRPs at time instances T-5, T-3, and T-1 are all measured RSRPs.

In Case 2, we consider predicting L3 cell-level RSRPs directly according to Sub-case 2 where:

- Sub-case 2 (Cell-to-cell Direct Approach): L3 cell-level RSRPs from 3 time instances (T-5, T-3, and T-1) are input to AI/ML model to predict L3 cell-level RSRP at time instance T directly.

Figure 2: Temporal-domain Case B Observation Windows (OWs in yellow) and Prediction Windows (PW in green) Pattern with 50% Measurement Reduction Rate

To study the capability of the AI/ML model to generalize to different deployment configurations (i.e., UMi and UMa), we consider the following generalization cases:

- Baseline: The AI/ML model is trained using the dataset of Configuration #A and tested using the dataset of Configuration #A.

- Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset of Configuration #B and tested using the dataset of Configuration #A, or vice versa.

- Generalization Case #2 (GC#2): The AI/ML model is trained using a mixed dataset with examples from both datasets of Configuration #A and Configuration #B and tested using the dataset of Configuration #A. It is worth noting that the mixed dataset used to train the mixed AI/ML model has the same size as the dataset of Configuration #A (or Configuration #B) for fair comparison.

Simulation parameters of Case 2 are listed in Table A.1 [3]. Evaluation and AI/ML model parameters are shown in Table 2. Generalization results are shown in Table 3.

Table 2: Evaluation and AI/ML model parameters of intra-cell intra-frequency FR1 to FR1 temporal domain case B Generalization

| Parameters | Value |
| --- | --- |
| UE trajectory option [2] | Option 1 |
| UE trajectory boundary processing option [2] | Option 3 |
| UE speed | 90 Km/h |
| Inter-frequency correlation assumption (yes or no) | N/A |
| Measurement reduction rate | 50% |
| Observation window | 1000 ms |
| Prediction window | 200 ms |
| Any other parameters | N/A |
| Number of training examples | ~14.7 k |
| Number of validation examples | ~2.1 k |
| Number of testing examples | ~4.2 k |
| AI/ML model input | L3 cell-level RSRPs from 3 time instances T-5, T-3, T-1 |
| AI/ML model output | L3 cell-level RSRPs at 1 time instance T |
| AI/ML model type | CNN |
| AI/ML model number of parameters | 0.104 M |
| AI/ML model number of FLOPs | 0.128 M |

Table 3: Evaluation results of intra-cell intra-frequency FR1 to FR1 temporal domain case B generalization

| Inference using the dataset of Configuration 1 (UMi) |  |  |  |
| --- | --- | --- | --- |
| Generalization Case | Trained with examples from dataset of Configuration 1 (UMi) | Trained with examples from dataset of Configuration 2 (UMa) | AIML-based Average L3 cell-level RSRP Difference (dB) |
| Baseline | ü |  | 0.4071 |
| GC#1 |  | ü | 0.4276 |
| GC#2 | ü | ü | 0.4061 |
| Inference using the dataset of Configuration 2 (UMa) |  |  |  |
| Generalization Case | Trained with examples from dataset of Configuration 1 (UMi) | Trained with examples from dataset of Configuration 2 (UMa) | AIML-based Average L3 cell-level RSRP Difference (dB) |
| Baseline |  | ü | 0.1109 |
| GC#1 | ü |  | 0.1135 |
| GC#2 | ü | ü | 0.1021 |

For a given deployment configuration (e.g., UMi or UMa), the AI/ML models trained with the same deployment configuration have slightly better accuracy than the AI/ML models trained with different deployment configuration (i.e., baseline vs. GC#1 cases). The minor degradation in accuracy is more noticeable in UMi configuration than UMa configuration.

The slight worsening in the accuracy of GC#1 cases can be negated by training using data samples from multiple deployment configurations. For example, all GC#2 cases lead to slightly better accuracy compared to the respective GC#1 cases. They also achieve comparable accuracy compared to the respective baseline case.

Training AI/ML models for RRM measurement prediction in similarly shaped deployment configurations (both UMi and UMa are outdoor deployment configurations with hexagonal sites/sectors) with different dimensionalities (e.g., different ISD, different BS height, different BS Tx power) can generalize well across those configurations either using GC#1 or GC#2 approaches.

Observation 1: Regarding GC#1 for UMi and UMa deployment configurations, there is a minor deterioration in the accuracy of the AI/ML model compared to the baseline case. This minor deterioration is more noticeable in UMi configuration than UMa configuration.

Observation 2: Using a mixed dataset from UMi and UMa deployment configurations (i.e., GC#2) slightly improves the accuracy of the AI/ML model compared to GC#1 cases, while offering comparable accuracy as the baseline case.

Observation 3: AI/ML models for RRM measurement prediction generalize well across similarly shaped deployment configurations (e.g., UMi and UMa) with different dimensionalities (e.g., different ISD, different BS height, different BS Tx power).

# 3. Conclusion

In this contribution, evaluation results for the RRM measurement prediction generalization across different deployment configurations (e.g., UMi and UMa) were discussed, and the following observations and proposals were made:

Observation 1: Regarding GC#1 for UMi and UMa deployment configurations, there is a minor deterioration in the accuracy of the AI/ML model compared to the baseline case. This minor deterioration is more noticeable in UMi configuration than UMa configuration.

Observation 2: Using a mixed dataset from UMi and UMa deployment configurations (i.e., GC#2) slightly improves the accuracy of the AI/ML model compared to GC#1 cases, while offering comparable accuracy as the baseline case.

Observation 3: AI/ML models for RRM measurement prediction generalize well across similarly shaped deployment configurations (e.g., UMi and UMa) with different dimensionalities (e.g., different ISD, different BS height, different BS Tx power).

# 4. References

[1]	RAN2-129 Chairman Notes, Athens, Greece, February 2025

[2]	R2-2500559: Email discussion summary for [Post128][018][AI Mob] generalization (Apple)

[3]	TR 38.744 V0.0.6: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR (Release 19), February 2025

[4]	TR 38.901: Study on channel model for frequencies from 0.5 to 100 GHz (3GPP TR 38.901 version 16.1.0 Release 16)

# Appendix

### A.1 Model generalization agreements from RAN2-129

Agreements on generalization over UE speed

- For intra-frequency temporal domain prediction in FR1/FR2, generalization performs well across all UE speeds

- Using a mixed dataset (GC#2) slightly improves the accuracy of the AI/ML model compared to GC#1 cases, while offering comparable accuracy as the baseline case (for the same data set size).

- For GC#1, as the difference between the UE speed that an AI/ML model trained at and the UE speed that the inference is being made decreases, the AI/ML achieves a closer performance to the baseline.

Agreements on generalization over frequency

- Generalization using GC#2 always outperform that of GC#1

- Training and generalization using the knowledge about the input & output frequency (or even an indication) outperforms the case where the model cannot recognize which frequency at the input or output.

- Considering the generalization parameter of different predicted frequencies in inter-freq prediction, the GC#1 case without any preprocessing based on the information of predicted frequency suffers from significant performance loss.

- For GC#2 case, the prediction accuracy is acceptable and close to the baseline

### A.2 Simulation assumptions of Intra-cell intra-frequency FR1 to FR1 temporal domain case B

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 – 4 GHz |
| Deployment | UMa/UMi, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Inter site distance | 500 m / 200 m |
| Channel model | According to TR 38.901 [4] |
| System BW | 20 MHz |
| Sub-carrier Spacing | 30 kHz |
| UE Speed | 90 Km/h |
| UE distribution | 100% outdoor (in vehicle) |
| BS Antenna Configuration | (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern |
| UE Antenna Configuration | (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Antenna radiation pattern | Omni-direction |
| BS Tx Power | 44 dBm / 40 dBm |
| UE receiver Noise Figure | 9 dB |
| BS Antenna height | 25 m / 10 m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR 38.901 [4] |
| UE trajectory model | Option 1 (Linear trajectory model with random direction change) |
| LOS Update | LOS Soft |
| Number of Beams | 4 Tx Beams (4 Azimuth angles x 1 Elevation angle) 1 Rx Beam (1 Azimuth angle x 1 Elevation angle) |
| L1-Filtering: Method | Non-sliding Averaging |
| L1-Filtering: Filter Window | 5 |
| Beam Consolidation: nrofSS-BlocksToAverage | 1 |
| Beam Consolidation: absThreshSS-BlocksConsolidation | -110 dBm |
| L3-Filtering: Filter Coefficient K | 4 |