3GPP TSG-RAN WG2 Meeting #129bis	R2-2502255

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.3.5

Source:	InterDigital Inc.

Title:	LCM and spec impact for AI/ML mobility

Document for:	Discussion

# 1. Introduction

The study item for AI/ML for mobility in NR has an objective related to LCM and specification impacts [1]:

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

In this contribution, we discuss how the currently agreed LCM framework can be reused for the mobility use cases.

# 2. Discussion

## 2.1 Background

As described in the SI for AI/ML mobility [1], the LCM for AI/ML is to be based on the LCM for the use cases studied regarding AI/ML for NR air interface. From the use cases studied under the NR air interface, the beam management (BM) and CSI prediction use cases are the closest to the AI/ML mobility use case, as the entities involved are the UE and the gNB, and only one-sided models are considered.

Observation 1: From the use cases of AI/ML for NR air interface that are being specified, the beam management and CSI prediction use cases are the closest to the AI/ML mobility use case from LCM point of view.

However, since only the BM case and positioning use cases are being specified in rel-19 and the discussion so far in RAN2 has not considered CSI prediction, the LCM for AI/ML mobility should be adapted to the LCM for the BM use case.

Based on the above, we propose:

Proposal 1: The LCM for all AI/ML mobility use cases that are being considered in rel-19 (i.e., RRM measurement prediction and RRM measurement event prediction) to reuse the LCM framework agreed for the BM use case.

## 2.2 LCM for AI/ML mobility

RAN2 has been discussing the LCM for the BM case for several meetings (see Appendix 1 for the LCM related agreements for the BM case since RAN2_127). The currently agreed upon framework/signalling based on the agreements so far can be summarized in the figure below:

Figure 1: LCM framework for AI/ML BM use case.

### 2.2.1 Supported functionalities

In Steps 1/2, the UE indicates its supported AI/ML functionalities to the Network via UE capability information.

For RRM measurement prediction, there are 6 sub-uses considered (see Appendix 2). For each sub-use case, the predictions can be performed in the temporal, frequency, or spatial domain (e.g., see table 5.2.1-1 in [7]). For the case of measurement event prediction, all these sub-use case combinations are also possible.

It can be discussed whether the reporting for AI/ML supported functionality should be at a granularity level that considers all possible sub-use cases. However, a granularity at least at the use case level (i.e., RRM measurement prediction and RRM measurement event prediction) is required.

For indirect measurement event prediction, the event prediction is based on the AI/ML RRM measurement prediction model (i.e., event prediction is a post-processing step based on predicted measurements). However, for direct measurement event prediction, a separate model is used for predicting the events, without intermediate measurement prediction.

Observation 2: A UE may support both measurement and event prediction via a single AI/ML model (i.e., if indirect event prediction is employed) or separate models (i.e., if direct event prediction is employed).

Based on the above, we propose:

Proposal 2: The UE indicates the supported AI/ML mobility functionalities in UE capability signalling, at least at the granularity of the use case level (i.e., RRM measurement prediction, RRM measurement event prediction).

Proposal 3: If the UE supports indirect measurement event prediction, it indicates that along with the RRM measurement prediction capability indication.

Proposal 4: If the UE supports direct measurement event prediction, it indicates that separately from the RRM measurement prediction capability indication.

Proposal 5: AI/ML mobility support indication at the granularity level of sub-use cases and/or the temporal/frequency/spatial domain is FFS.

### 2.2.2 Inference and reporting configuration

For the BM use case, it has been agreed that the network can provide inference configuration and NW-side additional conditions to the UE via CSI report configuration.

The NR RRM measurement configuration (i.e. MeasConfig) contains three main components:

  - Measurement objects: configuration of what is to be measured (e.g., frequency, cells, SSB/CSI-RS config, offsets, measured quantities, cell level measurement derivation parameters, etc.,)

  - Reporting configurations: configuration of when a measurement report is to be reported or action to be taken on event fulfilment (e.g., periodicity, event thresholds, TTT, hysteresis, CHO config, etc.

  - Measurement IDs: association of a measurement object with a reporting configuration, i.e., UE performs the measurements according to the measurement object configuration and reports it according to the associated periodic/event reporting configuration

It is natural to reuse the RRM measurement configuration framework to configure RRM measurement and RRM measurement event predictions. For example, inference parameters related to measurement prediction can be added to MeasObjectNR and parameters related to measurement event prediction and reporting of predicted measurements can be added to ReportConfigNR.

When it comes to configuration related to NW side additional conditions, which is needed by the UE to determine applicability (as will be discussed in section 2.2.3), one main difference between the BM and mobility use case is that in the latter, the prediction is likely to be performed for more than one cell (e.g., the serving cell and a certain number of neighbor cells). Thus, the network side additional conditions (e.g., associated IDs for the serving as well as the neighbor cell) may need to be provided to the UE along with the other inference related configuration.

Proposal 6: The NR RRM measurement framework will be extended to configure RRM measurement prediction and measurement event inference and reporting configurations:

- introduce parameters/IEs for configuring inference parameters and network side additional conditions (e.g., associated IDs for the serving cell and neighbour cells being predicted,

- introduce parameters/IEs related to RRM measurement event predictions and the reporting of predicted measurements and events.

Proposal 7: The modelling of the new parameters/IEs (e.g., within the current MeasObjectNR/ReportConfigNR or with new IEs) can be discussed during the normative stage in rel-20.

### 2.2.3 Applicability determination and reporting

Similar to the BM use case, the UE can determine the applicability of the AI/ML mobility functionality by considering model availability, network side additional conditions (if provided), UE-side additional conditions (internally known by UE) and inference configuration.

Proposal 8: The UE determines that an RRM measurement prediction and/or measurement event prediction configuration is applicable if the UE has at least one model that satisfies all the following:

- The model is trained for the indicated network side additional conditions (if provided).

- The model is trained for the current UE side additional conditions.

- The model can perform the predictions under the indicated inference configuration.

Since the RRM measurement/event prediction and reporting configuration is provided in a measConfig that is included in an RRCReconfiguration message, the UE can send the applicability report in the RRCReconfigurationComplete message.

Similar to the BM case, the UE can also be configured to perform proactive applicability monitoring and reporting, as a non-applicable RRM AI/ML mobility configuration may become applicable or an applicable one may become non-applicable (e.g., due to changes in UE conditions such as speed, HO, etc.). When such a change is detected, the UE can send the applicability report in a UEAssistanceInformation message.

Proposal 9: Upon receiving the RRM measurement/event prediction configurations in an RRCReconfiguration message, the UE determines the applicability of the received measurement/event prediction configurations and sends an RRCReconfigurationComplete message that includes an indication of which configurations are applicable, and which are not.

Proposal 10: If the UE was configured for applicability change monitoring/reporting, the UE starts monitoring the applicability/non-applicability of the concerned configurations and sends a UEAssistanceInformation message that includes an indication of the configuration whose applicability has changed (i.e., from applicable to non-applicable and vice versa).

### 2.2.4 Reporting of inferred measurements/events

In the BM case, if the CSI reporting configuration is semi-persistent, the UE must receive further lower layer activation signalling before it can perform the reporting of the predicted beams. In the case of RRM, there is no such a need, as everything related to RRM is configured at RRC level and assumed to be activated upon configuration. As such, if the one or more RRM measurement prediction and event prediction configurations are applicable, the UE will autonomously activate the measurement/event prediction and reporting configurations (that are associated with a measurement ID) and will perform the corresponding reporting when the configured reporting conditions are fulfilled.

Proposal 11: The UE automatically activates the applicable RRM measurement prediction and measurement event configurations (i.e., starts the measurement/event predictions according to the configurations, and sends measurement reports according to the reporting configurations).

When it comes to reporting of predicted RRM measurements/events, there are several configuration options:

- Periodic reporting of measurements: The UE is configured to report measurements periodically, and in the measurement report it includes current measurements and/or predicted measurements spanning a prediction window.

- Event triggered reporting based on current measurements: Events are triggered as in legacy NR (i.e., based on actual measurements), and the UE includes current measurements and/or predicted measurements in the measurement report.

- Event triggered reporting based on current/predicted measurements/events: Events triggered based on current/predicted measurements, and the measurement report that is sent includes current and/or predicted measurements (and may include information about the event prediction window for the case of direct event prediction, if not implictly known by the network).

Proposal 12: The UE can be configured to perform one or more of the following reporting based on predicted RRM measurements/events:

- Periodic measurement reports that include current measurements and predicted measurements.

- Events triggered measurement reports based on current measurements, where the measurement report includes current and/or predicted measurements and/or indications for predicted events.

- Event triggered measurement reports based on current/predicted measurements/events, where the measurement report includes current and predicted measurements.

# 3. Conclusion

In this contribution, we discussed LCM and specification impact of the AI/ML mobility use cases and the following observations and proposals were made:

Observation 1: From the use cases of AI/ML for NR air interface that are being specified, the beam management and CSI prediction use cases are the closest to the AI/ML mobility use case from LCM point of view.

Observation 2: A UE may support both measurement and event prediction via a single AI/ML model (i.e., if indirect event prediction is employed) or separate models (i.e., if direct event prediction is employed).

Proposal 1: The LCM for all AI/ML mobility use cases that are being considered in rel-19 (i.e., RRM measurement prediction and RRM measurement event prediction) to reuse the LCM framework agreed for the BM use case.

Proposal 2: The UE indicates the supported AI/ML mobility functionalities in UE capability signalling, at least at the granularity of the use case level (i.e., RRM measurement prediction, RRM measurement event prediction).

Proposal 3: If the UE supports indirect measurement event prediction, it indicates that along with the RRM measurement prediction capability indication.

Proposal 4: If the UE supports direct measurement event prediction, it indicates that separately from the RRM measurement prediction capability indication.

Proposal 5: AI/ML mobility support indication at the granularity level of sub-use cases and/or the temporal/frequency/spatial domain is FFS.

Proposal 6: The NR RRM measurement framework will be extended to configure RRM measurement prediction and measurement event inference and reporting configurations:

- introduce parameters/IEs for configuring inference parameters and network side additional conditions (e.g., associated IDs for the serving cell and neighbour cells being predicted,

- introduce parameters/IEs related to RRM measurement event predictions and the reporting of predicted measurements and events.

Proposal 7: The modelling of the new parameters/IEs (e.g., within the current MeasObjectNR/ReportConfigNR or with new IEs) can be discussed during the normative stage in rel-20.

Proposal 8: The UE determines that an RRM measurement prediction and/or measurement event prediction configuration is applicable if the UE has at least one model that satisfies all the following:

- The model is trained for the indicated network side additional conditions (if provided).

- The model is trained for the current UE side additional conditions.

- The model can perform the predictions under the indicated inference configuration.

Proposal 9: Upon receiving the RRM measurement/event prediction configurations in an RRCReconfiguration message, the UE determines the applicability of the received measurement/event prediction configurations and sends an RRCReconfigurationComplete message that includes an indication of which configurations are applicable, and which are not.

Proposal 10: If the UE was configured for applicability change monitoring/reporting, the UE starts monitoring the applicability/non-applicability of the concerned configurations and sends a UEAssistanceInformation message that includes an indication of the configuration whose applicability has changed (i.e., from applicable to non-applicable, or vice versa).

Proposal 11: The UE automatically activates the applicable RRM measurement prediction and measurement event configurations (i.e., starts the measurement/event predictions according to the configurations, and sends measurement reports according to the reporting configurations).

Proposal 12: The UE can be configured to perform one or more of the following reporting based on predicted RRM measurements/events:

- Periodic measurement reports that include current measurements and predicted measurements.

- Events triggered measurement reports based on current measurements, where the measurement report includes current and/or predicted measurements and/or indications for predicted events.

- Event triggered measurement reports based on current/predicted measurements/events, where the measurement report includes current and predicted measurements.

# 4. References

[1]	RP-242393, Study on AI/ML for mobility in NR, RAN_105, Melbourne, Australia, September 2024

[2]	TR 38.843, Study on AI/ML for NR air interface

[3]	3GPP 127 meeting Chairman notes.

[4] 	3GPP 127bis meeting Chairman notes.

[5] 	3GPP 128 meeting Chairman notes.

[6] 	3GPP 129 meeting Chairman notes.

[7]	TR 38.744, Study on AI/ML for mobility in NR.

# Appendix 1: Agreements related to LCM for BM use case

RAN2#127 meeting [3]:

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

RAN2#127b meeting [4]:

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- FFS if inference configuration can be signalled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

RAN2#128 meeting [5]:

Agreements

1.	When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

2.	When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

3.	FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

4.	Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

5.	Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

RAN2#129 meeting [6]:

Agreements

1.	Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

2.	The full inference configuration is sent in CSI-ReportConfig.

3.	Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

4.	FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

# Appendix 2: RRM measurements sub use cases

The following 6 sub-use cases are considered for L3 cell-level and L3 beam-level RRM measurement prediction in the AI/ML mobility SI [7]:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated

- Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s)

- Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)