3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502268

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.2.2

Source:	Huawei, HiSilicon

Title:	A-IoT paging

Document for: Discussion and Decision

# 1	Introduction

In the last RAN2 meetings, some related agreements were achieved in the following:

Parallel service requests by the same reader is not supported. The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed). The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - The A-IoT paging message can include a number of msg1 resources

Based on the current RAN2 progress, and focusing on the issues without conclusions, this contribution discusses subsequent paging, paging message design/content, and others.

# 2	Discussion

## 2.1	Subsequent paging

### 2.1.1	Multiple reader scenario

In RAN2#129 meeting, the following agreements on parallel paging procedures were achieved:

- Parallel service requests by the same reader is not supported. - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed).

Since the last meeting has agreed that the device is expected to only perform one procedure at a time, how to avoid the device to receive the parallel/interleaved services from the neighbour readers is further discussed.

For the readers “belonging” to the same BS, the BS can configure separate resources for different readers to ensure no multiple requests received by devices, and then the BS implementation can also avoid sending parallel/interleaved services to the devices.

Then we should consider the neighbour readers belonging to different BSs. In R3-250815, the TP to TS 38.300 BL CR has captured the following agreement.

- In this version of the specification, no A-IoT specific communication takes place between gNBs.

According to the above discussion in RAN3, it is not supported to have any inter-BS coordination. Therefore, we understand there are two potential methods to avoid parallel/interleaved services from the readers belonging to different BSs. In the first method, CN implementation ensures that a reader starts a new service after a reader from another BS successfully finishing its own service. In the other method, OAM can configure isolated resources for different BSs. Then the device will not receive another service request when it is already involved in one service under one BS.

Figure 2.1.1-1 Example of OAM configuration with isolated resources

Observation 1a:	Network implementation can avoid parallel services from the neighbour readers that belong to different BSs, e.g., OAM can allocate/configure non-overlapped time-domain resources.

Observation 1b:	Inter-BS is not really critical for indoor scenario.

In the last meeting, there was an FFS about device behaviour if multiple requests are received in parallel. Some companies proposed that device can ignore any other paging message received if it is participating in an ongoing paging round. Firstly, we think it is a corner case considering the above implementation. However, even if it happens, we think this solution is incomplete. Even if the device ignores the paging message, it may still receive other following messages, such as QueryRep-like message, which impacts on device counting access occasions. So the device should ignore all other R2D messages from a different reader rather than just paging messages. In details, ignoring all R2D messages from neighbour readers means that all R2D messages need to carry the transaction ID, which is an unacceptable overhead in the AS layer and also increases the processing complexity of the device.

We give an evaluation of signalling overheads caused by including the transaction ID in all R2D messages, and the details are in the Appendix. The signalling overhead and the impact on inventory latency are concluded as follows:

- Signalling overheads: If all R2D messages carry the transaction ID, an additional 6% in total R2D+D2R signalling overhead. Considering the R2D and D2R are two links, we further evaluate the additional R2D overhead’s percentage of total R2D signalling overhead. The results indicate an approximately 28% increase in R2D signalling overhead in some cases.

- Impact on inventory latency: Considering the data rate of R2D and D2R varies, we take 1 kbps and 7 kbps for example (according to TR 38.769), the maximum percentage increase in total inventory latency can reach about 19%.

Therefore, even when the NW implementation does not work well to avoid multiple requests being received in parallel, we do not think it is necessary to change the device behaviour to handle this. It can be just left as: the device just responds to the new BS and replaces the old transaction ID with the new transaction ID.

Figure 2.1.1-2 R2D messages impacts due to other readers

Observation 1c:	Parallel services from the neighbour readers require that all the R2D messages include the reader ID/transaction ID, which implies an unacceptable overhead at the AS layer.

Observation 1d:	Including 3-bit reader/transaction ID in all R2D messages may raise 28% R2D signalling overhead and 19% inventory latency.

Once transaction ID is introduced, how to use it needs further discussion. From our point of view, the device needs to maintain the transaction ID and the status (successful or not yet) of the current service. If the received transaction ID in the paging message is the same as the one maintained by the device, the device will determine whether to respond to the paging message based on the associated maintained status (successful or not). For example, when a device maintains a “transaction ID” and a corresponding “successful status”, the device will not respond to the following (subsequent) paging messages with the same transaction ID.

On the other hand, if the received transaction ID in the paging message is different from the one maintained by the device, the device updates its maintained “current transaction ID” as the received one. If the device is selected to access by the paging identifier in the paging message, the device will respond to this paging message corresponding to the new transaction ID and store its status (successful or not).

Proposal 1:	If the “transaction ID” received in the paging message differs from the one currently maintained by the device, the device will update its currently maintained “transaction ID” to the received value and act consequently.

### 2.1.2	Transaction ID

In RAN2#129 meeting, the following agreement on transaction ID were achieved:

- The “transaction ID” can be generated by reader based on CN correlation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size.

Considering that there may be a case that several readers are deployed near each other, as shown in Figure 2.1-2, a 1-bit transaction ID may be insufficient to distinguish old and new services from different BSs. In detail, if the transaction ID is too short, there could be different services from multiple different neighbour BSs that use the same transaction ID. Then the device will miss a service from one of the multiple readers. Therefore, more than one bit transaction ID may be needed. Besides, multiple-bit transaction ID will also reduce the possibility of wraparound issue. If the device failed to receive the paging messages for the new (the second) service, the device can still process the third service instead of regarding it as an old service (the first one).

Figure 2.1-2		Device in an overlapped coverage of multiple readers

To balance the purpose of reducing the required buffer size, 2-bit transaction ID in paging message seems enough to distinguish old and new services from different BSs. Nevertheless. 3-bit transaction ID in paging message seems an acceptable compromise.

Proposal 2:	2-bit transaction ID in paging message seems enough, while 3-bit transaction ID in paging message seems a reasonable compromise.

According to the discussion in RAN3/SA2, there may be a correlation ID transmitted over XX/NG-AP interface. The reader can use this correlation ID to associate the response(s) from devices to a specific service request from CN. If we use this correlation ID as the transaction ID, the length of the transaction ID could be large, since the correlation ID is used to identify a service among all the services from different CN nodes. On the other end, considering the transmission overhead and the buffer size required by the device storage for transaction ID, the transaction ID in the paging message should not be the same as the correlation ID from CN.

There could be multiple methods for reader implementation to generate the transaction ID. A first possible method could be for the transaction ID to use the right-most truncated part of CN correlation ID, which can be shorter than the correlation ID. A second method could be that the reader can try to use different transaction IDs for different CN correlation IDs, according to different implementation manners (i.e. different truncated ways). In yet another method, the reader can generate the transaction ID not considering the CN correlation ID.

How the reader will generate "transaction ID" can be implemented in multiple manners mentioned above, but the specific method used by the reader should be left to the network implementation. Therefore, it is suggested to not specify how reader generates the transaction ID.

Proposal 3:	It is suggested to not specify how the reader generates the transaction ID, while some reader implementation manners can be following:

The transaction ID is the right-most truncated part of CN correlation ID; or

The reader tries to use different transaction IDs for different CN correlation IDs; or

Reader generates the transaction ID not considering the CN correlation ID at all (on purpose).

## 2.2	Paging message design/content

### 2.2.1	Paging identifier size

A paging message contains both AS and upper-layer information. The AS information includes the message type, control information and access resource indication. The upper-layer information includes the paging identifier, which can be a permanent ID, and a temporary ID, or a mask/filter/group ID. The length of device ID is variable, as detailed in SA2 TR 23.700 [1]:

Figure 8.2.1-1: The example of Operator allocated ID NOTE 4: Within each type of ID, the length of Part 1 is fixed (i.e. the length of ID type, the Network Identifier (if present) and the Information used to identify a third party (if present) are fixed) and the length of Part2 is dynamic. The detail is left to stage 3.

Besides, the length of a mask is also variable. For example, in RFID, a mask is a bit string that is compared by a device to a memory location starting at the indicated pointer and spanning the indicated length of bits.

For process simplification, AS and upper-layer information should be divided into two non-overlapping fields. One approach is to place the AS fields before the upper-layer field, as shown in Fig.2.2.1-1. This way, when a device processes a paging message, the MAC layer can first read the AS fields, which have the fixed format. The remaining field is the upper-layer paging identifier information, which can be directly delivered to the upper layer, assuming there is no variable-size MAC padding in the paging message. This format design can avoid the need to indicate the length of the paging identifier.

Figure 2.2.1-1	An example of paging format design

Proposal 4a:	In an A-IoT paging message, there is no need of a length field for the paging identifier field. This is based on the assumption that there is no MAC padding in the paging message, and the paging identifier field will be placed at the end of the paging message.

If Proposal 4a is not agreed, RAN2 will need to indicate the length of paging identifier in an A-IoT paging message. This will require further conclusions and input from SA2/CT1/CT4.

Proposal 4b:	If Proposal 4a is not agreed, RAN2 asks SA2/CT1/CT4 for the size of paging identifier (e.g., permanent ID, temporary ID, mask/filter/group ID).

### 2.2.2	CBRA vs. CFRA type indication

In the last RAN2#129 meeting, it was agreed to include the number of Msg1 resources in the A-IoT paging message. In Rel-19, only one device ID can be included in one paging message. Hence, when a paging message includes one device ID and indicates only one access occasion, the CFRA is implicitly triggered.

Considering the forward compatibility, multiple device ID can be included in one paging. If a paging provides one dedicated access occasion for each device ID, each device will correspond to one dedicated access occasion. Hence, CFRA can also be implicitly triggered without explicit RA type indication.

Observation 2:	Even considering the forward compatibility (of multiple devices CFRA), it is still valid to determine CFRA “if paging provides one access occasion for each included device ID”.

Proposal 5:	Whether the random access is contention-based or contention-free can be implicitly indicated by the number of access occasion(s), e.g., one access occasion for one/each device ID implies contention-free access, i.e., there is no need for a dedicated explicit indication of the access type.

## 2.3	Others

### 2.3.1	Temporary ID impacts

As agreed in SA3 TR 33.713 (S3-250958), a temporary ID is used to protect device ID [3]:

The following aspects and principles are agreed for the conclusion on KI#3 - A mechanism to protect AIoT device ID based on the use of temporary ID shall be supported. - Mechanism shall allow unambiguous identification of the AIoT device. - A mechanism to re-synchronize de-synchronized temporary IDs shall be supported.

If CN allocates a temporary ID, the transmission of device ID over the air interface can be avoided due to the privacy protection. The temp ID can be applied to the two cases. For the first case, A-IoT paging messages can include temporary ID instead of device ID. Another case is that the device ID in Msg3 can be replaced by temporary ID during inventory.

Observation 3:	If device is allocated with a temporary device ID, this ID can be used in paging message and device ID reporting (e.g. Msg3) for inventory.

Note temporary ID is allocated by the CN and not visible to reader (especially when ciphered), and CN can assign the temporary ID in a Write command, which is to be stored by the device in its memory. The AS procedure in RAN2 can already supports a Write command procedure, and the upper-layer Write command is not visible to reader/RAN2. Therefore, there is no AS/RAN2 specific impact to support allocation of temporary ID.

RAN2 can wait for further SA3 and SA2 conclusions on whether this temporary ID is device generated or CN assigned. In the former case, there may be some SA3 related parameters to be carried as some upper layer information in MAC message, e.g., paging or Msg3. In the latter case, RAN2 can assume the temporary device ID can be allocated via a write command (or write-like command), if needed.

Proposal 6:	There is no immediate RAN2 specific impact due for supporting the temporary device ID, other than RAN2/reader may need to know the size of device ID. Further progresses in SA2/SA3 can trigger more RAN2 discussion, if needed.

### 2.3.2	RAN1 parameters

On one hand, the device ID size is one major part in terms of paging payload. Shorter device ID is then beneficial, if it can still achieve the global uniqueness, while the design is up to SA and CT.

On the other hand, the possible physical layer parameters also affect the size of paging message. In case RAN2 considers that the whole information/parameters in paging message would results in one big paging message, some of those physical layer parameters can be moved from paging message into another R2D message following the paging message.

Proposal 7:	RAN2 waits for further progress from RAN1 regarding which physical layer parameters need to be carried in the paging message.

### 2.3.3	Sub-group paging

Some companies propose to divide devices into sub-groups during a paging round to address the issue of a large number of devices being paged simultaneously, in order to aim at reducing collision probability. However, according to the optimal theoretical slotted Aloha efficiency value, the same effect can be achieved by allocating more access occasions in a single A-IoT paging. When many devices are paged, more access occasions are allocated and the devices can be randomly divided among the allocated occasions. On the other hand, sub-grouping has no gain on power saving, because the paged devices anyway need to continuously monitor to avoid missing the subsequent paging/R2D trigger messages.

Observation 4:	There is no motivation to support sub-group paging using temporary ID, since the reader can always allocate more access occasions in case of many device being selected, rather than dividing devices into separate paging group.

# 3	Conclusion

This contribution makes the following proposals:

Observation 1a:	Network implementation can avoid parallel services from the neighbour readers that belong to different BSs, e.g., OAM can allocate/configure non-overlapped time-domain resources.

Observation 1b:	Inter-BS is not really critical for indoor scenario.

Observation 1c:	Parallel services from the neighbour readers require that all the R2D messages include the reader ID/transaction ID, which implies an unacceptable overhead at the AS layer.

Observation 1d:	Including 3-bit reader/transaction ID in all R2D messages may raise 28% R2D signalling overhead and 19% inventory latency.

Observation 2:	Even considering the forward compatibility (of multiple devices CFRA), it is still valid to determine CFRA “if paging provides one access occasion for each included device ID”.

Observation 3:	If device is allocated with a temporary device ID, this ID can be used in paging message and device ID reporting (e.g. Msg3) for inventory.

Observation 4:	There is no motivation to support sub-group paging using temporary ID, since the reader can always allocate more access occasions in case of many device being selected, rather than dividing devices into separate paging group.

Multiple reader scenario

Proposal 1:	If the “transaction ID” received in the paging message differs from the one currently maintained by the device, the device will update its currently maintained “transaction ID” to the received value and act consequently.

Transaction ID

Proposal 2:	2-bit transaction ID in paging message seems enough, while 3-bit transaction ID in paging message seems a reasonable compromise.

Proposal 3:	It is suggested to not specify how the reader generates the transaction ID, while some reader implementation manners can be following:

The transaction ID is the right-most truncated part of CN correlation ID; or

The reader tries to use different transaction IDs for different CN correlation IDs; or

Reader generates the transaction ID not considering the CN correlation ID at all (on purpose).

Paging identifier size

Proposal 4a:	In an A-IoT paging message, there is no need of a length field for the paging identifier field. This is based on the assumption that there is no MAC padding in the paging message, and the paging identifier field will be placed at the end of the paging message.

Proposal 4b:	If Proposal 4a is not agreed, RAN2 asks SA2/CT1/CT4 for the size of paging identifier (e.g., permanent ID, temporary ID, mask/filter/group ID).

CBRA vs. CFRA type indication

Proposal 5:	Whether the random access is contention-based or contention-free can be implicitly indicated by the number of access occasion(s), e.g., one access occasion for one/each device ID implies contention-free access, i.e., there is no need for a dedicated explicit indication of the access type.

Temporary ID impacts

Proposal 6:	There is no immediate RAN2 specific impact due for supporting the temporary device ID, other than RAN2/reader may need to know the size of device ID. Further progresses in SA2/SA3 can trigger more RAN2 discussion, if needed.

RAN1 parameters

Proposal 7:	RAN2 waits for further progress from RAN1 regarding which physical layer parameters need to be carried in the paging message.

# 4	References

- 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things, Feb. 2025

- Y. Maguire and R. Pappu, “An Optimal Q-Algorithm for the ISO 18000-6C RFID Protocol,” in IEEE Transactions on Automation Science and Engineering, 2009.

- TR 33.713 V0.7.0 (S3-250958), Study on Security Aspect of Ambient IoT Services in 5G, Feb. 2025

# 5	Appendix

We give the evaluation of signalling overheads caused by including the transaction ID in all R2D messages in the following. Considering each QueryRep triggers one time-frequency resource (i.e. one access occasion), the inventory procedure consist of three kinds of access occasions, which are summarized as follows:

- Successful occasion, the transmitted messages include one QueryRep, one Msg1, one Msg2, and one Msg3.

- Idle occasion, there is only one QueryRep.

- Collided occasion, the transmitted messages include one QueryRep and more than one Msg1.

An example of successful, idle, and collided occasions is shown in Figure 5.1.

Figure 5.1	An example of successful, idle, and collided occasions

To evaluate the signalling overheads, we consider the size of different messages as follows:

Table 5.1		The assumption of each message size

| Msg type | Bit size without transaction ID | Bit size without transaction ID |
| --- | --- | --- |
| QueryRep | 3 3-bit message type | 6 By adding 3-bit transaction ID |
| Msg1 | 16 16-bit random ID | Same |
| Msg2 | 32 8-bit MAC header, 16-bit random ID, 8-bit control information | 35 By adding 3-bit transaction ID |
| Msg3 | 104 8-bit MAC header, 96-bit device ID | Same |

In ALOHA [2], the optimal successful access probability is Psuccess=36.8%. This corresponds to a collision probability Pcollision=26.4% and an idle-occasion probability Pidle=36.8%. Let the number of paged devices be N. The number of successful occasions is N. Since the proportion of successful occasions among all required occasions is Psuccess, the theoretically total number of required occasions is N/Psuccess. Consequently, the number of idle occasions is Pidle*N/Psuccess, and the number of collided occasions is Pcollision*N/Psuccess.

As analysed above, the total number of R2D and D2R messages can be calculated as follows:

N*QueryRep+Msg1+Msg2+Msg3+Pidle*NPsuccess*QueryRep+Pcollision*NPsuccess*QueryRep+Msg1

= N* QueryRep+Msg1+Msg2+Msg3+ QueryRep+ 26.4%36.8%*QueryRep+Msg1=N* 2.7*QueryRep+Msg2+1.7*Msg1 +Msg3.

The total number of only R2D messages can be calculated as:

N* 2.7*QueryRep+Msg2.

The percentage increase in total bit sizes due to the additional signalling overhead of R2D:

2.7*(LQueryRep+3bit)+(LMsg2+3bit)+1.7*LMsg1 +LMsg3 2.7*LQueryRep+LMsg2+1.7*LMsg1 +LMsg3-1≈6%,

where Lx is the bit length of a message, as shown in Table 5.1.

The percentage increase in total R2D signalling overhead due to the additional signalling overhead of transaction ID in R2D is:

2.7*(LQueryRep+3bit)+(LMsg2+3bit) 2.7*LQueryRep+LMsg2-1≈28%.

To see the addition R2D overhead’s impact on inventory latency, the evaluation are given in the following.

Assuming the D2R rate as RD2R and R2D rate as RR2D, the percentage increase in total inventory latency due to the additional signalling overhead of R2D is:

2.7*LQueryRep+3bitRR2D+LMsg2+3bitRR2D+1.7*LMsg1 RD2R+LMsg3RD2R 2.7*LQueryRepRR2D+LMsg2RR2D+1.7*LMsg1 RD2R+LMsg3RD2R-1.

According to TR 38.769, we take the reference data rate as example, such as 1 kbps and 7 kbps, so we further evaluate the percentage increase due to additional R2D overhead in overall inventory latency in terms of different data rates as follows:

Table 5.2		The percentage increase in overall inventory latency due to additional R2D overhead

| R2D rate | D2R rate | Increasing percentage of total inventory latency |
| --- | --- | --- |
| 7 kbps | 7 kbps | 6% |
| 1 kbps | 7 kbps | 19% |
| 7 kbps | 1 kbps | 1% |

In conclusion, the maximum increasing percentage of total transmission time can achieve about 19%.