3GPP TSG-RAN WG2 Meeting #129bis	R2-2502277

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.7.5

Source:	Nokia, Nokia Shanghai Bell

Title:	RLC enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#129 [R2-2501335]:

Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

On the topic of reporting PDCP SN gap upon mobility:

RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem to solve due to RLC enhancements. If it is a problem further discussions are needed on the alternatives.

This contribution will discuss the remaining open issues for the support of RLC Enhancements.

# 2	Autonomous Retransmissions

## 2.1	Remaining time determination

Two options have been discussed to calculate the remaining time to trigger autonomous retransmissions and or/ polling. A detailed pros/cons comparison of the options is as follows

| Option | Pros | Cons |
| --- | --- | --- |
| Timer at PDCP | Can re-use existing discard timer | For a PDCP SDU, the remaining time of PDCP discard timer at the time of initial RLC transmission can vary a lot, and can even reach the configured threshold before initial RLC transmission. Dependency between RLC AM and the discardTimer discardTimer has to have a non-infinity value configured (it is an optional parameter) |
| Timer at RLC | It is handled within RLC and has better control on early retransmissions (i.e. avoids the time-variability of using discardTimer). Can implement the same logic as t-Reassembly today: retransmission once the previous transmission has evidently spent enough time at MAC. | New timer needs to be introduced |

For the case of having a new timer in RLC, it was mentioned that since the timing for passing PDUs between RLC and PDCP is not specified, the start time of the timer will be uncertain and may differ across various UE implementations style. However, this issue can be avoided if the timer is started after the first submission of an RLC SDU to MAC.

Proposal 1: as the trigger for autonomous retransmission, use expiry of a new timer in RLC that starts upon the first submission of an RLC SDU or a segment thereof to lower layers.

If that is not agreeable, there should at least be a guarantee put in place that an autonomous retransmission always ends up in a different transport block than the one carrying the initial transmission.

## 2.2	Other Issues

The main concerns expressed with autonomous retransmissions had to do with capacity implications. However, the autonomous retransmissions can be kept to a minimum provided that RLC ACKs are provided rapidly. What makes the current RLC Status PDU difficult for rapid acknowledgment is that, among the SDUs that are acknowledged, all the other SDUs are negatively acknowledged. The t-Reassembly timer avoids such premature NACKs but also holds back the ACKs.

Observation 1: among the SDUs that are acknowledged in a RLC Status PDU, all the other SDUs are always negatively acknowledged.

Since RAN2 agreed to focus specifically on timely uplink retransmissions, a complete solution to this problem would be to make it configurable whether the UE considers SDUs for retransmission when NACKed. A UE that would not, would allow the network to send RLC ACKs rapidly without any concerns about premature NACKs.

Proposal 2: to minimize autonomous retransmissions, allow the network to send frequent ACKs, by introducing a possibility to configure the UE not to consider SDUs for retransmission based on received NACKs (to eradicate the problem of premature NACKs).

Another mechanism that may hold back RLC ACKs is LCP. Currently in LCP, there is no consideration regarding prioritizing control PDUs such as RLC Status PDU. The RLC Status PDU is prioritized over data PDU only within the same bearer/LCH when the LCH is treated in LCP. As a result, during LCP, the LCHs with RLC Status PDU awaiting transmission, may end up not getting enough allocation to put such PDUs in the MAC PDU being generated for transmission, delaying transmission of these PDUs.

Proposal 3: RAN2 to discuss possible enhancements in LCP regarding high priority RLC Status PDUs.

With autonomous RLC retransmissions in place, comes the question whether they should be reflected in BSR. If they are, the gNB is not able to distinguish the data volume for initial transmissions from the data volume for retransmissions and as a result, may prioritise the wrong UE. To avoid that, either autonomous retransmission data is not included in data volumes, or it is reported separately.

Proposal 4: autonomous retransmission data is either not included in data volumes, or is reported separately.

# 3	Unnecessary Retransmissions Avoidance

Related to the agreement made in RAN2 #128 for handling of abandoned RLC SDUs, received RLC ACKs are also used locally by PDCP, for instance to determine how high-numbered PDCP SDUs can be transmitted by PDCP:

NOTE 1: Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

Indeed, the PDCP specification also contains the following:

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows: - if RCVD_SN < SN(RX_DELIV) – Window_Size: - RCVD_HFN = HFN(RX_DELIV) + 1. - else if RCVD_SN >= SN(RX_DELIV) + Window_Size: - RCVD_HFN = HFN(RX_DELIV) – 1. - else: - RCVD_HFN = HFN(RX_DELIV); - RCVD_COUNT = [RCVD_HFN, RCVD_SN]. After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall: - perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT; - if integrity verification fails: - indicate the integrity verification failure to upper layer; - discard the PDCP Data PDU and consider it as not received; - if RCVD_COUNT < RX_DELIV; or - if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before: - discard the PDCP Data PDU;

That is, a received PDCP SN that is more than Window_Size (equal to half of the PDCP SN space) ahead of SN(RX_DELIV) is misinterpreted as representing a COUNT value below RX_DELIV, resulting in discarding the PDCP Data PDU with the SN. In the current specification, the PDCP transmitter is expected to take this into account by maintaining a local estimate of RX_DELIV at the peer entity, obtained as the lowest COUNT value not yet positively acknowledged. In absence of a PDCP Status report, these acknowledgements come in the form of RLC ACKs.

Observation 2: a received PDCP SN that is more than half of the PDCP SN space ahead of SN(RX_DELIV) is misinterpreted as representing a COUNT value below RX_DELIV, resulting in discarding the PDCP Data PDU with the SN.

Observation 3: to determine how high-numbered PDCP SDUs can be transmitted, the PDCP transmitter currently maintains a local estimate (lower bound) of RX_DELIV at the peer entity, which in absence of a PDCP Status report is obtained as the lowest COUNT value not yet RLC-ACKed.

By agreement #4 above, the new RLC timer can be started by a received SDU segment not yet delivered to PDCP. It follows that for an unreceived RLC SDU, the new RLC timer may start and expire earlier than PDCP’s t-Reordering, and consequently that the unreceived SDU may be RLC-ACKed before RX_DELIV at PDCP advances past that SDU. This means that RLC ACKs no longer allow the PDCP transmitter to maintain a reliable lower bound of RX_DELIV at the peer entity, also making it unable to determine how high-numbered PDCP SDUs can be transmitted.

Observation 4: accounting for possible RLC SDU segments, for an unreceived RLC SDU, the new RLC timer may expire earlier than PDCP’s t-Reordering, and consequently the unreceived SDU may be RLC-ACKed before RX_DELIV at PDCP advances past that SDU.

Observation 5: RLC ACKs no longer allow the PDCP transmitter to maintain a reliable lower bound of RX_DELIV at the peer entity, also making it unable to determine how high-numbered PDCP SDUs can be transmitted.

As a replacement, regular Status reporting would seem to be needed at PDCP.

Proposal 5: to allow the transmitting PDCP to reliably know how high-numbered PDCP SDUs it can proceed to transmit, the agreed RLC ACK no longer indicating successful delivery is combined with regular PDCP status reporting.

# 4	Reporting PDCP SN gap upon mobility

Based on the current Rel-18 specifications, the following alternative courses of events may have happened with a DRB mapped on RLC AM, just before the UE receives an RRC command for an event involving network-side change of the DRB’s PDCP-termination point (handover, or bearer-type change between MN- and SN-terminated bearer):

A.	The source RAN node successfully received a PDCP SN Gap report from the UE:

-	The current specs, such as XnAP SN Status Transfer, provide no means to transfer this information from the source RAN node to the target RAN node;

-	This case was main the topic of the recent RAN3 discussions.

B.	The UE transmitted a PDCP SN Gap report, but its RLC transmissions did not successfully complete before the layer was flushed by the RRC command:

-	In this case, there is no information to be transferred from the source RAN node to the target RAN node.

In the latter case B, a more detailed sequence can be as follows:

0.	All PDCP SDUs before SDU #X have been successfully delivered from UE to the network;

1.	SDU #X is discarded, and PDCP SN Gap report is triggered and transmitted by RLC, together with some SDUs from #(X+1) onwards;

2.	UE receives and executes an RRC command for handover, including RLC re-establishment for the DRB;

-	The RLC re-establishment takes place before successful delivery of any PDCP PDU transmitted in step 1;

3.	In line with the above, XnAP SN Status transfer from source RAN node to target RAN node indicates #X as the First missing UL SDU (and no bitmap indicating higher-numbered received SDUs);

4.	After handover completion by the UE (and the related delay), as part of PDCP Re-establishment, UE PDCP retransmits the SDUs from #(X+1) onwards. At this point, reception of these SDUs while SDU #X remains missing starts t-Reordering at the target RAN node, which will run until expiry before the received SDUs are delivered to upper layers.

It can be observed that t-Reordering runs unnecessarily in step 4, only because the RAN is missing the PDCP-SN-gap information.

Observation 6: Handover-related retransmissions by UE PDCP, of SDUs following a discarding gap, can unnecessarily start t-Reordering at the handover-target RAN node if the RAN is missing the related PDCP-SN-gap information.

Proposal 6: RAN2 agree that unnecessary starting of t-Reordering at PDCP-mobility target RAN node because of PDCP-SN-gap information missing in the RAN is a problem to be solved in Rel.19.

The only solution direction that also solves case B seems to involve the UE retransmitting PDCP SN Gap information to the target RAN node.

Observation 7: The UE PDCP may have transmitted a PDCP SN Gap report just before a PDCP-mobility event that interrupts the report’s RLC transmission. The only way to recover the report’s information seems to be retransmission by the UE PDCP, to the target RAN node.

A simple fix seems to be that, as part of PDCP Re-establishment (which is always invoked at any PDCP termination-point change), the UE re-transmit any previously transmitted PDCP SN Gap report(s) for which no lower-layer confirmation of successful delivery has been received.

Proposal 7: as part of PDCP entity re-establishment, for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, the transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers.

# 5	Conclusion

This document has made the following observations:

Observation 1: among the SDUs that are acknowledged in a RLC Status PDU, all the other SDUs are always negatively acknowledged.

Observation 2: a received PDCP SN that is more than half of the PDCP SN space ahead of SN(RX_DELIV) is misinterpreted as representing a COUNT value below RX_DELIV, resulting in discarding the PDCP Data PDU with the SN.

Observation 3: to determine how high-numbered PDCP SDUs can be transmitted, the PDCP transmitter currently maintains a local estimate (lower bound) of RX_DELIV at the peer entity, which in absence of a PDCP Status report is obtained as the lowest COUNT value not yet RLC-ACKed.

Observation 4: accounting for possible RLC SDU segments, for an unreceived RLC SDU, the new RLC timer may expire earlier than PDCP’s t-Reordering, and consequently the unreceived SDU may be RLC-ACKed before RX_DELIV at PDCP advances past that SDU.

Observation 5: RLC ACKs no longer allow the PDCP transmitter to maintain a reliable lower bound of RX_DELIV at the peer entity, also making it unable to determine how high-numbered PDCP SDUs can be transmitted.

And proposed the following:

Proposal 1: as the trigger for autonomous retransmission use expiry of a new timer in RLC that starts upon the first submission of an RLC SDU or a segment thereof to lower layers.

Proposal 2: to minimize autonomous retransmissions, allow the network to send frequent ACKs, by introducing a possibility to configure the UE not to consider SDUs for retransmission based on received NACKs (to eradicate the problem of premature NACKs).

Proposal 3: RAN2 to discuss possible enhancements in LCP regarding high priority RLC Status PDUs.

Proposal 4: autonomous retransmission data is either not included in data volumes, or is reported separately.

Proposal 5: to allow the transmitting PDCP to reliably know how high-numbered PDCP SDUs it can proceed to transmit, the agreed RLC ACK no longer indicating successful delivery is combined with regular PDCP status reporting.

Proposal 6: RAN2 agree that unnecessary starting of t-Reordering at PDCP-mobility target RAN node because of PDCP-SN-gap information missing in the RAN is a problem to be solved in Rel.19.

Proposal 7: as part of PDCP entity re-establishment, for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, the transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers.

For proposal 7, a TP is suggested in the Annex below.

# Annex: Text proposal to TS 38.323

### 5.1.2	PDCP entity re-establishment

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this clause for Uu or PC5 interface. After performing the procedures in this clause, the UE shall follow the procedures in clause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the ROHC protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the EHC protocol for uplink if drb-ContinueEHC-UL is not configured in TS 38.331 [3];

-	for AM DRBs, reset the UDC compression buffer to all zeros and prefill the dictionary if drb-ContinueUDC is not configured in TS 38.331 [3];

-	for SRBs and UM DRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers, and;

-	for AM DRBs for Uu interface whose PDCP entities were suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, for each PDCP SDU already associated with a PDCP SN:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer or the discardTimerForLowImportance, as specified in clause 5.2.1;

-	for AM DRBs whose PDCP entities were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4;

-	If drb-ContinueUDC is configured and if the PDCP SDU has been compressed before:

-	submit the PDCP SDU previously compressed to integrity protection and ciphering function;

-	else:

-	perform uplink data compression of the PDCP SDU as specified in clause 5.14.4, and submit the PDCP SDU to integrity protection and ciphering function;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1;

-	for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), if prior to the PDCP entity re-establishment a PDCP SN gap report was submitted to lower layers but its successful delivery has not been confirmed by lower layers:

-	re-submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface.

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

-	process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the clause 5.2.2.1;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	for SRBs, UM DRBs and UM MRBs, if t-Reordering is running:

-	stop and reset t-Reordering;

-	for UM DRBs and UM MRBs, deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression;

-	for AM DRBs and AM MRBs for Uu interface, perform header decompression using ROHC for all stored PDCP SDUs if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for AM DRBs for PC5 interface, perform header decompression using ROHC for all stored PDCP IP SDUs;

-	for AM DRBs and AM MRBs for Uu interface, perform header decompression using EHC for all stored PDCP SDUs if drb-ContinueEHC-DL is not configured in TS 38.331 [3];

-	for UM DRBs, AM DRBs, UM MRBs and AM MRBs, reset the ROHC protocol for downlink and start with NC state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs, AM DRBs, UM MRBs and AM MRBs, reset the EHC protocol for downlink if drb-ContinueEHC-DL is not configured in TS 38.331 [3];

-	for SRBs and UM DRBs, set RX_NEXT and RX_DELIV to the initial value;

-	for UM MRBs and AM MRBs, set RX_NEXT and RX_DELIV to the initial value if initialRX-DELIV is configured in TS 38.331 [3];

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure.

NOTE 1:	After PDCP re-establishment on a sidelink ‎SRB/DRB, UE determines when to transmit and receive with the new key and discard the old key as specified in TS ‎‎33.536 [14].‎

NOTE 2:	At PDCP re-establishment, the MRB type (i.e. UM MRB or AM MRB) is determined by the target configuration.‎