3GPP TSG-RAN WG2#129bis											     R2-2502286

Wuhan, China, 7-11 April 2025

Agenda Item:	8.2.2

Source: 	NEC

Title:	        Ambient-IoT Paging

Document for:	 Discussion

- Introduction

RAN2#129 meeting achieved following agreements:

Agreements The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/“filtering criteria”, while the exact format of latter is supposed to be designed by SA2. The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer The A-IoT paging message can include a number of msg1 resources. From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

This contribution provides our views on the paging of ambient-IoT.

- Discussion

  - Avoid duplicated response

In study phase we already agreed that the reader can send multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN and the duplicated response from devices for the same service request should be avoided [2].

6.3.3 A-IoT paging … It is supported that the reader can send multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN. The duplicated response from devices for the same service request should be avoided. The A-IoT paging message can include information to avoid this duplicated response from the device to a reader. It needs to be further discussed on how to design this information in A-IoT paging message (e.g., including stage-3 details and considering the related aspects from other WGs). Then, based on this information, the device determines whether to skip sending the response to A-IoT paging message or not (if the device had successfully responded the same service before). This information should be short and simple. This information is one ID, while it needs to be further discussed whether the ID is generated by the reader or by the core network. It needs to be further discussed on the size of this information. …

From our view, as least companies should consider whether device should skip respond to the paging under following cases:

- After the device has responded a paging; it then device receives a subsequent paging associated to the same service, which contains no identifier (intends for all devices);

- After the device has responded a paging; it then receives a subsequent paging associated to the same service, which contains the identifier of the device.

From the device's perspective, there could be the following cases of receiving subsequent paging messages:

- After the device has responded a paging; it then receives a subsequent paging associated to the same service, which contains no identifier (intends for all devices);

- After the device has responded a paging; it then receives a subsequent paging associated to the same service, which contains the identifier of the device.

For case a), a subsequent paging for all device may intends for providing resources for devices which failed to access to the reader previously. Therefore, if the device considers it had successfully responded the same service before, the device should skip sending the response to A-IoT paging message.

When the subsequent paging is for all devices (i.e., CBRA), device should skip sending the response to this subsequent paging if the device considers it had successfully responded the same service before.

Consider the cases that devices successfully respond to the paging, reader also has the knowledge of the identities of devices which had successfully responded. On top of that, like in case b), a subsequent paging contains the identifier of the device may be used to trigger another response to the same service. In that case, device may still need to respond again to the paging.

Device should send the response to a subsequent paging regardless of whether it has been responded successfully, if the subsequent paging explicitly indicates the identifier of the device.

During RAN2#129 we agreed:

The “transaction ID” can be generated by reader based on CN correlation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID. 1 bit solution is excluded .   FFS the size.  Aim to have a reasonable size.

For a service identified by a transaction ID, the device may store status information about whether it has been successfully responded, or whether re-access is needed. From the device perspective, if a received paging is associated with a “successfully responded service” or re-access status of the service is “not needed” (i.e., devices do not receive a "NACK" indication after last msg3 transmission), device should avoid duplicated response.

For a service identified by a transaction ID, the device may store status information about whether the service has been successfully responded, or whether re-access is needed.

If device considers it had successfully responded the received service before or re-access status of the service is “not needed” (i.e., device has transmitted msg3 and do not receive a "NACK" indication), device should avoid duplicated response.

  - Paging contents for RA type determination

In previous meeting RAN2 agreed that “For all use cases, device determines the random access type from the paging message”, and the information used for random access type determination can be explicitly or implicitly. From our perspective, at least an implicit or explicit information need to be introduced to indicate whether RA resources indicated in paging is for contention-based or contention-free. This especially applicable to the cases that the paging message is for more than one device (e.g., paging message contains group id, all devices, or multiple devices). While for the cases that paging message only includes identity for a single device, the random access resources indicated in paging is for contention-free random access.

Paging message need to implicitly or explicitly indicate whether RA resources indicated in paging is for CBRA or CFRA.

Some information can be considered to implicitly indicate the contention free random access. For example, if paging message only includes identity for a single device, RA resources indicated in paging is for CFRA.

For cases that paging message only includes identity for a single device, RA resources indicated in paging is for CFRA.

If multiple device identifiers in single paging is supported, assuming a device ID list is used to indicate multiple device IDs, the list can also implicitly indicate the resource allocation for random access. For example, each random access resource may be mapped to a device according to the ranking of the device in the device ID list. By using the mapping relationship, A-IoT devices can be scattering to different random access resources and achieve contention free random access.

Figure 2.2-1 mapping relationship between the entry of device ID list and random access resources

For contention free RA configuration when paging message contains multiple device identifiers, random access resource indicated in paging message are mapped to devices according to the ranking of the device identifier in the list.

  - Paging contents for msg1 resources

During the study phase, RAN1 studied slotted-ALOHA based access and FDMA of Msg.1. For the TDMA of Msg.1, TR38.769 summarized that,

-	A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg.1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s).

Fig.1 shows the slotted-ALOHA based access in which TDMA (X time domain resources) and FDMA (Y freq. domain resources) are assumed.

Fig.1 Slotted-ALOHA based access: TDMA (X time domain resources), FDMA (Y freq. domain resources)

An opportunity of time-frequency resource for A-IoT device(s) to perform access (e.g., transmitting the A-IoT Msg1 by the device) is defined as “Access Occasion”. A set of access occasion(s) (referred to as an Access Occasion Set in Fig.1) for different A-IoT device(s) is scheduled by the reader. It is evident that A-IoT paging message indicates the start of the first “Access Occasion Set” that follows it. However, how to indicate the start of subsequent “Access Occasion Set” is still open.

RAN2#129 meeting agreed that “the A-IoT paging message can include a number of msg1 resources.” In our understanding, a number of msg.1 resources included in the A-IoT paging message is represented by followings:

(1)	the number of Access Occasion Set (in time domain)

(2)	the number of time domain resources within an access occasion set (X)

(3)	the number of frequency domain resources within an access occasion set (Y)

A number of msg.1 resources included in the A-IoT paging message is represented by followings:

- - the number of Access Occasion Set in time domain (M)

- - the number of time domain resources within an access occasion set (X)

- - the number of frequency domain resources within an access occasion set (Y)

Upon receiving the above three parameters, devices with matching filter criteria pick three random values within the range (0, M-1), (0, X-1) and (0, Y-1), respectively. Assuming time synchronization is maintained within an access occasion set, in our understanding, only one counter is needed for determination of the Access occasion set (similar to the slot counter in RFID).

Upon receiving A-IoT paging message, devices with matching filter criteria pick three random values within the range (0, M-1), (0, X-1) and (0, Y-1) respectively.

One counter is needed for determination of the Access Occasion set (similar to the slot counter in RFID).

If a device, in response to the A-IoT paging message, loads its Access Occasion Set counter with zero, it replies to the A-IoT paging message immediately (i.e., using the randomly picked time/frequency domain resources within the first Access Occasion Set). Otherwise, the device remains silent.

An R2D message is assumed to indicate the start of one access occasion set (like QueryRep in RFID).

An R2D message is used to indicate the start of an access occasion set, except for the one that follows immediately after an A-IoT paging message.

The expected device behavior upon receiving such an R2D message is:

-	decrement its Access Occasion Set counter by 1, and

-	if the Access Occasion Set counter=0 after the decrement, reply to the A-IoT paging message immediately (i.e., using the randomly picked time/frequency domain resources within the current Access Occasion Set).

The expected device behavior upon receiving such an R2D message is:

- - decrement its Access Occasion Set counter by 1, and

- - if the Access Occasion Set counter=0 after the decrement, reply to the A-IoT paging message immediately (i.e., using the randomly picked time/frequency domain resources within the current Access Occasion Set).

Regarding the R2D message, two potential ways were discussed at RAN#129 meeting:

2. From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later:

- Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or

- Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field.

As observed in Observation 4, the expected device behavior upon receiving the R2D message, which indicates the start of an access occasion set, differs from the behavior upon receiving the A-IoT paging message. We prefer to introduce a new R2D message.

Introduce a new R2D message to indicate the start of an access occasion set, except for the one that follows immediately after an A-IoT paging message.

- Conclusion

Observation 1:	From the device's perspective, there could be the following cases of receiving subsequent paging messages:

a)	After the device has responded a paging; it then receives a subsequent paging associated to the same service, which contains no identifier (intends for all devices);

b)	After the device has responded a paging; it then receives a subsequent paging associated to the same service, which contains the identifier of the device.

Observation 2:	Upon receiving A-IoT paging message, devices with matching filter criteria pick three random values within the range (0, M-1), (0, X-1) and (0, Y-1) respectively.

Observation 3:	One counter is needed for determination of the Access Occasion set (similar to the slot counter in RFID).

Observation 4:	An R2D message is used to indicate the start of an access occasion set, except for the one that follows immediately after an A-IoT paging message.

Observation 5:	The expected device behavior upon receiving such an R2D message is:

- decrement its Access Occasion Set counter by 1, and

- if the Access Occasion Set counter=0 after the decrement, reply to the A-IoT paging message immediately (i.e., using the randomly picked time/frequency domain resources within the current Access Occasion Set).

Proposal 1:	When the subsequent paging is for all devices (i.e., CBRA), device should skip sending the response to this subsequent paging if the device considers it had successfully responded the same service before.

Proposal 2:	Device should send the response to a subsequent paging regardless of whether it has been responded successfully, if the subsequent paging explicitly indicates the identifier of the device.

Proposal 3:	For a service identified by a transaction ID, the device may store status information about whether the service has been successfully responded, or whether re-access is needed.

Proposal 4:	If device considers it had successfully responded the received service before or re-access status of the service is “not needed” (i.e., device has transmitted msg3 and do not receive a "NACK" indication), device should avoid duplicated response.

Proposal 5:	Paging message need to implicitly or explicitly indicate whether RA resources indicated in paging is for CBRA or CFRA.

Proposal 6:	For cases that paging message only includes identity for a single device, RA resources indicated in paging is for CFRA.

Proposal 7:	For contention free RA configuration when paging message contains multiple device identifiers, random access resource indicated in paging message are mapped to devices according to the ranking of the device identifier in the list.

Proposal 8:	A number of msg.1 resources included in the A-IoT paging message is represented by followings:

- the number of Access Occasion Set in time domain (M)

- the number of time domain resources within an access occasion set (X)

- the number of frequency domain resources within an access occasion set (Y)

Proposal 9:	Introduce a new R2D message to indicate the start of an access occasion set, except for the one that follows immediately after an A-IoT paging message.

- References

[1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Huawei (moderator, RAN1 Vice-Chair).

[2] TR 38.769, Study on solutions for ambient IoT (Internet of Things)