3GPP TSG-RAN WG2 Meeting #129bis		     R2-2502291

Wuhan, CN: 7th Apr – 11th Apr 2025

Source:	Qualcomm Incorporated

Title:	On Network Side Data Collection

Agenda Item:	8.1.3 NW Side Data Collection

Document for:	Discussion and Decision

# Introduction & Background

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured in the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, in RAN #102, a new Work Item on artificial intelligence/machine learning for NR air interface has been approved [3].

This contribution paper discusses the data collection for the network side model training for beam management, CSI Feedback, and positioning enhancements use cases.

# Data Collection for Network Side Models

This section discusses the following:

- Data collection for network-side model training for beam management use case,

- Data collection for network-side model training for positioning enhancements use case.

## 2.1   Data Collection for Network-side Models for the Beam Management

In this section, considering RAN2 agreements for NW-side data collection, we discuss the following open issues

- UE capability considerations

- Further discussion on event-triggered data collection,

- Multiple configurations and dynamic activation/deactivation,

- Handling data collection configuration and report during UE mobility.

This section discusses the FFS issues mentioned above.

### 2.1.1	UE Capability Considerations

The training data configuration consists of measure targets and logging configuration (e.g., periodicity, event triggers for logging). The configuration/activation of data collection configuration for NW-side training should be subject to UE capability. As the CSI processing unit (CPU) is shared across different CSI-related tasks, when configuring/activating the training data collection at the UE, the network should ensure that SSB/CSI-RS configured for measurements across different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring) and different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes) lies within the maximum number of SSB/CSI-RS measurements supported at the UE.

Proposal 1: The network should continue to ensure that the maximum number of SSB/CSI-RS configured/activated for measurements lies within the maximum number of SSB/CSI-RS measurements supported at the UE per slot and per FR. That includes SSB/CSI-RS used for

- Different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes), and

- Different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring), and

- Different AI/ML features (e.g., beam management, CSI prediction, CSI Feedback).

### 2.1.2	Event-Trigger Data Collection

In the RAN2#129 meeting [7], for the event-triggered measurement logging, RAN2 agreed on the following

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

- Low power bit indication is supported

- Data availability indication is supported.  FFS when this would be triggered

Note that the UE performs L1 measurements upon the trigger of the configured event which may occupy the CSI processing unit. TS 38.214 [10] outlines the rule for computing CPU occupancy (number and timelines of CPU occupancy associated with different CSI resource/resource set measurements and reporting). For the configuration, activation, or deactivation of different CSI resource/resource set measurements and reporting, the network needs to be aware of the CPU occupancy at the UE. Therefore, when the event is triggered the UE should send an indication to the network. Similarly, UE should send an indication to the network when the triggered event ends. Furthermore, the START and STOP indications can be used by the network for retrieving the logged data at the UE.

Observation 1:  The UE performs L1 measurements upon the trigger of the configured event which may occupy the CSI processing unit (TS 38.214).

Observation 2: For the configuration, activation, or deactivation of different CSI resource/resource set measurements and reporting, the network needs to be aware of the CPU occupancy at the UE.

Proposal 2: UE sends START and STOP indications to the network to indicate when the configured event for training data collection (L1 measurements) gets triggered and ends.

Proposal 3: The START and STOP indications can be used by the network for retrieving the logged data at the UE.

### 2.1.3	Multiple Configuration and Dynamic Activation/Deactivation

In the RAN2, the following are FFS issues associated with data collection configuration for network-side model training,

- Whether the UE can be configured with multiple RS configurations for network-side model training,

- Whether dynamic activation/deactivation is supported

Note that the performance of AI/ML functionalities/models depends on the quality/distribution of the training data. For example, with periodic data collection, the network may be able to collect the training data across different beams of the serving cells. However, if a UE is static and configured with periodic data collection, then the collected training data may be redundant. Note that when redundant data is used for training, it may create significant biases. One solution may be that redundancy is removed during the training by the NW-side training server. However, this will require a huge amount of training data to be collected at the UE, as after removing the redundancy the network side may end up with a significantly low amount of meaningful training data. Therefore, the following issues may happen with periodic data collection:

- Huge amount of training data collection at the UEs,

- Significantly high redundancy causing biases during training, if not handled by the training server properly.

Observation 3: Periodic data collection has the following issues,

- Requires a huge amount of training data collection at the UEs, as there may exist significantly high redundancy,

- The network-side training server needs to handle redundancy in the training data, otherwise, it may cause biases in the trained models.

As described in Fig. 2.1.2-1 and Fig. 2.1.2-2, for the L1-RSRP predictions of top-1/top-K beams, the training data should be independent and identically distributed (i.i.d) across beams. We need a solution that alleviates/minimizes the redundancy in the data collection and captures a good representation across beams, across beam center vs beam center, and others. This can be achieved by introducing randomness in the training data logging/reporting.

Fig. 2.1.3-1: An example of training data collection where the training data is distributed (equally) across the beams, across the beam center, beam edge, etc.

This section further discusses the FFS issues (discussed above) associated with the data collection configuration.

As discussed previously, to achieve adequate AI/ML functionality/model performance, the redundancy in the training data should be minimized. This can be achieved by introducing randomness in the training data logging/reporting. Following the above observation 2 – observation 4 from the legacy CSI framework, Fig. 2.1.3-2 illustrates a potential solution for redundancy in the training data collection.

Fig. 2.1.3-2: Training data collection: UE can be configured with periodic/semi-persistent/aperiodic RS configuration, but the CSI report configuration is semi-persistent/aperiodic.

Proposal 4: Dynamic activation/deactivation of training data collection and logging should be used to randomize the training data collection and logging, following semi-persistent and aperiodic CSI reporting as the reference.

### 2.1.4	Handling Data Collection Configuration and Reporting during UE Mobility

In the RAN2#129 meeting [7], RAN2 agreed on the following for handling logged training data and configuration

- UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

- FFS how to handle idle/inactive and RLF cases and whether we have a unified.

Note that during the handover, the source node may be able to determine whether the target/candidate cells are from the same infra-vendor or different, for example, based on the cell ID. Furthermore, in the legacy procedure, the UE sends the availability indication in the RRC complete messages only if RPLMN belongs to the PLMN identity list stored in the UE variable for the report. PLMN identity list indicates the list of equivalent PLMNs where UE is allowed to send the SON and logged MDT availability indication and report. This was designed to avoid inter-PLMN leakage. For the AI/ML data collection, there may be concerns with

- Inter PLMN leakage (see Fig 2.1.4-1), and

- Inter-vendor leakage (see Fig 2.1.4-2).

Observation 4: In the legacy SON and logged MDT report, to avoid inter-PLM leakage UE checks whether RPLMN belongs to the PLMN identity list stored in the UE variable for the report before sending the availability indication and report.

Observation 5: For the AI/ML data collection, there may be concerns with

- Inter PLMN leakage (see Fig 2.1.4-1), and

- Inter-vendor leakage (see Fig 2.1.4-2).

Fig. 2.1.4-1: OAM-centric data collection, whether the logged training data can be reported to other PLMN or other-infra-vendor cells if they belong to the same/different PLMN.

Fig. 2.1.4-2: gNB-centric data collection, whether the logged training data can be reported to other infra-vendor cells if they belong to the same/different PLMN.

RAN2 should consider a common solution to retrieve the logged training data during RRC Reconfiguration (with HO command), RRC Reestablishment, RRC Establishment, or RRC Resume. Following the legacy logged MDT configuration, the UE should be configured with a cell ID list or PLMN ID list where UE can send availability indications and data collection reports.

Proposal 5: RAN2 should consider a common solution to retrieve the logged training data during RRC Reconfiguration (with HO command), RRC Reestablishment, RRC Establishment, or RRC Resume.

Proposal 6: Following the legacy logged MDT configuration, the UE should be configured with a cell ID list (to mitigate inter-vendor information leakage) or PLMN ID list (to mitigate inter-PLMN information leakage) where UE can send availability indications and data collection reports.

#### 2.1.4.1 Handing of Training Data Collection Configuration during UE Mobility

##### 2.1.4.1.1 UE Behaviour Related to the Training Data Collection Configuration

In the legacy CSI framework, the CSI-MeasConfig is used to configure CSI-RS (reference signal) belonging to the serving cell. Following the legacy CSI framework, the training data collection configuration for the beam management may be released upon handover/RLF/RRC state transitions from CONNECTED to IDLE/INACTIVE states. Therefore, UE is not configured with area configuration for the measurements.

Proposal 7: UE is not configured with area configuration for the measurement of resources or resource sets for training data collection, as these configurations are cell-specific.

##### 2.1.4.1.2 Handing of Training Data Collection Configuration at the Network

The OAM may indicate cell(s)/frequency(ies) on which the training data collection should be performed to the NR-RANs. However, as the CSI resource configuration is serving cell-specific, therefore during handover/Reestablishment/RRC state transition, the new NG-RAN should provide data training collection configuration based on area configuration indicated by OAM.

Proposal 8: For OAM-centric data collection (i.e., for management-based MDT), the OAM may indicate cell(s)/frequency(ies) to the NG-RANs on which the training data collection should be performed.

Proposal 9: During the handover/Reestablishment/RRC state transition, the new NG-RAN should provide data training collection configuration based on area configuration indicated by OAM to the NG-RANs.

## 2.2   Data Collection for Network-side Model for Positioning Enhancements

### 2.2.1	Network-based positioning with gNB-side model (Case 3a) and LMF-side model (Case 3b)

At RAN#107, the previously 2nd priority positioning cases 2a and 2b have been removed from the Work Item objectives [3]. Therefore, data collection for network side models is applicable to network-based positioning enhancements only (Case 3a/3b). For both, Case 3a and Case 3b the UE only transmits positioning SRS, and therefore, no additional RAN2 specification impacts are foreseen (unless RAN1 defines new/additional SRS parameter). RAN2 already agreed at RAN2#126:

- RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

Case 3a and Case 3b are currently discussed in RAN3, which already made progress on these two cases.

Observation 6: Procedures to support AI/ML positioning Case 3a and Case 3b are discussed in RAN3 and there is no additional RAN2 specification work for Case 3a/3b foreseen.

# Conclusion

Data collection and reporting for beam management use case

Proposal 1: The network should continue to ensure that the maximum number of SSB/CSI-RS configured/activated for measurements lies within the maximum number of SSB/CSI-RS measurements supported at the UE per slot and per FR. That includes SSB/CSI-RS used for

- Different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes), and

- Different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring), and

- Different AI/ML features (e.g., beam management, CSI prediction, CSI Feedback).

Observation 1:  The UE performs L1 measurements upon the trigger of the configured event which may occupy the CSI processing unit (TS 38.214).

Observation 2: For the configuration, activation, or deactivation of different CSI resource/resource set measurements and reporting, the network needs to be aware of the CPU occupancy at the UE.

Proposal 2: UE sends START and STOP indications to the network to indicate when the configured event for training data collection (L1 measurements) gets triggered and ends.

Proposal 3: The START and STOP indications can be used by the network for retrieving the logged data at the UE.

Observation 3: Periodic data collection has the following issues,

- Requires a huge amount of training data collection at the UEs, as there may exist significantly high redundancy,

- The network-side training server needs to handle redundancy in the training data, otherwise, it may cause biases in the trained models.

Proposal 4: Dynamic activation/deactivation of training data collection and logging should be used to randomize the training data collection and logging, following semi-persistent and aperiodic CSI reporting as the reference.

Observation 4: In the legacy SON and logged MDT report, to avoid inter-PLM leakage UE checks whether RPLMN belongs to the PLMN identity list stored in the UE variable for the report before sending the availability indication and report.

Observation 5: For the AI/ML data collection, there may be concerns with

- Inter PLMN leakage (see Fig 2.1.4-1), and

- Inter-vendor leakage (see Fig 2.1.4-2).

Proposal 5: RAN2 should consider a common solution to retrieve the logged training data during RRC Reconfiguration (with HO command), RRC Reestablishment, RRC Establishment, or RRC Resume.

Proposal 6: Following the legacy logged MDT configuration, the UE should be configured with a cell ID list (to mitigate inter-vendor information leakage) or PLMN ID list (to mitigate inter-PLMN information leakage) where UE can send availability indications and data collection reports.

Proposal 7: UE is not configured with area configuration for the measurement of resources or resource sets for training data collection, as these configurations are cell-specific.

Proposal 8: For OAM-centric data collection (i.e., for management-based MDT), the OAM may indicate cell(s)/frequency(ies) to the NG-RANs on which the training data collection should be performed.

Proposal 9: During the handover/Reestablishment/RRC state transition, the new NG-RAN should provide data training collection configuration based on area configuration indicated by OAM to the NG-RANs.

Data collection and reporting for positioning enhancements use case:

Observation 6: Procedures to support AI/ML positioning Case 3a and Case 3b are discussed in RAN3 and there is no additional RAN2 specification work for Case 3a/3b foreseen.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-250792, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #107, March 2025

- Meeting Report, “RAN2#126 meeting report”

- Meeting Report, “RAN2#127 meeting report”

- Meeting Report, “RAN2#127bis meeting report”

- Meeting Report, “RAN2#129 meeting report”

- [POST128][019][AI PHY] NW side data collection (Nokia) email discussion report

- TS 38.331, RRC Specification

- TS 38.214

- 3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

- 3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".