3GPP TSG-RAN WG2 Meeting #129bis	R2-2502294

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	7.0.2.17

Source: 	CATT

Title: 	Discussion on the reply LS from RAN1/RAN4 on soft satellite switch

Document for: 	Discussion and Decision

Introduction

In RAN2#128, RAN2 discussed an issue on SSB pattern for soft satellite switch scenario [1] and sent LS [2] to RAN1 and RAN4 with the following question.

To RAN4, RAN1 ACTION: RAN2 kindly asks RAN4 and RAN1 whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

RAN1 and RAN4 replied in [3][4]. Based on their replies, this contribution further discusses this issue.

Discussion

2.1	Discussion on the Questions from RAN1

RAN1 replies to our LS as follows:

RAN1 has discussed the question in LS R2-2410978, and RAN1 would like to ask RAN2 for further clarification on the question and assumptions below: - Is RAN2 asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service, or whether RAN2 assumes that some RAN1 specification work may be needed? - RAN1 assumes that the meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset). - RAN1 assumes that the SIB19 of the source satellite includes ephemeris information of the target satellite, and that such information will not get outdated by the time the satellite switch occurs.

For the first bullet, we think RAN2 can clarify that RAN2 is asking whether a Rel-18 UE can maintain the pre-acquired DL sync in the condition in question based on current Rel-18 RAN1 specifications, i.e., the former understanding is right. This is a key point that needs to be confirmed by related working groups, as otherwise the target satellite sync pre-acquisition will not make big sense and the benefit to support soft-switching vanishes. At this stage, RAN2 does not expect RAN1 to undertake further specification work. In addition, RAN2 can also confirm the RAN1 assumptions in the second bullet and third bullet above.

Proposal 1: RAN2 replies to RAN1 questions as follows:

- For question bullet 1, the former understanding (i.e. whether UE can maintain the pre-acquired target satellite DL Sync based on current Rel-18 RAN1 specifications) is correct;

- For question bullet 2 and 3, RAN1 assumptions are confirmed.

2.2	Discussion on the Questions from RAN4

RAN4 replies to our LS as follows:

Under following assumptions, - Starting boundaries of SFN0 of the source satellite and the target satellite are aligned at the uplink time synchronization reference point; - The parameter ssb-TimeOffset is the relative offset between the SSB of the source satellite and the SSB of the target satellite during [t-ServiceStart, t-Service]; RAN4 reached the conclusion that the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. RAN4 would like to note that the temporarily SSB pattern shift (i.e., subject to ssb-TimeOffset) during [t-ServiceStart, t-Service] and shifting back after t-Service was not considered during RAN4 requirement definition and it may require additional UE processing for shifting. Also, it may have impact to UE performing initial access during [t-ServiceStart, t-Service]. It is also noted that the RAN4 requirements are defined assuming the parameter ssb-TimeOffset is the relative time offset between SSB of the source satellite and the SSB of the target satellite with same SSB index. In addition, RAN4 would like RAN2 to confirm which one of the following is the correct understanding of parameter ssb-TimeOffset, - Option 1: the relative offset between the SSB of the source satellite and the SSB of the target satellite, and the starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point - Option 2: the relative offset between the starting boundaries of SFN0 of the source satellite and the target satellite Regarding the assumptions above, if option 1 is the correct understanding, RAN4 kindly ask RAN1/2: 1) Whether it is feasible to configure ssb-TimeOffset with values other than multiple of 5. 2) How the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service].

Regarding RAN4's question on which understanding corresponding to ssb-TimeOffset is correct, RAN2 can confirm that Option 1 is correct. Option 2 actually refers to the shifted cell timing option as previously discussed in RAN2#128 [1], which was already excluded by RAN2.

Below RAN4's further questions based on option 1 were discussed:

1) Whether it is feasible to configure ssb-TimeOffset with values other than multiple of 5.

Following the design of the existing SSB pattern, the transmission of an SSB burst set is restricted within a half frame and the specific SSB occasions appear at certain positions within a half frame as determined by the SSB SCS [5]. This means that, to follow the legacy SSB design, the ssb-TimeOffset should not be configured with values other than multiple of 5 in the case of satellite switch with re-sync.

2) How the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service].

We understand RAN4 is asking how the gNB maintains the DL timing of the serving cell via source satellite and target satellite during [t-ServiceStart, t-Service]. The following explanation may be helpful to RAN4: The cell’s timing via source satellite and target satellite are aligned at the uplink time synchronization reference point. The gNB on the ground sends SSBs to source satellite and target satellite at different half frames (with a time offset ssb-TimeOffset), referencing the timing at the uplink reference point. The PBCH received from the source satellite and target satellite configures the actual half-frame indication and SFN indication values where the SSBs were sent to them respectively (with the uplink reference point as the timing reference)..

Proposal 2: RAN2 replies to RAN4 questions as follows:

- RAN2 first confirms that Option 1 related to RAN4 understanding on ssb-TimeOffset is correct, and for the 2 questions in the case of Option 1:

  - Answer to question 1): RAN2 confirms that to follow the legacy SSB design, the ssb-TimeOffset should not be configured with values other than multiple of 5, in the case of satellite switch with re-sync.

  - Answer to question 2): The gNB on the ground sends SSBs to the source satellite and the target satellite at different half frames (with the time offset ssb-TimeOffset) referring to the timing at uplink reference point. The PBCH received from the source satellite and target satellite configure the actual half-frame indication and SFN indication values where the SSBs were sent to them respectively (with the uplink reference point as the timing reference).

2.3	Further RAN2 considerations based on RAN1/RAN4 replies

Based on the RAN1/RAN4 reply LS, we would like to have a further discussion on the SSB shifting issue from RAN2’s perspective. We prefer not supporting the temporarily shifted SSB pattern mechanism with the following reasons.

- RAN4 has not been considering such a mechanism and thinks this may require additional UE processing for shifting and may impact the initial access procedure. Considering that we are currently in the maintenance phase of Rel-18, it is not suitable for major modifications in whichever WGs.

RAN4 would like to note that the temporarily SSB pattern shift (i.e., subject to ssb-TimeOffset) during [t-ServiceStart, t-Service] and shifting back after t-Service was not considered during RAN4 requirement definition and it may require additional UE processing for shifting. Also, it may have impact to UE performing initial access during [t-ServiceStart, t-Service]. It is also noted that the RAN4 requirements are defined assuming the parameter ssb-TimeOffset is the relative time offset between SSB of the source satellite and the SSB of the target satellite with same SSB index.

- Companies' concern at previous meeting was that after t-Service, the NW needs to reconfigure SMTC for the serving cell and/or neighbour cell. However, the reconfigure SMTC issue can be solved by NW implementation. For instance, the NW can already configure multiple SMTC windows, considering the SSB positions of both source satellite and target satellite, so that reconfiguration of the SMTC after soft switching is not needed.

To this end, RAN2 can already conclude that there is no need to support temporarily SSB pattern shift for the soft satellite switching, and also signal this information to RAN1/4.

Proposal 3: RAN2 concludes that there is no need to support temporary SSB pattern shift solution and sends this conclusion in the reply LS to RAN1 and RAN4.

We prepare a draft LS reply in the Annex, which can be taken as the baseline for further LS correspondence with RAN1/4, if above agreements are agreeable.

Proposal 4: Take the draft LS in Annex as baseline.

Conclusion

In this contribution, DL coverage enhancement is further discussed with the corresponding observations and proposals listed below:

Proposal 1: RAN2 replies to RAN1 questions as follows:

- For question bullet 1, the former understanding (i.e. whether UE can maintain the pre-acquired target satellite DL Sync based on current Rel-18 RAN1 specifications) is correct;

- For question bullet 2 and 3, RAN1 assumptions are confirmed.

Proposal 2: RAN2 replies to RAN4 questions as follows:

- RAN2 first confirms that Option 1 related to RAN4 understanding on ssb-TimeOffset is correct, and for the 2 questions in the case of Option 1:

  - Answer to question 1): RAN2 confirms that to follow the legacy SSB design, the ssb-TimeOffset should not be configured with values other than multiple of 5, in the case of satellite switch with re-sync.

  - Answer to question 2): The gNB on the ground sends SSBs to the source satellite and the target satellite at different half frames (with the time offset ssb-TimeOffset) referring to the timing at uplink reference point. The PBCH received from the source satellite and target satellite configure the actual half-frame indication and SFN indication values where the SSBs were sent to them respectively (with the uplink reference point as the timing reference).

Proposal 3: RAN2 concludes that there is no need to support temporary SSB pattern shift solution and sends this conclusion in the reply LS to RAN1 and RAN4.

Proposal 4: Take the draft LS in Annex as baseline.

Reference

- R2-2410971	Report for [AT128][301][R18 NR NTN] SMTC issues (Huawei)

- R2-2410978	LS soft satellite switch with resync (Huawei)

- R1-2501606	Reply LS on soft satellite switch with re-sync

- R4-2502701	Reply LS on soft satellite switch with re-sync

- 3GPP TS 38.213: "Physical layer procedures for control"

- Annex: Draft LS reply to RAN1/4

Title:	Reply LS on soft satellite switch with re-sync

Response to:	R1-2501606, R4-2502701

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	RAN2

To:	RAN1, RAN4

Cc:	-

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN2 thanks RAN1 and RAN4 for the reply LSs on R2-2410978. And RAN2 would like to answer the questions proposed by RAN1 and RAN4 as following.

1. For the questions from RAN1:

RAN1 has discussed the question in LS R2-2410978, and RAN1 would like to ask RAN2 for further clarification on the question and assumptions below: - Is RAN2 asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service, or whether RAN2 assumes that some RAN1 specification work may be needed? - RAN1 assumes that the meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset). - RAN1 assumes that the SIB19 of the source satellite includes ephemeris information of the target satellite, and that such information will not get outdated by the time the satellite switch occurs.

RAN2’s answers:

- For the question in the first bullet, RAN2 is asking whether a Rel-18 UE can maintain the pre-acquired DL sync from the target satellite in the related condition based on current Rel-18 RAN1 specifications, i.e., the former understanding is right.

- For the second and third bullets, RAN2 confirms the RAN1 assumptions are correct.

2. For the questions from RAN4:

In addition, RAN4 would like RAN2 to confirm which one of the following is the correct understanding of parameter ssb-TimeOffset, - Option 1: the relative offset between the SSB of the source satellite and the SSB of the target satellite, and the starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point - Option 2: the relative offset between the starting boundaries of SFN0 of the source satellite and the target satellite Regarding the assumptions above, if option 1 is the correct understanding, RAN4 kindly ask RAN1/2: 1) Whether it is feasible to configure ssb-TimeOffset with values other than multiple of 5. 2) How the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service].

RAN2’s answers:

RAN2 confirms that for the understanding of parameter ssb-TimeOffset, Option 1 is correct.

Regarding the questions in the case of Option 1:

- For question 1), RAN2 confirms that to follow the legacy SSB design, the ssb-TimeOffset should not be configured with values other than multiple of 5, in the case of satellite switch with re-sync.

- For question 2), RAN2 understands that the gNB on the ground sends SSBs to the source satellite and the target satellite at different half frames (with the time offset ssb-TimeOffset) referring to the timing at uplink reference point. The PBCH received from the source satellite and target satellite configure the actual half-frame indication and SFN indication values where the SSBs were sent to them respectively (with the uplink reference point as the timing reference).

Based on the reply from RAN1 and RAN4, RAN2 had a further discussion and agreed that temporary SSB pattern shift is not supported by RAN2.

# 2	Actions

To RAN1 and RAN4:

ACTION: 	RAN2 respectfully asks RAN1 and RAN4 to take the above information into account.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #130	2025-05-19 - 2025-05-23 	Malta, MT

TSG RAN WG2 Meeting #131	2025-08-25 - 2025-08-29  	India, IN