3GPP TSG-RAN WG2 Meeting #129bis	R2-2502338

# Wuhan, China, April 7th - 11th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0979 | rev | - | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on eDRX configuration in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2025-3-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current specification, the NW is allowed to release a UE to RRC_INACTIVE state with eDRX for RAN paging when the UE is configured with emergency PDU session. However, in this case, it is unclear whether the UE should follow the RAN eDRX or not, if it follows, then it may result in long delay for the MT call back of the emergency call. Regarding above issue, RAN2 sent LS (R2-2411041) to SA2 and based on reply LS from SA2 (R2-2501752/S2-2502427), SA2 response that in SA2 specifications, they already specify that each PLMN/operator can reserve a specific ARP value for emergency PDU session, and the ARP value will be indicated to RAN node when the PDU session is established. So, based on the received ARP value, RAN node can be aware of the presence of emergency PDU session. Therefore, we propose to address the identified issue by not releasing the UE configured with emergency PDU session to RRC_INACTIVE with eDRX for RAN paging. This CR is provided to capture the expected NW behaviour when the UE is configured with emergency PDU session. On whether the UE configured with emergency PDU session can be released to RRC_IDLE with/without eDRX for CN paging, no change is needed because it is already covered by existing specification, i.e. the UE in RRC_IDLE does not apply eDRX for CN paging. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In chapter 9.2.10, to add text “When the UE is configured with emergency PDU session by upper layer, the UE is not expected to be released to RRC_INACTIVE with eDRX for RAN paging”. Impact Analysis Impacted 5G architecture options: NR SA, NR-DC, NE-DC Impacted functionality: eDRX configuration in RRC_INACTIVE Inter-operability: - If the UE is implemented according to this CR but the network is not, the NW may release the UE to RRC_INACTIVE with eDRX for RAN paging when the UE is configured with emergency PDU session. In this case, it is unclear whether the UE should ignore the eDRX configuration, but there is no inter-operability issue. - If the network is implemented according to this CR but the UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If this CR is not approved, the network may release the UE to RRC_INACTIVE with eDRX for RAN paging when the UE is configured with emergency PDU session. Then the delay of the MT call back that follows the emergency call will be too long for the emergency service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

9.2.10	Extended DRX for RRC_IDLE and RRC_INACTIVE

When extended DRX (eDRX) is used, the following applies:

-	For RRC_INACTIVE, eDRX configuration for RAN paging is decided and configured by NG-RAN. In RRC_INACTIVE the UE monitors both RAN and CN paging; When the UE is configured with emergency PDU session by upper layer, the UE is not expected to be released to RRC_INACTIVE with eDRX for RAN paging;

-	For RRC_IDLE, eDRX for CN paging is configured by upper layers. In RRC_IDLE the UE monitors only CN paging;

-	Information on whether eDRX for CN paging and RAN paging is allowed on the cell is provided separately in system information;

-	The maximum value of the eDRX cycle is 10485.76 seconds (2.91 hours) for RRC_IDLE and 10.24 seconds for RRC_INACTIVE, while the minimum value of the eDRX cycle is 2.56 seconds for both RRC_IDLE and RRC_INACTIVE;

-	The hyper SFN (H-SFN) is broadcast by the cell and increments by one when the SFN wraps around;

-	Paging Hyperframe (PH) refers to the H-SFN in which the UE starts monitoring paging according to DRX during a Paging Time Window (PTW) used in RRC_IDLE. The PH and PTW are determined based on a formula (see TS 38.304 [10]) that is known by the AMF, UE and NG-RAN;

-	H-SFN, PH and PTW are used if the eDRX cycle is greater than 10.24 seconds;

-	When the RRC_IDLE eDRX cycle is longer than the system information modification period, the UE verifies that stored system information remains valid before resuming/establishing an RRC connection.

End of change