3GPP TSG-RAN WG2 Meeting #129bis 	R2-2502343

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.2.4

Source:	Panasonic

Title:	Discussion on AS ID

Document for:	Discussion

# Introduction

In previous meeting, the following agreements have been achieved for AS ID.

Agreements - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - From device perspective, it is only required to use one AS ID. - CFRA is not supported for group ID - RAN2 assumes, AS ID is needed for CFRA at least for inventory + command procedure - For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: - Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. - Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 - Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2

A post meeting email discussion has been carried out to further discuss open issues related to AS ID [1]. In the following, we discuss our views based on the outcome of the post email discussion.

# Discussion on AS ID

AS ID assignment for CBRA

The following options for CBRA have been discussed in post email discussion with pros and cons.

| Options | Option 1: Msg 2 for AS ID assignment; | Option 3 (reuse the number of CFRA): “New Msg” for AS ID assignment | Option 4 (reuse the number of CFRA): Msg 4 (First Command message) for AS ID assignment |
| --- | --- | --- | --- |
| Pros | - The AS ID assigned in the “Msg2 “can be used for “the first Command message i.e. Msg 4 and subsquent Command messages” to associate the resources and identify the device; - Not need to introduce new procedures | - The AS ID assigned in the “new Msg “can be used for “the first Command message, i.e. Msg 4 Comand message, and “subsequent R2D Command messages” to associate the resources and identify the device; - No impact on Msg 2; | - The AS ID assigned in the “Msg 4 Command message “can be used for “Msg 6” and subsequent R2D Command message” to associate the resources and identify the device; - No impact on Msg 2; - Not need to introduce new procedures; |
| Cons | - Additional signalling overhead in Msg 2, especially when multiplexing is supported; - Not useful for Inventory only case, may result in different msg2 message types or waste resources; | - Additional delay/overhead/procedure due to the new message; - Device ID or RN16 needs to be contained in “new Msg” in order to identify the device, to associate with the newly assigned AS ID in the new Msg; FFS on RN 16 collision case | - Device ID or RN16 needs to be contained in “Msg 4 Command message” in order to identify the device, to associate with the newly assigned AS ID in the new Msg; FFS on RN 16 collision case - The reader has to reuse RN16 or device ID for the first Command message; |

In the post email discussion, no company supported option 3, Therefore, option 3 can be removed from further consideration. There were split supports for option 1 and option 4 among companies. Here we would like to point out a potential issue on option 1 which was missed during email discussion.

For option 1, Msg 2 for AS ID assignment schedules Msg 3. In case that Msg 3 is not received by the reader, reader cannot distinguish the following two reasons (see Figure 1 below):

-	Reason 1: Msg 2 is missed by device, meaning that AS ID is not acquired by device;

-	Reason 2: Device has successfully received Msg 2 but Msg 3 delivery fails, meaning that AS ID is acquired by device.

Figure 1. Failure of Msg 3 creates reader’s confusion on AS ID availability at device

Based on the following RAN2 agreement, the random ID will be used for reader to request the retransmission of Msg3.

Agreements 1 RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID. 2 If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported).

As a result, if device has successfully received Msg2 carrying AS ID assignment, device needs to keep both random ID and AS ID, in order to handle the Msg3 retransmission (i.e. to be reachable by reader using random ID). This increases the memory requirement and complexity of the device.

By contrast, if option 4 (Msg4 for AS ID assignment) is adopted, the inventory procedure (up to and include Msg3) will not be affected. The device only needs to maintain random ID during the inventory procedure. Once device receives Msg4 carrying AS ID, device will replace the random ID with the newly received AS ID and proceed with the transmission of scheduled D2R (i.e. “Msg5”). If “Msg5” is not received by reader, reader can schedule retransmission (if supported) using AS ID or simply terminate the procedure by recognizing this is an error case.

Observation 1. For CBRA, Option 1 (Msg2 for AS ID assignment) requires device to maintain both random ID and newly assigned AS ID in order to handle possible Msg 3 retransmission. This increases unnecessary complexity and memory requirements even for devices to perform inventory-only operation.

Proposal 1. RAN2 agree Option 4 (Msg 4 (First Command message) for AS ID assignment) for CBRA.

AS ID assignment for CFRA

The following options for CFRA have been discussed in post email discussion with pros and cons.

| Options | Option 2: the device includes a random ID in Msg 1 (Inventory Response). Same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID (The assignment can be done via Option 3 or Option 4). | Option 3: “New Msg” for AS ID assignment, work with/without option 2 | Option 4: Msg 2 (Command message) for AS ID assignment, work with/without option 2 |
| --- | --- | --- | --- |
| Pros | - The random ID included in Msg1 (Inventory Response) can be used in “New Msg2” or “Msg 2 Command” to associate the resources and identify the device if the ID can be reused; - If new ID is needed, the RN 16 indicated in Msg1 can be used to identify the device for the new AS ID allocation and associate with the resources. - Align the procedure of CFRA and CBRA as the device always includes the random ID in the first message following paging Note: Opponents think it is not needed since the reader to identify the device based on the allocated resources. | - The AS ID assigned in the “new Msg “can be used for “the first Command message, i.e. Msg 2 Comand message, and “subsequent R2D Command messages” to associate the resources and identify the device; - No impact on Msg 1 (Inventory Response) if option 2 is not supported; | - The AS ID assigned in the “Msg 2 Command message “can be used for “Msg 4” and subsequent R2D Command message” to associate the resources and identify the device; - No impact on Msg 1 (Inventory Response) if option 2 is not supported; - No additional delay/overhead/procedure compared to Option 3 |
| Cons | - Additional signalling overhead in Msg 1 since RN16 is added (Inventory Response); But same message size as Msg3 for CBRA if RAN2 agrees that AS ID is contained in D2R message if available. FFS on RN 16 collision case - RN16 is not needed for Inventory-only procedure; however, from a device perspective, it cannot differentiate whether the CFRA is for Inventory-only or Inventory+Command procedure, therefore it has to always transmit RN16 even if it is not needed (For Inventory Only.). Note: we should avoid optimization on this case in order to simplify the device behavior. | - Additional delay/overhead/procedure due to the new message; - FFS on whether Device ID needs to be contained in “new Msg” in order to identify the device, to associate with the newly assigned AS ID in new Msg if option 2 is not supported; - Additional device complexity caused by different procedure between CBRA and CFRA. | - FFS on whether Device ID needs to be contained in “Msg2” in order to identify the device, to associate with the newly assigned AS ID in Msg2 if option 2 is not supported, i.e. AS ID cannot be used for the first Command message; |

Looking at the rapporteur’s summary on the post email discussion [1], there was no support on option 3. Therefore, option 3 can be removed from further consideration. One big debating point is whether option 4 only is sufficient or option 4 should be combined with option 2. This is related to the FFS: which ID is used to identify the device in option 4 (whether device ID needs to be contained in the assignment message to identify the target device).

Some companies had the opinion that for Rel-19 CFRA supports only a single device. In this case, all R2D messages are targeted at one single device until the service is completed. Therefore, there is no need to include device ID to address the target device for the assignment message. Based on this thinking, those companies supported option 4 only.

However, if this is the case, why do we need to support AS ID for CFRA in the first place? If the device can receive the assignment message (e.g. “Msg2” as in option 4) without address information, device can receive any follow-up R2D command message without need for AS ID for addressing as well.

We want to reiterate the use cases of supporting AS ID for CFRA. It is true that parallel service requests by the same reader is not supported, meaning that the device will not be triggered by another request from the same reader before it completes the ongoing procedure. However, in the multi-reader overlapping area, the device could accidentally receive R2D message from neighboring readers. Note that this may happen even if the neighboring readers transmit R2D message over a different time-frequency resource than the reader that device is “associated with”, because device needs to detect/scan time-frequency resources in order to find out possible R2D. RAN2 is currently discussing whether to use “transaction ID” or “reader ID” to indicate reader specific information. If the R2D message can include such reader specific information, device would be able to differentiate different readers. Otherwise, without AS ID included in R2D message for addressing the target device, the device would be confused by another R2D message from other readers on whether the device is the intended target device or not.

As a further use case, CFRA could be used to trigger multiple devices beyond Rel-19. In such case, having AS ID can provide a simpler way to facilitate the D2R scheduling and R2D reception. The design of Rel-19 can take this into consideration for future prove.

With the above discussion, we can conclude that the R2D assignment message such as “Msg2” for option 4 needs addressing information as well. The addressing options could be:

- Random ID (assuming option 2 is also supported in addition to option 4)

- Device ID

- NAS (temporary) ID

- the time frequency resource of previous PDRCH

The last option would be minimum signaling overhead but is only possible when the specific reader is identified. The dependency of time frequency resource can make future enhancement complicated. Therefore, we don't recommend it. Then comparing the among 3 options other than the last option, using random ID can have less signaling overhead than the other options. Therefore, option 2 can be supported together with option 4.

Proposal 2. RAN2 agree to support both Option 2 (device includes a random ID in Msg 1 (Inventory Response) and Option 4 (Msg 2 (Command message) for AS ID assignment) for CFRA.

# Conclusion

In this paper, we have discussed some remaining issues for AS ID. We have the following observations and proposals.

Observation 1. For CBRA, Option 1 (Msg2 for AS ID assignment) requires device to maintain both random ID and newly assigned AS ID in order to handle possible Msg 3 retransmission. This increases unnecessary complexity and memory requirements even for devices to perform inventory-only operation.

Proposal 1. RAN2 agree Option 4 (Msg 4 (First Command message) for AS ID assignment) for CBRA.

Proposal 2. RAN2 agree to support both Option 2 (device includes a random ID in Msg 1 (Inventory Response) and Option 4 (Msg 2 (Command message) for AS ID assignment) for CFRA.

# Reference

[1] [Post129][036][AIoT] AS ID (Xiaomi) P2_Summary v01.docx, ftp.3gpp.org/Email_Discussions/RAN2/[RAN2#129]/[POST129][036][AIoT] AS ID (Xiaomi)