3GPP TSG-RAN WG2 Meeting#129bis	R2-2502349

Wuhan, CN, April 7–11, 2025

Agenda item:	 8.10.4

Source:		Lenovo

Title:		Discussion on MRO for intra-NTN mobility

Document for:		Discussion and Decision

1	Introduction

RAN2#129 meeting agreed that:

Existing IEs for CHO are used to report the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”.

In this paper, we would further discuss MRO for intra-NTN mobility.

2	Discussion

RAN3#124 meeting agreed MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover, and the following trigger conditions for MRO for intra-NTN mobility shall be considered:

- Measurement-based trigger condition;

- A time-based trigger condition;

- A location-based trigger condition;

- A time-based and Measurement-based combined trigger condition;

- A location-based and Measurement-based combined trigger condition;

For time-based and measurement-based combined trigger condition:

For CHO configured with time-based and measurement-based combined trigger condition, there may be a case that an RLF happens due to the absolute time value (t1-Threshold + duration) arrives, but measurement based CHO execution condition is not fulfilled. Since signal strength/quality difference between NTN cell center and cell edge is small, time-based trigger condition has bigger impacts to mobility performance compared with measurement-based trigger condition in CHO NTN, it is beneficial to enable network know that measurement based CHO execution condition is setting inappropriately to avoid such a failure case, therefore, measurement results of source cell, target cell and candidate cell(s) at the absolute time value (t1-Threshold + duration), may be stored or reported by the UE in RLF report.

Proposal 1: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) at the absolute time value (t1-Threshold + duration).

For location-based and measurement-based combined trigger condition:

For both earth fixed case and earth moving case, for CHO configured with location-based and measurement-based combined trigger condition, there may be a case that location based CHO execution condition is fulfilled but measurement based CHO execution condition is not fulfilled before RLF happens, since signal strength/quality difference between NTN cell center and cell edge is small, location-based trigger condition has bigger impacts to mobility performance compared with measurement-based trigger condition in CHO NTN, to enable network know that measurement based CHO execution condition is setting strictly, measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled before RLF happens, may be stored or reported by the UE in RLF report.

Proposal 2: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled before RLF happens.

Additionally, for the joint trigger condition, there may be another case that measurement based CHO execution condition is fulfilled but location based CHO execution condition is not fulfilled before RLF happens, to enable network know that location based CHO execution condition is not setting properly, it is beneficial to enable network know the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled before RLF happens, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled before RLF happens, we can see that network can’t derive them because it does not know the time and UE location when only measurement based CHO execution condition is fulfilled. Therefore, the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled before RLF happens, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled before RLF happens, may be stored or reported by the UE in RLF report.

Proposal 3: RLF report can be enhanced to include the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled before RLF happens, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled before RLF happens.

3	Conclusion

In this paper, MRO enhancements for intra-NTN mobility are discussed. We have the following proposals:

Proposal 1: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) at the absolute time value (t1-Threshold + duration).

Proposal 2: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled before RLF happens.

Proposal 3: RLF report can be enhanced to include the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled before RLF happens, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled before RLF happens.