3GPP TSG-RAN WG2 Meeting #129bis	                                 	R2- 2502371

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.5.2

Source: 	Lenovo

Title: 	Issues on the procedure of on-demand SSB SCell operation

Document for:	Discussion and decision

# Introduction

In RAN2#127bis, some agreements were achieved [1].

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

And the issue on L3 RRM framework is FFS including two potential options:

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

In RAN1#120, some agreements were achieved [2].

Agreement

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2.

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2.

  - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency)

    - FFS: Whether this parameter can be absent for Case #2

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst)

    - FFS: Whether this parameter can be absent for Case #2

  - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity)

  - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing)

    - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB.

  - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId)

    - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB.

  - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex)

  - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower)

    - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx)

  - FFS: Which of the above parameters are a list

In RAN1#118b, some agreements were achieved [3].

Agreement

- For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options.

  - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication

  - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - Option 3: Configuration/indication of the duration of on-demand SSB transmission window

  - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell

  - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired

  - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed

  - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI

  - FFS: Each option is applicable to which Cases or Scenarios

  - FFS: Details related to each of the above options

In this contribution, we provide analysis and proposals on procedures and signalling method(s) to support on-demand SSB SCell operation for UE in CONNECTED mode configured with CA, for both intra-/inter-band CA, also we discuss the impact on UE in IDLE/INACTIVE mode.

# Discussion

Configuration of OD-SSB

Referring to RAN1 progress, it was agreed to provide at least Frequency of the on-demand SSB, SSB positions within an on-demand SSB burst by using signalling similar to ssb-PositionsInBurst and Periodicity of the on-demand SSB via higher layer RRC signalling. Also, in RAN1#118, it was agreed to configure multiple candidate values by RRC for on-demand SSB at least including Periodicity of the on-demand SSB and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. Since the existing RRC message can configure cell specific parameters of a UE's serving cell, e.g. when configuring a UE with an SCell or with an additional cell group (SCG), we can reuse the RRC message to configure the on-demand SSB related information.

Proposal 1: Reuse existing RRC message to configure the on-demand SSB related information to UE.

Considering the existing configuration of periodicity for SSB is enumerated which cannot provide multiple values, new parameters have been agreed to be added to provide on-demand SSB configuration including frequency, periodicity, Sub-carrier spacing of the on-demand SSB, etc. for both Case #1 and Case #2. However, how to provide multiple configurations of the on-demand SSB is not clear. On the other hand, the details for what can be indicated by MAC CE for on-demand SSB e.g. other relevant parameters except periodicity of the on-demand SSB, have not been concluded. There could be different signalling structures corresponding to different contents that the MAC CE includes. For example, to configure the OD-SSB in RRC, the new IE with “SEQUENCE” structure for each of the multiple configurations of OD-SSB is used e.g.

SSB-r19 ::=      SEQUENCE {

ssb-Periodicity-List-r19             SEQUENCE (SIZE (1..maxSSB-r19)) OF ssb-Periodicity-r19       OPTIONAL,

ssb-frequency-List-r19	        SEQUENCE (SIZE (1..maxSSB-r19)) OF ARFCN-ValueNR       OPTIONAL,

ssbSubcarrierSpacing-List-r19                SEQUENCE (SIZE (1..maxSSB-r19)) OF SubcarrierSpacing-r19                                                   OPTIONAL,

...

}

Then it can work if a bit (or 2 bits depending on the final RAN1 outcome) will be included in the MAC CE  to indicate which parameters are included corresponding to each field in the MAC CE e.g., if the periodicity, SCS and frequency is being included. A corresponding field in the MAC CE will indicate the index value of the current received configuration for each of the included field(s).

Alternatively, a new RRC structure is used to provide multiple configurations of the on-demand SSB e.g.

SSB-addModList-r19        SEQUENCE {(SIZE (1..maxSSB-r19)) OF SSB-r19

SSB-r19 ::=      SEQUENCE {

ssb-configId                INTEGER (0.. maxConfigIDs),

ssb-Periodicity -r19             ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,

ssb-frequency-r19	ARFCN-ValueNR       OPTIONAL,

ssbSubcarrierSpacing-r19                SubcarrierSpacing-r19                                                   OPTIONAL,

...

}

Then, it can work if an index (ssb-configId) is indicated by MAC CE. We can wait for RAN1 progress to determine the detailed RRC signalling for OD-SSB configuration. Comparing the two alternatives,  it is better to provide an AddModList to configure the OD-SSB in the RRC configuration to simplify the design of MAC CE

Proposal 2: Use an AddModList to configure the OD-SSB parameters in the RRC configuration and include a corresponding index (config Id) in the MAC CE once RAN1 has determined the detailed RRC parameters for OD-SSB configuration.

L3 measurement

Once informed about serving and/ or neighbor cell transmitting SSB on on-demand basis, UE prioritizes measurement of such object: UE may start with the measurement of the said object before any other measurements. Network may use measSequence to achieve this purpose and UE shall prioritize measurement of such an object. Otherwise, UE signals a start time to measure a measurement object (SSB provided on-demand) and the end time for the same can also be indicated accordingly, to enable the network to stop SSB transmission as soon as possible.

Proposal 3: A UE informs the network about the start time and end time of L3 measurement on a frequency with at least one cell providing SSB on-demand.

Considering the dynamical change of OD-SSB transmission e.g. OD-SSB transmission, it would be complicated to configure RRM measurement with SMTC. To simplify the implementation and reduce spec impact, performing RRM measurement based on OD-SSB pattern and RAN4 requirement when OD-SSB is transmitted.

Proposal 4: RRM measurement is performed based on OD-SSB pattern and RAN4 requirement when OD-SSB is transmitted.

# Conclusion

In summary, we have following observations and proposals on the operation of on-demand SSB for SCell:

Proposal 1: Reuse existing RRC message to configure the on-demand SSB related information to UE.

Proposal 2: Use an AddModList to configure the OD-SSB parameters in the RRC configuration and include a corresponding index (config Id) in the MAC CE once RAN1 has determined the detailed RRC parameters for OD-SSB configuration.

Proposal 3: A UE informs the network about the start time and end time of L3 measurement on a frequency with at least one cell providing SSB on-demand.

Proposal 4: RRM measurement is performed based on OD-SSB pattern and RAN4 requirement when OD-SSB is transmitted.

# References

- RAN2#127bis chairman note.

- RAN1#120 chairman note.

- RAN1#118b chairman note.