3GPP TSG-RAN-WG2 Meeting #129bis	R2-2502383

Wuhan, China, 7th – 11th April, 2025

Agenda item	:		8.6.4

Source	: 	Sharp

Title	: 	Discussion on issues for supporting conditional LTM

Document for	:	Discussion and Decision

Introduction

Revised WID [1] for enhancement mobility was approved and RAN2 started discussion on the WI. For conditional LTM, RAN2 starts discussion from this meeting;

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this paper, we discuss some issued related to conditional LTM.

Discussion

TA maintenance

In the last meeting, RAN2 made following agreements:

5. FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

RAN2 need to discuss whether the received CLTM TAs of other candidate cell is released or not. In our understanding, depending on the cell quality or the beam quality, CLTM to another cell may be required again immediately after the CLTM cell switch. In that case, TAT values for candidate cells other than the serving cell might be reused. It should improve robustness of mobility. Therefore, TAT for candidate cells should not be stopped upon CLTM cell switch execution.

Proposal 1. The TAT for candidate cells should not be stopped upon CLTM cell switch execution.

Evaluation phase

Layer Interactions

In the RAN2#128, is was agreed that L1-based evaluation is at MAC level and L3-based evaluation is at RRC level. And in the last meeting, it was agreed that the network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell. This means that the RRC layer evaluates L3 execution conditions and the MAC layer evaluates L1 conditions simultaneously. (e.g. L1 cond is configured for candidate cell A, and L3 cond is configured for candidate cell B, …)

Observation 1. The RRC layer evaluates L3 execution conditions, and the MAC layer evaluates L1 execution conditions simultaneously. (e.g. L1 cond is configured for candidate cell A, and L3 cond is configured for candidate cell B, …)

In the current MAC CR, the MAC entity evaluates L1 conditions and determines that the event for CLTM is satisfied, and indicate to upper layer that the event for CLTM is satisfied. Therefore, RRC layer knows all fulfilled candidate cells and needs to determine the one candidate cell that will be executed.

Observation 2. The RRC layer knows all fulfilled candidate cells.

Observation 3. The RRC layer needs to determine the one candidate cell that will be executed.

In the current RRC CR, there is FFS on how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution. In the Rel-18 CHO procedure, if more than one triggered cell exists, the RRC layer selects one of the triggered cells as the selected cell for conditional reconfiguration execution. This way is simple and should be reused by CLTM.

Proposal 2. If there are more than one LTM candidate cell fulfil the LTM cell switch execution, the RRC layer selects one of the triggered cells as the selected cell for conditional LTM execution.

Conclusion

Proposal 1. TAT for candidate cells should not be stopped upon CLTM cell switch execution.

Observation 1. The RRC layer evaluates L3 execution conditions, and the MAC layer evaluates L1 execution conditions simultaneously. (e.g. L1 cond is configured for candidate cell A, and L3 cond is configured for candidate cell B, …)

Observation 2. The RRC layer know all fulfilled candidate cells.

Observation 3. The RRC layer needs to determine the one candidate cell that will be executed.

Proposal 2. If there are more than one LTM candidate cell fulfil the LTM cell switch execution, the RRC layer selects one of the triggered cells as the selected cell for conditional LTM execution.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4