3GPP TSG-RAN WG2 Meeting #129bis	R2-2502411

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.1.2.2

Title:	Continuous Discussion on LCM for UE side Model

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In RAN2#129, following agreements were reached[1]

Agreements

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

And

- Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities FFS option B Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

Further consideration is provided in this contribution.

Discussion

  - On applicability reporting for option A

One FFS issue after RAN2#129 meeting is whether explicit cause is reported in applicability reporting.

Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause

Although RAN2 agreed to support explicit reporting of applicability/inapplicability in initial reporting and subsequent reporting, there is no consensus on whether we report explicit causes.

First of all, we think explicit cause may only be needed for inapplicability reporting. For applicable functionality, no explicit reason is needed.

Further, following reasons of inapplicability can be considered:

Functionality has not been trained: This cause means there is no corresponding trained model in UE side or at UE’s server, Note that although inference configuration is provided based on the UE reported capability, it is possible the UE side has no trained model due to some kind of reason, such as, lack of collected data, the test of the functionality is not ideal, etc.

Upon receiving this reason, the inapplicable functionality will never become applicable in a while, the NW can remove the inference configuration and there is no need for NW to wait for subsequent applicability reporting.

Functionality has been trained at UE server but UE has not downloaded. This cause means the model has been trained at UE server but not available in UE local storage. The NW may decide to wait the UE to finish model transfer and try again later.

Inference resources temporarily unavailable. This cause means the model has been stored at UE side , but the current hardware resources, including GPU or memory resource, cannot perform inference for the functionality temporarily. The resource may be available later. The NW may decide to wait for subsequent applicability reporting.

In summary, the reported cause may help NW being aware whether the inapplicability is temporary or not. Thus the NW can decide whether to de-configure the functionality, or keep the configuration and wait the functionality becomes applicable.

Observation 1: Reporting explicit inapplicability causes enables the NW to determine whether the inapplicability is temporary or not, allowing the NW to decide whether to remove the functionality configuration or retain it while waiting for the functionality to become applicable.

Based on above consideration, we have the following proposal:

Proposal 1 Explicit reason is supported in inapplicability reporting. RAN2 to consider at least following reasons:

- Functionality has not been trained

- Model need to be downloaded

- Inference resources are temporarily unavailable.

In the agreement that were reached in RAN2#127, NW configures whether UE is allowed to do UAI reporting via OtherConfig in step 3:

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.

In RAN2#129 meeting, following agreements related to the applicability reporting for option A were achieved. According to the agreements, UE sends the initial applicability report in RRCReconfigurationComplete, and UAI can be sent to update applicability.

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

It is not crystal clear for us whether this agreement implies applicability report can be sent via UAI without allowing via otherConfig. Therefore there are following two options of understanding:

Option 1: By default UAI can be sent to update applicability for a functionality if full inference configuration is provided, no matter it is provided in step 3 or step 5.

Option 2: A functionality is allowed to do UAI reporting only when it is allowed to do so via otherConfig.

In our understanding, understanding 1 has advantage in signaling overhead because no additional configuration in otherConfig is needed. While understanding 2 provides more flexibility to NW. The NW can choose whether update of applicability for a functionality is needed. For example, NW may be interested in the initial applicability reporting for a functionality. And then NW may deactivate the functionality according to  performance monitoring results but not rely on applicability reporting.

Proposal 2: RAN2 to confirm whether a functionality needs to be configured in otherConfig to enable applicability reporting via UAI when full inference configuration is configured.

Option 1: A functionality is allowed to do UAI reporting as long as full inference configuration has been provided.

Option 2: A functionality is allowed to do UAI reporting only when it is allowed to do so via otherConfig even if full inference configuration has been provided.

It is necessary for the NW to be able to control when the UE can initiate subsequent applicability reporting via UAI. One option is to reuse prohibit timer mechanism. The proposed prohibit timer is started when UE initiates applicability reporting via UAI or RRCReconfigurationComplete for a functionality that becomes applicable from inapplicable. The UE should not initiate another UAI to report applicability for the same functionality if the the prohibit timer is running.

On the other hand, if the reporting is triggered due to the functionality becomes inapplicable from applicable, the UAI should be transmitted in spit of the prohibit timer. Therefore the NW can deactivate the functionality immediately to avoid further performance loss.

Proposal 3: The UE can be configured with a prohibit timer. This timer starts once the UE initiates applicability reporting for a specific functionality either via the UAI or RRCReconfigurationComplete. Before the prohibit timer expires, the UE will refrain from initiating another UAI to report applicability for the same functionality. Reporting of inapplicability is not restricted by this prohibit timer.

In current running CR, it seems the CSI reporting configuration for AIML inference configuration are located in the same csi-ReportConfigToAddModList.

In our understanding, to enable AIML inference configuration, additional IE will be added to CSI-ReportConfig IE. Such as associated ID, information of set A and set B, and the time instance information for BM-case 2. With such IEs, the UE can distinguish the configured CSI-ReportConfig is for AIML functionality but not for legacy CSI reporting.

However CSI-ReportConfig may be configured for various AIML purposes, such as for inference and for UE side model data collection. Note that RAN1 has agreed that associated ID is placed in CSI-ReportConfig, thus UE side model data collection are most likely reuse CSI-ReportConfig IE.

UE needs to know whether the purpose of the configured CSI-ReportConfig in order to behave accordingly. For example, for inference configuration the UE needs to report applicability and perform reporting based on inference. While for UE side data collection configuration, the UE only needs to perform measurements but not report measurement results.

Observation 2: The CSI-ReportConfig IE can be used for various purposes, including AI/ML inference, UE-side model data collection, and legacy CSI reporting. The UE must distinguish the purpose of the configured CSI-ReportConfig and act accordingly.

There are two options to indicate the purpose:

Option 1: CSI-ReportConfig configured for different purposes are placed in different CSI-ReportConfig ToAddModList.

Option 2: CSI-ReportConfig configured for different purposes are placed in same CSI-ReportConfig ToAddModList and can be distinguished according to unique indicator or unique IEs. For example, an indicator can be included in CSI-ReportConfig IE to indicate it is for inference or for UE side model data collection or both.

We propose RAN2 to discuss this issue and decide which option is be to adopted.

Proposal 4: RAN2 to decide one of the following options to enable the UE to distinguish the purpose of the configured CSI-ReportConfig.

Option 1: The CSI-ReportConfig configured for different purposes are placed in different CSI-ReportConfig ToAddModList.

Option 2: The CSI-ReportConfig configured for different purposes are placed in a same CSI-ReportConfig ToAddModList and can be distinguished according to unique indicator or unique IEs.

  - On functionality activation/deactivation

It was agreed in RAN2#128 that “When a functionality configured by the network to be reported via UAI becomes from inapplicable to applicable, the UE can reports it to the network.”

- When a functionality configured by the network to be reported via UAI, becomes from inapplicable to applicable, the UE can reports it to the network.   FFS detailed design

It was agreed in RAN2#129 that the periodic CSI reporting configuration that configured in step 3 can be activated autonomously by UE in step 4 without need to wait RRCReconfiguration in step 5:

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

Then the question is when periodic CSI reporting configuration is provided as inference configuration, and the functionality becomes applicable from inapplicable, whether UE can autonomously activate the periodic CSI reporting configuration, similar to the autonomous activation of periodic CSI reporting configuration in step 4 ?

There are following two options:

Option 1: UE activates autonomously the functionality after it reports applicability to NW via UAI.

Option 2: UE keeps the functionality as deactivated after UE reports applicability to NW via UAI and waits for NW’s indication to activate it.

With option 2, the UE expects the NW to activate the functionality. Thus the NW may not reserve the resource for the inapplicable CSI reporting configuration. While with option 1, the NW has to reserve resource even when the functionality is inapplicable.

Based on above consideration, we have following proposal:

Proposal 5: When a periodic CSI reporting configured for inference becomes applicable from inapplicable, the UE does not activate the functionality autonomously. NW is expected to activate the functionality after it receives applicability reporting.

For periodic CSI reporting configuration configured for inference, there are cases that the periodic CSI reporting is not applicable:

Case 1: The periodic CSI reporting configuration is provided in step 3 for inference configuration but it is not applicable. The UE will activate the CSI reporting.

Case 2: When a configured periodic CSI reporting configuration becomes inapplicable from applicable.

In both of the above cases, there are two options to handle the inapplicable periodic CSI report configuration:

Option 1: The inapplicable CSI reporting configuration is removed by UE autonomously.

Option 2: The inapplicable CSI reporting configuration is kept in UE side but is deactivated.

Option 1 is obviously not a 3GPP convention.

Proposal 6: The UE does not remove the periodic CSI reporting configuration autonomously after it becomes inapplicable from applicable.

Thus the NW have two choices on how to treat the inapplicable CSI reporting configuration after receiving inapplicable reporting from the UE.

The NW can remove the periodic CSI reporting configuration by transmitting a RRCReconfiguration message. But if the NW wants to receive subsequent applicability reporting, the NW has to configure the functionality in otherConfig. In other words, the specification will support a functionality is configured in otherConfig but it not configured with full inference configuration.

Observation 3: If inapplicable periodic CSI reporting configuration is removed by NW, the NW has to configure the information that is necessary for applicability reporting in otherConfig to enable the UE to report subsequent applicability reporting for the functionality.

Thus we propose RAN2 to consider following proposal:

Proposal 7: RAN2 to decide whether a functionality can be configured in otherConfig with necessary information for applicability reporting but is not configured with full inference configuration.

On the other hand, if NW wants to deactivate the periodic CSI reporting configuration but not to remove it in order to save signaling overhead, the specification needs to support activation/deactivation mechanism for periodic CSI reporting configuration, which is not supported in current specification. Otherwise, the inapplicability reporting configuration can only be removed after it becomes inapplicable and be added again after it becomes applicable.

Observation 4: If activation / deactivation is not supported for periodic CSI reporting configuration, the NW has to remove the periodic CSI reporting configuration when it becomes inapplicable, and reconfigure it after it becomes applicable.

Thus it seems reasonable for RAN2 to consider whether activation / deactivation of periodic CSI reporting configuration is supported.

Proposal 8: RAN2 to decide whether activation / deactivation of periodic CSI reporting configuration is supported.

Conclusion

Observation 1: Reporting explicit inapplicability causes enables the NW to determine whether the inapplicability is temporary or not, allowing the NW to decide whether to remove the functionality configuration or retain it while waiting for the functionality to become applicable.

Observation 2: The CSI-ReportConfig IE can be used for various purposes, including AI/ML inference, UE-side model data collection, and legacy CSI reporting. The UE must distinguish the purpose of the configured CSI-ReportConfig and act accordingly.

Observation 3: If inapplicable periodic CSI reporting configuration is removed by NW, the NW has to configure the information that is necessary for applicability reporting in otherConfig to enable the UE to report subsequent applicability reporting for the functionality.

Observation 4: If activation / deactivation is not supported for periodic CSI reporting configuration, the NW has to remove the periodic CSI reporting configuration when it becomes inapplicable, and reconfigure it after it becomes applicable.

Proposal 1 Explicit reason is supported in inapplicability reporting. RAN2 to consider at least following reasons:

- Functionality has not been trained

- Model need to be downloaded

- Inference resources are temporarily unavailable.

Proposal 2: RAN2 to confirm whether a functionality needs to be configured in otherConfig to enable applicability reporting via UAI when full inference configuration is configured.

Option 1: A functionality is allowed to do UAI reporting as long as full inference configuration has been provided.

Option 2: A functionality is allowed to do UAI reporting only when it is allowed to do so via otherConfig even if full inference configuration has been provided.

Proposal 3: The UE can be configured with a prohibit timer. This timer starts once the UE initiates applicability reporting for a specific functionality either via the UAI or RRCReconfigurationComplete. Before the prohibit timer expires, the UE will refrain from initiating another UAI to report applicability for the same functionality. Reporting of inapplicability is not restricted by this prohibit timer.

Proposal 4: RAN2 to decide one of the following options to enable the UE to distinguish the purpose of the configured CSI-ReportConfig.

Option 1: The CSI-ReportConfig configured for different purposes are placed in different CSI-ReportConfig ToAddModList.

Option 2: The CSI-ReportConfig configured for different purposes are placed in a same CSI-ReportConfig ToAddModList and can be distinguished according to unique indicator or unique IEs.

Proposal 5: When a periodic CSI reporting configured for inference becomes applicable from inapplicable, the UE does not activate the functionality autonomously. NW is expected to activate the functionality after it receives applicability reporting.

Proposal 6: The UE does not remove the periodic CSI reporting configuration autonomously after it becomes inapplicable from applicable.

Proposal 7: RAN2 to decide whether a functionality can be configured in otherConfig with necessary information for applicability reporting but is not configured with full inference configuration.

Proposal 8: RAN2 to decide whether activation / deactivation of periodic CSI reporting configuration is supported.

References

- RAN2#129 chairman notes