3GPP TSG-RAN WG2 Meeting #129bis		R2-2502418

Wuhan, China, 7th – 11th Apr. 2025

Source:	vivo

Title:	Discussion on relay UE (re)selection for intermediate relay UE

Agenda Item:		8.13.2

Document for:	Discussion and Decision

- Introduction

In RAN2 #129 meeting[1], it was agreed that the intermediate relay UE can trigger relay (re)selection, as follows:

Agreements:

The intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the R17 remote UE. FFS upper layer conditions and whether the spec impact will be modelled as a separate relay UE behaviour or through functioning as a remote UE.

Upon intermediate relay UE performing relay (re)selection, it can notify its child node.  If the cause of (re)selection already causes a notification, there is no double-trigger of the notification.

FFS if (re)selection as such is captured as a cause of notification (intention to maintain no double-trigger)

FFS if the notification is required in all cases

FFS the content in the notification

FFS the child node behaviour after receiving this notification

FFS other condition/timing that the intermediate relay UE notifies its child node

This contribution will further discuss the remaining FFS issues on the relay (re)selection at intermediate relay UE.

- Discussion

With all the FFS issues, we would like to attempt to answer the following questions about the intermediate relay UE triggering (re)selection:

- Question 1: Whether the intermediate relay UE should notify the child node upon performing relay (re)selection?

- Question 2: If yes for Q1, when should the intermediate relay UE do the notification?

- Question 3: If yes for Q1, what should be included in the notification message?

- Question 4: What is the child node behaviour after receiving the notification?

We would like to use the Figure in SA2 specification[2] as the example:

Figure 6.4.3.8.3-2: Relay reselection for 5G ProSe Remote UE with 5G ProSe Multi-hop UE-to-Network Relays with Root Relay change

According to SA2 specification[2], ‘During the transmition of re-selection indication to 5G ProSe Remote UE, after receving the re-selection indication a 5G ProSe Intermediate UE-to-Network Relay in the path based on configuration may determine to relay the re-selection indication to 5G ProSe Remote UE, or perform relay reselection by itself as described above’.

Logically, to avoid so-called ‘double trigger’ in intermediate relay UE triggering re(selection), we share the same view in SA2 spec that the intermediate relay UE will either do the reselection itself, or it should notify its child node without performing any reselection behaviour. To minimize the spec impact, we think we can leave it to intermediate relay UE implementation whether to perform relay (re)selection itself or transfer the re-selection indication to child node.

Proposal 1: When Relay (re)selection conditions are fulfilled at intermediate relay UE, it is up to intermediate Relay UE implementation whether to do relay (re)selection itself or not. If not, it will send the notification message to its child node.

However, to move one step further, it should also be discussed that, even the relay (re)selection is performed by intermediate relay UE itself, whether the intermediate relay UE should notify its child node, after a successful/failed reselection procedure.

In our understanding, notification to child node upon successful or failed reselection procedure at intermediate relay UE, can be reasonable sometimes, because:

- Case 1: Upon successful (re)selection procedure at intermediate relay UE, the notification to child node can just serve as an information message, to tell the child node about the change of its parent link. After all, the initial multi-hop path was selected by the remote UE (at least for discovery model B), so it seems fine to notify the remote UE.

- Case 2: Upon failed (re)selection procedure at intermediate relay UE (e.g. not able to find a suitable last relay UE), the notification to child node should be used to further trigger the child node to perform relay (re)selection by the child node itself. The failure of relay (re)selection at intermediate relay UE does not mean the same failure at remote UE.

However, if we would like to distinguish the above two cases, it would be more complicated because we will need different cause value in the notification message, also, for case 1, the intermediate relay UE may further notify the child node about the new last relay UE’s information. On the other hand, even the remote UE does not know the parent link has been changed, it seems not a big deal as the whole multi-hop link still works.

Observation 1: Upon successful relay (re)selection procedure at intermediate relay UE, it is ok for remote UE not being notified, as the link between remote UE and first relay UE is unchanged and the whole multi-hop link still works.

Then, we can simplify the design. On top of proposal 1, we can have another proposal that even the intermediate Relay UE performs the relay (re)selection itself, it can notify its child node (e.g. when not able to find a suitable last relay UE). We prefer to also leave it to intermediate Relay UE implementation. As long as the indication is sent from intermediate Relay UE to child nodes, it means that the link recovery will not be completed by intermediate Relay UE anymore. As also stated in SA2 spec:

- If the 5G ProSe Intermediate UE-to-Network Relay can not complete the reselection, e.g. it is not able to discover the 5G ProSe UE-to-Network Relay, or the 5G ProSe Intermediate UE-to-Network Relay determines to not perform the reselection procedure, it may send re-selection indication and optionally with a cause value to 5G ProSe Remote UE via the established multi-hop relay path.

There may be multiple factors to decide whether the reselection is done at intermediate relay UE or transferred to child node so it is better to leave it to intermediate relay UE implementation.

Proposal 2: When relay (re)selection conditions are fulfilled at intermediate relay UE and intermediate Relay UE performs relay (re)selection itself, it is up to intermediate Relay UE implementation to send the notification message to its child node e.g. later when not able to find a suitable last relay UE.

Then, based on Proposal 1 and Proposal 2, the notification message will always trigger the remote UE to perform relay UE (re)selection, so it can be regarded as a relay (re)selection indication. After receiving this notification, the remote UE triggers relay (re)selection itself.

This is also related to other FFS issues as follows[1]:

Agreements:

For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE.

For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition:

-	PC5 signal strength of current first relay UE is below a (pre)configured signal strength threshold.

-	When U2N Remote UE receives a PC5-S link release message from current first relay UE.

-	When U2N Remote UE detects PC5 RLF with the current first relay UE.

-	Indicated by upper layer.

L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design):

-	cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network.

-	release of RRC connection between an intermediate relay UE and the network.

-	PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s).

-	FFS: link quality degradation of the upstream links resulting in a notification.

-	FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE).

-	FFS on the signalling contents of the PC5-RRC indication from the first relay UE.

-	FFS applicability to L3.

In our understanding, there is no need for the remote UE to know the reason of PC5-RRC signaling from the first relay UE indicating relay (re)selection. The reason now can be that:

- Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network.

- Release of RRC connection between an intermediate relay UE and the network.

- PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s).

- (re)selection is performed by intermediate relay UE but is not successful

Therefore, we can just have a simple notification message which is actually a relay (re)selection indication. The cause value in the message could reuse legacy cause value, e.g. PC5 RLF. As long as the remote UE receives the notification message, the remote UE should perform relay (re)selection.

Proposal 3: Reuse one of the legacy cause values (e.g. PC5 RLF) to be included in the notification message from intermediate relay UE to child nodes.

Proposal 4: Whenever receiving the notification message indicating relay (re)selection from first relay UE, the remote UE triggers the relay (re)selection.

- Conclusion

In this contribution, we discussed the relay (re)selection for intermediate relay UE and tried to give some simple proposals by making the notification message transmission mainly up to intermediate relay UE implementation. We have the following Observations and Proposals:

Observation 1: Upon successful relay (re)selection procedure at intermediate relay UE, it is ok for remote UE not being notified, as the link between remote UE and first relay UE is unchanged and the whole multi-hop link still works.

Proposal 1: When Relay (re)selection conditions are fulfilled at intermediate relay UE, it is up to intermediate Relay UE implementation whether to do relay (re)selection itself or not. If not, it will send the notification message to its child node.

Proposal 2: When relay (re)selection conditions are fulfilled at intermediate relay UE and intermediate Relay UE performs relay (re)selection itself, it is up to intermediate Relay UE implementation to send the notification message to its child node e.g. later when not able to find a suitable last relay UE.

Proposal 3: Reuse one of the legacy cause values (e.g. PC5 RLF) to be included in the notification message from intermediate relay UE to child nodes.

Proposal 4: Whenever receiving the notification message indicating relay (re)selection from first relay UE, the remote UE triggers the relay (re)selection.

- References

- RAN2 #129 meeting chair notes;

- 3GPP TS 23.304 V19.3.0 (2025-03).