# 3GPP TSG-WG2 Meeting #129bis                                         			            R2-2502447

China, Wuhan, April 7th – 11th 2025

Source:                      Qualcomm Incorporated

Title:                          LP-WUS operation in IDLE and Inactive state

Agenda Item:	8.4.2

Document for:	Discussion and Decision

# Introduction & Background

The following objective is approved for IDLE and Inactive mode LP-WUS, this contribution discusses the related issues.

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

# Discussion

## 2.1 LP-WUS configuration

There were some discussions in RAN2#127 meeting that whether dedicated LP-WUS configuration can be provided to the UE. It is understood that the LP-WUS configuration here means entry/exit condition, it is unclear what is the motivation for dedicated entry/exit condition to be introduced. And if the dedicated condition is introduced, the UE needs to maintain both the dedicated configuration and common configuration and RAN2 also needs to discuss in what condition the dedicated configuration is invalid.

- Do not support dedicated LP-WUS monitoring entry/exit condition configuration.

In RAN2#127 meeting, there were proposals and discussion that network can activate or deactivate LP-WUS monitoring using dedicated RRC signalling. If the UE is deactivated LP-WUS monitoring, then the UE will never use LP-WUS to save power consumption as long as the UE stays in IDLE state in this cell. And the motivation is not clear. Then it is proposed not to introduce LP-WUS activation/deactivation in dedicated RRC message.

- Do not introduce LP-WUS activation/deactivation in dedicated RRC message.

Some companies proposed to indicate in SIB whether the UE should skip PEI monitoring when the UE is monitoring LP-WUS. The motivation is to replace PEI with LP-WUS totally, and the network does not need to send indication signalling in both LP-WUS and PEI. But the issue is, when the UE moves outside of LP-WUS coverage, the UE will not indicate to the network as the baseline RAN2 made in the last meeting and the network cannot enable PEI, so the UE cannot use PEI. Another issue is, LP-WUS and PEI can be used together to reduce more false paging. Also, RAN1 has discussed this issue, and the agreement is that it is left to UE implementation to monitor PEI or not.

Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE

Then, it is proposed network sends paging indication in both of LP-WUS and PEI if LP-WUS and PEI are supported by both of UE and gNB, and do not introduce indication in SIB to indicate whether UE should skip PEI monitoring.

- Network sends paging indication in both of LP-WUS and PEI if LP-WUS and PEI are supported by both of UE and gNB.

- Do not introduce indication in SIB to indicate whether UE should skip PEI monitoring.

In RAN2#127 meeting, it has been agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

One issue is, if the entry/exit conditions for both of measurement types are configured, which one of should be used by the UE. In this case, it should allow the UE to select the measurement metrics the UE can support.

- It is up to UE implementation to choose which measurement type is used for entry/exit condition if both of SSB based measurement, OOK based LP-SS measurement types are configured.

From UE perspective, UE may have different capabilities to support SSB based measurement, OOK based LP-SS measurement. If the supported measurement type has not be configured entry/exit condition, it is up to UE implementation to determine the entry/exit condition.

- It is up to UE implementation to determine the entry/exit condition if there is no entry/exit condition configured for the supported measurement type.

## 2.3 Serving cell quality

For the serving cell quality by MR, in order to have aligned UE behavior for LP-WUS and legacy serving cell evaluation, it is proposed to use existing Srxlev/Squal as MR measurement quantities.

- Use existing Srxlev/Squal for MR measurement based entry/exit condition evaluation.

For LR serving cell quality, if MR using Srxlev/Squal for relaxation evaluation using the following formula,

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset )– Pcompensation - Qoffsettemp

Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

Then LR should also use similar way instead of using measured value.

- Introduce Srxlev_lp/Squal_lp for LR measurement based entry/exit condition evaluation. The formula to derive Srxlev/Squal from measured MR value can be  used for deriving Srxlev_lp/Squal_lp from measured LR value.

## 2.2 UE based subgrouping

Currently, for UE based subgrouping in PEI, UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192. Similarly, for LP-WUS, the UE_ID calculation can be reused

- For UE based subgrouping, the UE ID is same as the ID used in PEI, i.e,

UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192.

## 2.3 eDRX support

If the UE operates in eDRX with an eDRX cycle no longer than 1024 radio frames, it monitors POs as DRX. Otherwise, a UE operating in eDRX monitors POs during a periodic Paging Time Window (PTW) configured for the UE. With LP-WUS configuration, if the UE is configured eDRX, then UE calculates PTW as legacy, and within the PTW UE uses LP-WUS to monitor paging. Then there is no enhancement to use LP-WUS with eDRX configuration.

- LP-WUS can be used with eDRX configuration without any enhancements.

## 2.3 Emergency PDU session handling

In last RAN2 meeting, there is open issues on emergency PDU session as follows.

FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In SA2 #167, 17-21 February 2025, meeting there were discussions based on [2] on whether it is necessary to disable paging mechanisms like PEI, eDRX for a UE with an active emergency PDU session. SA2 agreed to remove the emergency PDU session restrictions for Paging Early Indication and LP-WUS from Rel.19 version of the specifications, as PSAP callback is delivered as normal call in IMS PDU session. SA2 prepared CR S2-2502428 [3], for this agreement.

- RAN2 does not specify specific LP-WUS handling when the UE has an emergency PDU session.

# Conclusion

- LP-WUS mode control

- Do not introduce indication in SIB to indicate whether UE should skip PEI monitoring.

- It is up to UE implementation to choose which one is used if both of SSB based measurement, OOK based LP-SS measurement are configured.

- Network sends paging indication in both of LP-WUS and PEI if LP-WUS and PEI are supported by both of UE and gNB.

- Do not introduce indication in SIB to indicate whether UE should skip PEI monitoring.

- It is up to UE implementation to choose which measurement type is used for entry/exit condition if both of SSB based measurement, OOK based LP-SS measurement types are configured.

- It is up to UE implementation to determine the entry/exit condition if there is no entry/exit condition configured for the supported measurement type.

Serving cell quality

- Use existing Srxlev/Squal for MR measurement based entry/exit condition evaluation.

- Introduce Srxlev_lp/Squal_lp for LR measurement based entry/exit condition evaluation. The formula to derive Srxlev/Squal from measured MR value can be  used for deriving Srxlev_lp/Squal_lp from measured LR value.

- LP-WUS subgrouping

- For UE based subgrouping, the UE ID is same as the ID used in PEI, i.e,

UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192

eDRX support

- LP-WUS can be used with eDRX configuration without any enhancements.

Emergency PDU session handling

- RAN2 does not specify specific LP-WUS handling when the UE has an emergency PDU session.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2]    S2-2500133, Emergency call back and “delayed” paging, Qualcomm Incorporated, 3GPP TSG-SA         WG2#166-AdHoc-e

[3]   S2-2502428, 23.501 CR6083 (Rel-19, 'F'): Removing unnecessary restrictions for emergency PDU sessions	Qualcomm	Rel-19