3GPP TSG-RAN WG2 Meeting #129bis	R2-2502452

China, Wuhan, April 7th – 11th 2025

Source:                Qualcomm Incorporated

Title:	SON and MDT for Slicing

Agenda Item:	8.10.2.1

Document for:	Discussion and Decision

# Introduction & Background

In this paper, we discuss aspects related to SON/MDT for network slicing based on agreements RAN3 incoming LS in [1].

RAN3 has discussed how to optimize slice based cell reselection via e.g. logged MDT and identified the following scenario:

- A UE supporting slice-based cell reselection may not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, thereby UE have to camp on cells belonging to the lower ranked NSAG or eventually use legacy cell reselection procedure to find a suitable cell.

RAN3 would like to ask RAN2 to take the above scenario into account, and to discuss whether and how it can be addressed and whether any standard changes are needed.

# Discussion

In order to support slice-based cell reselection, a UE may be provided with slice-based cell reselection information via RRCRelease or SIB16 e.g., reselection priorities per NSAG per frequency and corresponding list(s) of cells where the slices of the NSAG are supported or not supported.

When a UE supports slice-based cell reselection, and when slice-based cell reselection information is provided to the UE, then the UE uses the slice-based cell reselection information.

When no slice-based cell reselection information is provided for any NSAG that was determined to be considered during cell reselection, then the UE uses the general cell reselection information, i.e., without considering the NSAG(s) and their priorities.

A UE provided with slice-based cell reselection information may reselect different cells compared to a UE not provided with slice-based cell reselection information parameters because of different reselection priorities and based on a bias introduced by whether the cell supports a certain slice or not. An operator trying to build coverage maps via logged MDT might not be able to build a true coverage map if it doesn’t know why UEs traversing a similar path see a different coverage (i.e., connect to different cells) even though the radio conditions are similar.

There are benefits in making the OAM aware that the logged MDT reports were collected by a UE using slice-based cell reselection information (and not general cell reselection information).

- UE can indicate via a flag in logged MDT report that it collected logged MDT using the slice-based cell reselection parameters (and not general cell reselection parameters).

# Conclusion

- There are benefits in making the OAM aware that the logged MDT reports were collected by a UE using slice-based cell reselection information (and not general cell reselection information).

- UE can indicate via a flag in logged MDT report that it collected logged MDT using the slice-based cell reselection parameters (and not general cell reselection parameters).

# Reference

[1]   R2-2501734	LS on SON for Network Slicing (R3-250914; contact: ZTE)	RAN3