3GPP TSG-RAN WG2 Meeting #129bis	R2-2502459

Wuhan, China, 7th – 11st April, 2025

Agenda item:	8.17

Source:	Samsung

Title:	On RAN2 aspects of IoT NTN TDD

WID/SID:	IoT_NTN_TDD

Document for:	Discussion and Decision

# Introduction

In RAN#106 the WID for IoT NTN TDD was agreed [1]:

This work item includes the following objectives: - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

In this contribution, we provide some initial discussion on IoT NTN TDD.

# Discussion

- The key aspect of the work item is to adapt NB-IoT NTN to a special TDD band, which can be seen in Figure 1. The UL and DL frames are 8.28 ms, the simplex frame is 20.32 ms and with guard bands the total comes in to 90 ms.

Figure 1.

- The challenging aspects of this specific frame is that the periodicity of the TDD bands has a periodicity that is odd to 3GPP specifications, and that each UL or DL frame being 8.28 ms makes allocating the subframes difficult. Another challenging aspect has to do with the fact that legacy services prevent all of the UL and DL frames to be available for NB-IoT. This means that very few subframes are available, potentially interrupting or delaying procedures. Due to the existing services, it was agreed in the WID that only a single UL and DL frame with 8 subframes each, are available every 90 ms.

## RAN1 agreements

- In RAN1#120, the following was agreed:

Agreement

The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be [48…50] ms at the ULSRP.

- NOTE: This corresponds to the largest separation between DL and UL Iridium slot pairs. Adjustments to align with different UL/DL pair can be achieved by adjusting the common TA.

Agreement

The downlink subframes within D=8 are fixed to:

- Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames).

Agreement

NPSS/NSSS/NPBCH transmissions are dropped in non-D NB-IoT subframes

- NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

- FFS: handling for SIB1-NB.

Agreement

The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” from the specifications

- NPUSCH is mapped as per current specifications.

- NOTE: “non-U NB-IoT subframes” are subframes not contained within the set of U=8 usable consecutive uplink subframes in the TDD structure.

Agreement

The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications

- NPDCCH / NPDSCH (not containing SI) are mapped as per current specifications.

- FFS: additional NB-IoT TDD-specific rules for handling non “NB-IoT DL subframes”

- FFS: Introduction of new periodicities for NPDCCH monitoring (e.g. by the introduction of scaling factor)

- FFS: whether all the configuration values for NPDCCH are valid for NB-IoT NTN TDD.

- FFS: handling of DL transmission gap

- NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

Agreement

For NPRACH, further consider the two options until RAN1#120bis:

- Option 1: New NPRACH periodicities (multiple of 90ms) are introduced for NB-IoT NTN TDD.

- Option 2: NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s)

  - FFS: The unit of postponement (e.g. PRU)

Agreement

NPRACH format 2 is not supported in NB-IoT NTN TDD.

Working assumption

Uplink transmission gaps do not apply in NB-IoT NTN TDD.

Agreement

For segmented pre-compensation, RAN1 to further discuss at least the following options:

- Option 1: Dropped/postponed NPUSCH / NPRACH are counted for segment determination

- Option 2: A new pre-compensation segment is started at the beginning of each uplink burst

- Option 3: UL segment should only be defined in consecutive U subframes

- Option 4: Segmented pre-compensation does not apply for NB-IoT NTN TDD.

- FFS: How to handle the UL segment gap.

- With the current FFSes or options in RAN1, we propose that does not take time to discuss the following options:

Proposal 1: RAN2 avoids discussing the FFSes or options from RAN1:

- Periodicities for NPDCCH monitoring

- DL transmission gaps

- NPRACH periodicities or postponement

- Segmented pre-compensation

## Timers

- In the last meeting, the following agreement was taken:

5. RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system.

- The contribution for which the proposal originated from discussed whether the current timer ranges are sufficient as IoT NTN TDD pattern only maximum allows for 8 ms DL/UL every 80 ms. However, while in theory the transmission durations may be increased due to the low amount of resources, we do not think that it would be a reasonable system design to revisit the timer values because it may require a lot of work in RAN2 and CT1, with likely next no to reasonable practical reasons for it. If the repetitions are only transmitted during the 8 ms out of 90 ms duty cycle, it may in theory be possible to assume that transmission times are increased by a factor 90/8 on top of the maximum 2048 repetitions, but there are no reasonable deployments where such long transmission times would be used. In other words, the deployment would have to ensure that the number of repetitions would be dimensioned to take into account the reduced resource in an IoT NTN TDD network. Therefore we propose:

Proposal 2: RAN2 agrees current value range of NB-IoT timers are sufficient for a practical deployment.

## Cell barring

- Another agreement that was taken in the last meeting is the following:

7. Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

- This agreement came due to discussions whether a new type of barring indication would be required for IoT NTN TDD. However, it was determined that the IoT NTN TDD is band-specific. This would mean that any UE that supports the IoT NTN TDD band shall support the IoT NTN TDD operation as will be specified in Release 19, which shall include all of the mandatory adaptations to enable IoT NTN TDD operations. Whether there will be optional feature in IoT NTN TDD operation is yet to be seen. A text proposal can be seen in Appendix 5.1.

Proposal 3: A UE that supports the IoT NTN TDD shall be considered to support the IoT NTN TDD operation, which includes all of the mandatory features and adaptations to support IoT NTN TDD operation.

Proposal 4: Take the text proposal of how this can be specified in 36.306 in Appendix 5.1 as a baseline.

Proposal 5: Rel-17 NTN barring procedures are used for IoT NTN TDD, unless there are any other optional features which require barring in IoT NTN TDD.

## On extended k-Mac

In the last meeting it was proposed in multiple contributions to introduce an extension of k-Mac. Our understanding is that intention of this would be to support transparent multi-hop cases where the k-Mac occasionally is longer than the already existing maximum value of 512 subframes. While multi-hop has been discussed in RAN3 during the course of the work in NTN, our understanding is that the extension of k-Mac for multi-hop scenarios should not cause any RAN3 work.

Proposal 6: Introduce extended k-Mac for Rel-19 NB-IoT NTN, without impact to RAN3. New maximum value is FFS.

The extension can be introduced by introducing a new value and not configuring the legacy value k-MAC. A text proposal can be seen in Appendix 5.2.

Proposal 7: The extended k-Mac is introduced by introducing a new value, where the legacy k-Mac is not configured.

Proposal 8: Take the RRC implementation example in Appendix 5.2 as a baseline.

# Conclusion

In this contribution, we provided some proposals on IoT NTN TDD:

Observation 1: One challenging aspect has to do with the fact that very few subframes are available regularly, potentially interrupting or delaying procedures.

Proposal 1: RAN2 avoids discussing the FFSes or options from RAN1:

- Periodicities for NPDCCH monitoring

- DL transmission gaps

- NPRACH periodicities or postponement

- Segmented pre-compensation

Proposal 2: RAN2 agrees current value range of NB-IoT timers are sufficient for a practical deployment.

Proposal 3: A UE that supports the IoT NTN TDD shall be considered to support the IoT NTN TDD operation, which includes all of the mandatory features and adaptations to support IoT NTN TDD operation.

Proposal 4: Take the text proposal of how this can be specified in 36.306 in Appendix 5.1 as a baseline.

Proposal 5: Rel-17 NTN barring procedures are used for IoT NTN TDD, unless there are any other optional features which require barring in IoT NTN TDD.

Proposal 6: Introduce extended k-Mac for Rel-19 NB-IoT NTN, without impact to RAN3. New maximum value is FFS.

Proposal 7: The extended k-Mac is introduced by introducing a new value, where the legacy k-Mac is not configured.

Proposal 8: Take the RRC implementation example in Appendix 5.2 as a baseline.

# Reference

RP-243293, Revised WID for introduction of IoT NTN TDD mode, Iridium Satellite LLC, RAN#106, Madrid, Spain, December 2024.

# Appendix

## 36.306 TP

------------------------ Example based on 36.306 V18.4.0 ------------------------

# 7	Conditionally Mandatory features

. . .

## 7.XX    IoT NTN

### 7.XX.1    IoT NTN TDD

It is mandatory to support IoT NTN TDD operation and adaptation for UEs that indicate support of IoT NTN TDD band, see TS 36.300 [30] and TS 36.102 [43]. A UE supporting IoT NTN TDD shall indicate support of ntn-Connectivity-EPC-r17, and the UE shall in addition to the features in ntn-Connectivity-EPC-r17 also support the following enhancements:

- General:

  - FFS

- Higher layers:

  - FFS

- Physical layer:

  - FFS

------------------------ Example based on 36.306 V18.4.0 ------------------------

## 36.331 TP

------------------------ Example based on 36.331 V18.5.0 ------------------------

–			SystemInformationBlockType31

The IE SystemInformationBlockType31 contains satellite assistance information for the serving cell. SystemInformationBlockType31 is only signalled for an NTN cell.

SystemInformationBlockType31 information element

-- ASN1START

SystemInformationBlockType31-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...,

[[	servingSatelliteInfo-v1820	ServingSatelliteInfo-v1820		OPTIONAL	-- Need OR

]]

}

ServingSatelliteInfo-r17 ::=	SEQUENCE {

ephemerisInfo-r17			CHOICE {

stateVectors				EphemerisStateVectors-r17,

orbitalParameters			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r17			SEQUENCE {

nta-Common-r17					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r17				INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

ul-SyncValidityDuration-r17		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900},

epochTime-r17					SEQUENCE {

startSFN-r17					INTEGER (0..1023),

startSubFrame-r17				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Offset-r17					INTEGER (0..1023),

k-Mac-r17						INTEGER (1..512)				OPTIONAL,	-- Need OP

...

}

ServingSatelliteInfo-v1820 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18					OPTIONAL,	-- Need OR

referenceLocation-r18			CHOICE {

fixedReferenceLocation-r18			ReferenceLocation-r18,

movingReferenceLocation-r18			ReferenceLocation-r18

}															OPTIONAL,	-- Need OR

distanceThresh-r18				INTEGER(0..65535)			OPTIONAL	-- Need OR

}

-- ASN1STOP

SystemInformationBlockType31 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE (as specified in TS 36.304 [4]) and RRC_CONNECTED. Each step represents 50m.

epochTime Epoch time of the satellite ephemeris data and common TA parameters, see TS 36.213 [23]. This field also indicates the epoch time for the reference location of earth moving cells if present. The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point of the serving cell. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. For serving cell, the startSFN indicates the current SFN or the next upcoming SFN after the frame where the message indicating the epochTime is received. If the field is absent, the epoch time is the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB31(-NB) is transmitted. E-UTRAN always includes epochTime when SIB31(-NB) is provided through dedicated signalling. In case of handover or conditional handover, this field is based on the timing of the target cell, i.e. the startSFN and startSubFrame number indicated in this field refers to the SFN and sub-frame of the target cell, and UE considers the target cell epoch time (indicated by the startSFN and startSubFrame in this field) to be the frame nearest to the frame where RRCConnectionReconfiguration message is received. In case of handover or conditional handover, the reference point for epoch time of the target NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point of the target cell.

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in ms. If the field if absent, the UE uses the value in extK-Mac in SIB31-NB if configured, else the UE uses the (default) value of 0.

k-Offset Scheduling offset used in the timing relationships in NTN, see TS 36.213 [23]. Unit in ms.

nta-Common Network-controlled common TA, see TS 36.213 [23]. Unit of μs. Step of 32.55208 ×10-3 μs. Actual value = field value * 32.55208 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDrift Drift rate of the common TA, see TS 36.213 [23]. Unit of μs/s. Step of 0.2 ×10-3 μs/s. Actual value = field value * 0.2 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDriftVariation Drift rate variation of the common TA, see TS 36.213 [23]. Unit of μs/s2. Step of 0.2 ×10-4 μs/s2. Actual value = field value * 0.2 ×10-4. If the field is absent, the UE uses the (default) value of 0.

orbitalParameters Instantaneous values of the satellite orbital parameters. The signalled values are valid at least for the duration as defined by ul-SyncValidityDuration and epochTime.

referenceLocation Reference location of the NTN (quasi-)earth fixed cell or earth moving cell, used in location-based measurement initiation in RRC_IDLE (as specified in TS 36.304 [4]) and RRC_CONNECTED if distanceThresh is also configured. If configured by an earth moving cell, the broadcast reference location corresponds to the epoch time and is also used in the evaluation of Event D2 and CondEvent D2, and the UE derives the real-time reference location based on the serving satellite ephemeris, see TS 36.304 [4].

stateVectors Instantaneous values of the satellite state vectors. The signalled values are valid at least for the duration as defined by ul-SyncValidityDuration and epochTime.

ul-SyncValidityDuration Validity duration of the satellite ephemeris data and common TA parameters, i.e. maximum time duration (from epochTime) during which the UE can apply the satellite ephemeris without acquiring new satellite ephemeris, see TS 36.213 [23]. Unit in second. Value s5 corresponds to 5 seconds, value s10 corresponds to 10 seconds and so on. The ul-SyncValidityDuration is only updated when at least one of epochTime, nta-CommonParameters, ephemerisInfo is updated.

<OMITTED>

–			SystemInformationBlockType33

The IE SystemInformationBlockType33 contains satellite assistance information for neighbour cells.

SystemInformationBlockType33 information element

-- ASN1START

SystemInformationBlockType33-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18	NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING							OPTIONAL,

...

}

NeighSatelliteInfoList-r18 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfo-r18 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18,

ephemerisInfo-r18				CHOICE {

stateVectors-r18				EphemerisStateVectors-r17,

orbitalParameters-r18			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r18		SEQUENCE {

nta-Common-r18					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18			INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

epochTime-r18					SEQUENCE {

startSFN-r18					INTEGER (0..1023),

startSubFrame-r18				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r18						INTEGER (1..512)				OPTIONAL,	-- Need OP

t-ServiceStartNeigh-r18		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

-- ASN1STOP

SystemInformationBlockType33 field descriptions

epochTime Epoch time of the neighbour satellite ephemeris data and common TA parameters, see TS 36.213 [23]. The reference point for epoch time of the neighbour satellite ephemeris and Common TA parameters is the uplink time synchronization reference point of the serving cell when this field is provided in an NTN cell and the eNB when this field is provided in a TN cell. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. If this field is absent in an NTN cell, the UE uses epoch time of the serving cell, otherwise the field is based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the serving cell. The startSFN indicates the SFN nearest to the frame where the message indicating the epochTime is received. If this field is absent in a TN cell, the epoch time is the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB33(-NB) is transmitted.

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in ms. If the field if absent, the UE uses the value in extK-Mac for the associated satellite info element if configured, else the UE uses (default) value of 0.

neighValidityDuration Validity duration of the neighbour satellite ephemeris data and common TA parameters, i.e. maximum time duration (from epochTime) during which the UE can apply the satellite ephemeris without acquiring new satellite ephemeris, see TS 36.213 [23]. Unit in second. Value s5 corresponds to 5 seconds, value s10 corresponds to 10 seconds and so on. If this field is absent in an NTN cell, the UE uses validity duration from the serving cell assistance information. If this field is absent in a TN cell, how the UE sets validity duration is left to UE implementation.

nta-Common Network-controlled common TA, see TS 36.213 [23]. Unit of μs. Step of 32.55208 ×10-3 μs. Actual value = field value * 32.55208 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDrift Drift rate of the common TA, see TS 36.213 [23]. Unit of μs/s. Step of 0.2 ×10-3 μs/s. Actual value = field value * 0.2 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDriftVariation Drift rate variation of the common TA, see TS 36.213 [23]. Unit of μs/s2. Step of 0.2 ×10-4 μs/s2. Actual value = field value * 0.2 ×10-4. If the field is absent, the UE uses the (default) value of 0.

t-ServiceStartNeigh Indicates the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell(s) served by the satellite indicated by satelliteId, see 5.5.3.1, 5.5.8 and 36.304 [4]. This field is only present for the NTN quasi-Earth fixed neighbour cell(s).

<OMITTED>

–				SystemInformationBlockType31-NB

The IE SystemInformationBlockType31-NB contains satellite assistance information. SystemInformationBlockType31-NB is only signalled in a NTN cell.

SystemInformationBlockType31-NB information element

-- ASN1START

SystemInformationBlockType31-NB-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...,

[[	servingSatelliteInfo-v1820	ServingSatelliteInfo-v1820		OPTIONAL	-- Need OR

]],

[[	servingSatelliteInfo-v19xy	ServingSatelliteInfo-NB-v19xy	OPTIONAL	-- Need OR

]]

}

ServingSatelliteInfo-NB-v19xy ::=	SEQUENCE {

extK-Mac-r19					INTEGER (513..FFS)				OPTIONAL	-- Need OP

}

-- ASN1STOP

<OMITTED>

–	SystemInformationBlockType33-NB

The IE SystemInformationBlockType33-NB contains satellite assistance information for neighbour cells.

SystemInformationBlockType33-NB information element

-- ASN1START

SystemInformationBlockType33-NB-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18		NeighSatelliteInfoList-r18		OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...,

[[  neighSatelliteInfoList-v19xy	NeighSatelliteInfoList-NB-v19xy OPTIONAL    -- Need OP

]]

}

NeighSatelliteInfoList-NB-v19xy ::=		SEQUENCE {

extK-Mac-r19							INTEGER {513..FFS}		OPTIONAL	-- Need OP

}

-- ASN1STOP

------------------------ Example based on 36.331 V18.5.0 ------------------------