3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502479
Wuhan, China, 7 – 11 April, 2025

Agenda Item:	8.7.5

Source:	InterDigital

Title:	Discussion on RLC enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion

# 1. Introduction

In RAN2 #129 meeting, we made several agreements for RLC enhancement [1], which are provided in the Appendix. In this contribution we further discuss the RLC enhancement topics. Specifically, we further discuss enhancement to avoid unnecessary retransmission and timely RLC retransmission.

# Discussion

  - 2.1 Timely RLC retransmissions

In the last meeting, RAN2 agreed that autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specific threshold. One remaining issue is whether to determine the remaining time based on the discardTimer at PDCP or a new timer at RLC. In our view, the remaining time based on the discardTimer at PDCP dictates accurately the remaining time of the RLC SDU. It was mentioned that reusing the discardTimer at PDCP requires interaction between PDCP and RLC. However, interaction between layers is already feasible such as the interaction between PDCP and MAC for DSR reporting and between PDCP and RLC for PDCP discard indication. Therefore, it is feasible to use the discardTimer at PDCP for determining the remaining time of the RLC SDU.

Proposal 1:	The remaining time for autonomous retransmission and/or polling is determined based on discardTimer at PDCP associated with the RLC SDU.

Autonomous retransmission:

It is possible that the RLC SDU or a RLC SDU segment has not been transmitted when the remaining time of the RLC SDU falls below the configured threshold. In this case, it is not meaningful to trigger an autonomous re-transmission since that would not improve the success in transmitting the original RLC SDU; in fact, both the original and the autonomously re-transmitter RLC SDU (segment) could end up being multiplexed into a same TB at MAC. Therefore, it is necessary to fully transmit the original RLC SDU before triggering autonomous retransmission.

Proposal 2: 	Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been fully transmitted by lower layers.

It is possible that retransmission of a RLC SDU (segment) is already considered due to reception of NACK status report before the remaining time of the RLC SDU falls below a threshold. In this case, autonomous transmission should not be triggered as such autonomous retransmission may result in wasting of transmission resource.

Proposal 3: 	Autonomous retransmission is not triggered if the RLC SDU (segment) is already considered for retransmission (e.g., based on receiving negative acknowledgement).

According to legacy [2], retransmission of an RLC SDU (segment) due to NACK status report is always prioritized over initial transmission of another RLC SDU (segment). Another issue is how to prioritize between autonomous retransmission of an RLC SDU and retransmission of another RLC SDU (segment) due to NACK status report. It is possible that the RLC SDU (segment) considered for autonomous retransmission has been successfully received from the receiver RLC entity. On the other hand, the RLC SDU (segment) subject for retransmission due to NACK report from the receiver absolutely requires retransmission to be successfully received. Therefore, this RLC SDU (segment) should be prioritized over any other autonomous retransmission RLC SDU (segment).

Proposal 4:	Status Report based retransmission of an RLC SDU (segment) is prioritized over autonomous retransmission of any other RLC SDU (segment).

Polling enhancement:

As RAN2 agreed in the last meeting that polling can be triggered when the remaining time of a RLC SDU (segment) falls below a threshold. Like autonomous retransmission case, the remaining time of a RLC SDU (segment) may fall below a threshold before the original RLC SDU is completely transmitted. In this case, enabling polling may not be beneficial as no status for such RLC SDU is expected in the status report. It is necessary to enable polling bit when the RLC SDU is completely transmitted. In another words, polling bit can be enabled at earliest when the RLC SDU or the last segment of the RLC SDU is being transmitted.

Proposal 5: 	When the remaining time of the RLC SDU falls below a specified threshold, polling is triggered for an RLC SDU at the earliest when the last segment of the RLC SDU is being transmitted.

  - 2.2 Avoiding unnecessary retransmissions

In the last meeting, it was agreed that a new RLC timer at the Rx is introduced to determine the obsolete RLC SDUs at the Tx RLC, and the timer starts when the gap is detected at RLC layer. Upon the expiry of the newly introduced timer, an RLC status reporting should be triggered. It is also agreed that the discarded RLC SDUs are reported as ACK in the RLC status report. This is to minimize useless re-transmissions at the Tx side when the Rx has already moved its receiving window such that those RLC SDUs would get discarded.

Proposal 6:	At the Rx RLC, an RLC status report is triggered upon expiry of the timer.

# Conclusion

In this contribution, the following proposals are made for timely RLC retransmission and avoiding unnecessary RLC retransmission of RLC enhancement:

Proposal 1:	The remaining time for autonomous retransmission and/or polling is determined based on discardTimer at PDCP associated with the RLC SDU.

Proposal 2: 	Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been fully transmitted by lower layers.

Proposal 3: 	Autonomous retransmission is not triggered if the RLC SDU (segment) is already considered for retransmission (e.g., based on receiving negative acknowledgement).

Proposal 4:	Status Report based retransmission of an RLC SDU (segment) is prioritized over autonomous retransmission of any other RLC SDU (segment).

Proposal 5: 	When the remaining time of the RLC SDU falls below a specified threshold, polling is triggered for an RLC SDU at the earliest when the last segment of the RLC SDU is being transmitted.

Proposal 6:	At the Rx RLC, an RLC status report is triggered upon expiry of the timer.

# References

- RAN2#129 chair notes.

- 3GPP TS 38.322, Radio Link Control (RLC) protocol specification, v.18.0.0, 2023-12.

# Appendix: RAN2 #129 on RLC enhancement for XR

Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review