3GPP TSG-RAN WG2 Meeting # 129bis 	                R2-2502498

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.18

Source: 	Sony

Title:	Removing Initial DL BWP restriction for MT-SDT

Document for:	Discussion and Decision

# Introduction

In the last meeting, RAN2 discussed how to addresses the BWP restriction in MT-SDT (i.e., increase the resources for MT-SDT), with the following minutes:

R2-2410542	Quality Indication in Msg3 for SDT	Ericsson	discussion	Rel-19	TEI19

Observation 1	MT-SDT operation over initial BWP restricts DL MT-SDT transmissions to the bandwidth of CORESET 0 in RRC_INACTIVE state.

Observation 2	MT-SDT BWP restriction with no DL quality feedback results in smaller, more conservative TB allocations in the downlink at low MCS values. For larger data volumes, smaller TBS translate to a higher number of subsequent transmissions.

Proposal 1	RAN2 addresses the BWP restriction in MT-SDT by introducing a Channel Quality Indication (CQI) in Msg3 or MsgA.

Option 1: Remove the CORESET 0 restriction for DL BWP for SDT from/after first DL MT-SDT data

Option 2: Introduce Channel Quality Indication (CQI) in Message 3 or MsgA

-	ZTE has sympathy on the problem statement but not sure about the solution, we should just remove the restriction.   The CQI solution is not addressing the problem identified.

-	Qualcomm thinks that removing the BWP restriction is not a good choice.   If we don’t reports CQI in msg3 it will impact RAN4 as well.

-	LG thinks that the BWP restriction is the root cause and we should remove it.   Nokia agrees.

-	Sony explains that a similar issues was identified in MBS and we agreed to expand coreset 0, introduce CFR.  We can reuse that solution.

RAN2 will address this issue.  FFS the exact solution.

Noted

[AT129][028][TEI19 SDT]  (Ericsson)

Intended outcome: downselect between early CQI reporting and BWP restriction removal

Deadline:  Thursday.

Postpone the discussion to next meeting.

In this contribution, we discuss at least three options for addressing the BWP restriction in MT-SDT and provide a proposal at the end.

## Removing Initial BWP restriction in MT-SDT

In the current Rel-18 specification, as discussed in [1], there is restriction of resources used for DL MT-SDT in RRC_INACTIVE state, which is limited to the size of CORESET0 whilst the UL MO-SDT employs the entire initial UL BWP (configured initial BWP). The following three options for addressing the BWP restriction in MT-SDT are discussed below.

Option 1: Introduce Channel Quality Indication (CQI) in Message 3/MsgA

As proposed and discussed in the last meeting [1], for DL MT-SDT, regardless of whether the channel condition is good or not, the scheduler usually uses conservative MCS for scheduling MT-SDT due to lack of CQI feedback. Hence proposal in [1] is to specify an indicator of the channel quality from the UE. This will enable to employ a better modulation and coding rates when the channel is good, and as a result, a large TB sizes can be scheduled to the UEs. Whilst this proposal improves a bit from a single UE perspective, but it does not address the root cause of the BWP restriction. There is a need to schedule a reasonable number of UEs at the same time in order to increase the system capacity, as well as to reduce the power consumption at the UE (not very large BWP as the case of configured DL BWP).

Observation 1. Introducing Channel Quality Indication (CQI) in Message 3/MsgA does not address the root cause of the BWP restriction.

Option 2: Separate initial downlink BWP for MT-SDT

Another option is to configure an initial downlink BWP for MT-SDT where a UE switches to another initial DL BWP configured for MT-SDT. By doing so, the network can schedule MT-SDT data to a separate initial BWP. However, in order to minimize the impacts of receiving other services for IDLE/INACTIVE UEs, like paging and MBS, it is preferable that this separate initial BWP should overlap CORESET0.

Option 3: Reusing MBS CFR

In MBS for Rel-17/18, a similar issue was discussed [2], and a solution was specified for MBS UEs in IDLE/INACTIVE where a Common Frequency Resource (CFR) larger than CORESET0 (i.e., initial DL BWP) can be configured by SIB1 from the network. The MBS CFR includes CORESET0, hence, a UE does not need BWP switching as shown on Figure 1, and larger resources can be scheduled to the UEs.

Figure 1. MBS CFR larger than initial DL BWP for IDLE/INACTIVE UEs.

The most important thing is to remove the BWP restriction. We are open to discuss the last two options (option 2 and 3) provided that this separate initial BWP should overlap CORESET0, in order to minimize the impacts of receiving other services for IDLE/INACTIVE UEs, like paging and MBS.

Observation 2: Separate initial BWP should overlap CORESET0, in order to minimize the impacts of receiving other services for IDLE/INACTIVE UEs, like paging and MBS.

Proposal 1: Remove the BWP restriction by introducing CFR or separate initial DL MT-SDT that overlaps CORESET0.

## Conclusions

In this contribution, we have discussed a few options for addressing the BWP restriction in MT-SDT. We have the following observations and proposal.

Observation 1. Introducing Channel Quality Indication (CQI) in Message 3/MsgA does not address the root cause of the BWP restriction.

Observation 2: Separate initial BWP should overlap CORESET0, in order to minimize the impacts of receiving other services for IDLE/INACTIVE UEs, like paging and MBS.

Proposal 1: Remove the BWP restriction by introducing CFR or separate initial DL MT-SDT that overlaps CORESET0.

## References

[1] R2-2410542	, “Quality Indication in Msg3 for SDT,”	Ericsson, RAN WG2.

[2] R1-2102281, “Final Report of 3GPP TSG RAN WG1 #104-e v1.0.0,” MCC, RAN WG1.