3GPP TSG-RAN WG2 Meeting #129bis		R2-2502499

Wuhan, China, 7-11th April 2025

Agenda Item: 	8.2.2

Source:		Sony

Title:	Multiple Paging involving reader(s) and service(s)

Document for:	Discussion & Decision

# Introduction

Email discussion [POST129][035][AIoT] Paging made following proposals:

Proposal 1: Discuss and agree on the device behavior if it gets a new service request while one procedure is still ongoing (i.e. not completed or failed yet): (a) ignore all new requests, or (b) terminate the ongoing procedure and respond to the latest request. (a/b/ffs = 17/7/5) Proposal 2: Discuss and decide how a device determines the procedure is ‘ongoing’ or not. Proposal 3: Discuss and decide whether the device needs to distinguish and behave differently between when the same service request is received again from the same reader vs a different reader (yes/no/ffs = 14/12/2). Proposal 4: Discuss and decide whether RAN2 needs to consider the case where the device has already successfully responded to the same service request but now received the same request from a different reader e.g. for location use case (yes/no/unclear = 9/16/3). Proposal 5: Discuss and decide whether Reader ID information should be provided to the device (yes/no/FFS = 11/17/1). Proposal 6: Wait for SA2/RAN3 progress before further discussing on how the reader calculates transaction ID. Proposal 7: Discuss whether to send an LS: To SA2 asking whether correlation ID is expected to be same or different when the same service is requested from different readers, and to RAN3 asking whether coordination between readers is expected. Proposal 8: Postpone discussion on exact size of transaction ID until further progress on other issues captured in previous proposals. Proposal 9: Discuss whether and how the reader knows the association between the paging identifier (presumably used by the CN in the request to the reader) and the AS-ID (presumably used over the air for command delivery to device in inventory+command use case.)

In this contribution we provide our views on:

- New service request while old one is “ongoing”. Define ongoing.

- New service request from same or different reader?

- New service request when already responded but now from a different reader. Need to identify reader.

# Discussions

Paging from same reader and definition of ongoing:

One of the open issues during the email discussion is that if a device should respond to a new service request while old one is ongoing. Then define what is ongoing? We explain the paging procedure by using an example of Access Round and Engaging period. As shown on Figure 1, there are two access rounds: an access round 1 and access round 2 where each access round comprises a number of engaging periods for different devices. The engaging period is where the device gets its opportunity for random access, DL receptions and UL transmissions. For example, Device1 occupies on Engaging period 1, and device 2 will occupy on Engaging period 2, both with the same Reader 1.

Figure 1: paging procedure by using an example of Access Round and Engaging period.

After Device 1 has completed its Engaging period 1 or during the engaging period or during Access round 1, if it has again received another paging message before access round period 1 is completed, then Device 1 will not respond to this paging message. We wait for RAN1 to define the periods like Access round and engaging period.

Proposal 1: Device responds to a Paging message within an Access Round and ignore all other Paging requests during this period. A device may be allocated different engaging period within an Access Round. Wait for RAN1 to define the periods like Access round and engaging period.

Proposal 1 should apply to the case even if Paging procedure has failed. We think this will allow the network to balance the load.

If a service request is received outside of device’s access round, then it should respond to the paging treating it as a new service request.

Proposal 2: If a service request is received outside of device’s access round then it should respond to paging treating it as a new service request.

Paging from different readers:

It has been agreed that RRC connection management and periodical system information (SIB and MIB) are not supported by A-IoT devices. This may mean that a device is not connected to a specific cell. Figure 2 shows a device in the presence of multiple readers. If Core Network (CN) initiates the paging message via gNB/Reader for the Device or a group of Devices, it is likely that paging messages may be sent to multiple close-by readers as CN does not know the exact location of a specific device especially where e.g. device ID is not included.

Figure 2. A-IoT device locating in between Readers.

There are three options:

Option 1: The device does not respond to Paging messages from different readers within an access round period i.e., device ignores Paging from same or different readers within the current access round period.

Option 2: The device responds to Paging messages from different readers after multiple access round periods have been completed with the current Reader (i.e., Device is locked to the current Reader). In this case, some form of reader ID should be included in Paging message to identify the current reader.

Option 3: The device receives a configuration from the current Reader/Network whether the device is allowed to respond other paging messages from different readers within one or more access round periods.

Proposal 3: RAN2 to discuss and select one or more options for scenarios involving paging from different readers.

# Conclusion

In this contribution, we propose RAN2 discuss and agree following proposals:

Proposal 1: Device responds to a Paging message within an Access Round and ignore all other Paging requests during this period. A device may be allocated different engaging period within an Access Round. Wait for RAN1 to define the periods like Access round and engaging period.

Proposal 2: If a service request is received outside of device’s access round then it should respond to paging treating it as a new service request.

Proposal 3: RAN2 to discuss and select one or more options for scenarios involving paging from different readers.

Option 1: The device does not respond to Paging messages from different readers within an access round period i.e., device ignores Paging from same or different readers within the current access round period.

Option 2: The device responds to Paging messages from different readers after multiple access round periods have been completed with the current Reader (i.e., Device is locked to the current Reader). In this case, some form of reader ID should be included in Paging message to identify the current reader.

Option 3: The device receives a configuration from the current Reader/Network whether the device is allowed to respond other paging messages from different readers within one or more access round periods.

# References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

- TR 38.848, Study on Ambient IoT (Internet of Things) in RAN (Release 18)