3GPP TSG-RAN WG2#129bis											   R2-2502502

Wuhan, China, April. 14th – 18th, 2025

Agenda Item:	8.8.2

Source: 	NEC

Title:	        Downlink coverage enhancement

Document for:	 Discussion

# Introduction

Downlink coverage enhancement consists of two aspects:

- system level coverage enhancement with beam hopping, focus on SSB periodicity extension up to 160ms (i.e., 320ms and longer periodicity are excluded).

- link level coverage enhancement with Msg4 PDSCH repetition, being mainly discussed in RAN1.

This document will address RAN2 remaining issues on SSB extension and msg4 PDSCH repetition.

# Discussion

## Possible SMTC impact

RAN2 has extensively discussed whether a maximum of 4 SMTCs configuration is insufficient when beam hopping is applied. Agreement made in last meeting are as following:

1. RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer 2. RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame 3. RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) 4. The number of SMTC/gaps a UE needs to consider at any time will not be increased further 5. From RAN2 perspective, we support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC. 6. RAN2 observes that if the cells active simultaneously are in clusters, existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may be sufficient (FFS if any solution identified for option b applies to option a as well). 7. RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

After last RAN2 meeting discussion, the main concerned scenario is scattered cells illumination, and some possible solutions were listed:

Possible solutions (for information only): 1. Configure multiple SMTCs (more than 4) and SMTC selection (e.g. location-based SMTC selection, UE autonomous selection) 2. SSB-specific SMTC 3. UE assistance information reporting 4. Configure one SMTC with small periodicity, in each period (SMTC occasion) UE is indicated which PCI to use

It should be noticed that beam hopping is coming along with extended/longer SSB periodicity. It should be possible that NW beams transmission can hop from one cell group to another cell group with regular pace. for example, NW overall transmits SSB every 20ms, but hop between 4 cell groups with same pace, then SSB periodicity of each cell could be 80ms. And a single SMTC configuration with 20ms periodicity could cover all these neighbouring cells measurement. This matches the mentioned solution 4 above. Apparently, for each SMTC occasion, only a subset of neighbouring cells are transmitting SSBs and detectable, it seems helpful to configure these PCI subset for each SMTC occasions. However, even in the scenario without beam hopping, UE cannot assume that all PCIs or same PCI set will be detected in every SMTC occasion. In another word, UE implementation has to deal with the case that some PCIs are detected in one SMTC occasion and other PCIs are detected in another SMTC occasion. It is always up to UE implementation to use history information to enhance the measurements e.g., use stored PCIs/frequencies for different SMTC occasion. So we failed to see the necessary to indicate UE which PCIs to use for each SMTC occasion.

Besides, there are couple of NW implementation choices to avoid more than 4 SMTC overall for typically 6 neighbouring cells’ measurement:

Option A: deploy neighbour cells using different frequencies. SMTC is configured per frequency, 4 SMTC per frequency configuration would mean maximum n*4 SMTCs for n frequencies. It is up to network implementation to make sure that maximum 4 neighbouring cell are deployed with same frequency if their SSB transmission are all in different time window and need separated SMTC configuration.

Option B: place some(not necessary all) neighbouring cells in the same beam hopping cell group, i.e., with same SSB transmission timing. in this case, one SMTC configuration could cover neighbouring cells in the same beaming hopping group. and less than 6 SMTCs are needed. it is up to network implementation to make sure all (intra-frequency)neighbouring cell are not in more than 4 beam hopping groups.

With more SMTCs are configured, UE will need to spend time and energy to scan neighbouring cells during all configured SMTCs. According to RAN4 specification,  UE may choose to sequentially perform the measurement over the configured SMTCs, this will result in longer measurement periodicity over each neighbouring cell and reduce the measurement performance:

#### 4.2C.2.3 Measurements of intra-frequency NR cells […] The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in Table 4.2C.2.3-1 and Table 4.2C.2.3-2 is expected.

To avoid this, furthermore, proponent companies propose to allow UE selecting a subset of the configured SMTCs to measure based on UE location, e.g., UE only need to measure SMTC corresponding neighbouring cell which are closer to it (solution 1). However this will require UE know the linkage between configured SMTC and satellites/cell.

In summary, we think solution 4 one SMTC with small periodicity works well without any further enhancement.

With beam hopping, it is enough to configure one SMTC with small periodicity(i.e. equal to overall/joint beam transmission periodicity, smaller than individual cell’s SSB periodicity), and no need to increase the maximum number of configurable SMTCs.

## Access control

As below, RAN2 agreed during RAN2 #128 meeting to introduce a new barring bit, in order to bar pre-Rel19 and Rel-19 UEs not supporting DL-CE, but we have not confirmed yet if it is needed.

If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

Since it is confirmed in RANP that the SSB periodicity is extended only up to 160ms, as we agreed in RAN2 #127 meeting, there is no RAN2 impact regarding SSB configuration:

if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration; If the SSB periodicity is larger than 160ms, several parameters may need to be extended.

The new barring bit if needed will be added in SIB1 of an cell with extended SSB periodicity. if an legacy UE already detected the cell and able to read its SIB1to check the new barring bit, it also be able to read SIB1 including the SSB periodicity configuration up to 160ms. In conclusion, we see no need to kick legacy UE or UE not support extended SSB periodicity, out of the cell at the stage of SIB1 reading:

no need of new barring bit in SIB1 due to extended SSB periodicity.

## Cell reselection enhancement

It was discussed whether to prevent UE not supporting DL CE to cell reselect to a DL CE cell:

(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

more specifically, the solution is to include DL CE cells in the legacy excluded cell lists , and at the same introduce a new cell list to indicate these cells to UE capable DL CE feature. This will allow DL CE capable UEs cell-reselecting to these cells, but prevent DL CE incapable UEs from cell-reselecting to these cell at the same time.

However, with the same logic as same for not introducing new barring bit. We do not see the necessity to prevent legacy UEs to cell reselect to a cell with extended SSB periodicity, if the legacy UE by chance can detect the cell based on neighbouring measurement procedure.

no need to enhance cell reselection due to extended SSB periodicity.

## Msg4 PDSCH repetition

As agreed in RAN1[2], Msg4 PDSCH repetition can be supported. Then in the following, we will discuss how to support Msg4 PDSCH repetition. The first question is how to determine whether Msg4 PDSCH repetition is needed. If it is determined by the UE, a threshold should be configured by the network similar to PUCCH repetition of Msg4 HARQ-ACK. When a threshold is configured and the RSRP of the DL pathloss reference is less than the configured threshold, Msg4 PDSCH repetition is needed. Then the UE will request Msg4 PDSCH repetition.

RAN2 to discuss whether threshold for Msg4 PDSCH repetition is needed.

When the UE determines Msg4 PDSCH repetition is needed based on the configured threshold and measured results, it will request Msg4 PDSCH repetition to the network. If Msg4 PDSCH repetition is determined by the network, the UE also needs to inform the UE capability of supporting Msg4 PDSCH repetition to the network. In other words, regardless of who determines Msg4 PDSCH repetition, the UE capability of supporting Msg4 PDSCH repetition or a request for Msg4 PDSCH repetition can be needed. For the UE capability indication or request indication, the UE may inform the network by one of the following options:

- Option 1: RACH resource

  - Spare bit in the feature combination of RACH resource configuration can be used to indicate Msg4 PDSCH repetition

- Option 2: LCID for CCCH message of Msg3

  - Considering (e)RedCap UEs, 6 LCIDs will be needed

For option 1, a spare bit is used for Msg4 PDSCH repetition, which is very simple. But this option can also introduce more RACH resource fragmentation. If option 2 is adopted, for Msg4 PDSCH repetition, 6 LCIDs can be needed. Considering there are more reserved LCIDs in the current specification, we propose to select option 2, i.e. LCID for CCCH message of Msg3 can be used for indicating UE capability or repetition request.

LCID for CCCH message of Msg3 could be used for indicating UE request/UE capability of Msg4 PDSCH repetition.

# Conclusion

In summary, we would like to share our view on downlink coverage enhancement with following proposals:

Proposal 1:	With beam hopping, it is enough to configure one SMTC with small periodicity(i.e. equal to overall/joint beam transmission periodicity, smaller than individual cell’s SSB periodicity), and no need to increase the maximum number of configurable SMTCs.

Proposal 2:	no need of new barring bit in SIB1 due to extended SSB periodicity.

Proposal 3:	no need to enhance cell reselection due to extended SSB periodicity.

Proposal 4:	RAN2 to discuss whether threshold for Msg4 PDSCH repetition is needed.

Proposal 5:	LCID for CCCH message of Msg3 could be used for indicating UE request/UE capability of Msg4 PDSCH repetition.

# References

[1] Draft_RAN2_128_Meeting_Report_v1.

[2] Chair notes RAN1#119.