3GPP TSG RAN WG2 Meeting #129bis                          	   R2-2502542

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.5.4

Source:	Qualcomm Incorporated

Title:	Discussion on Adaptation of Common Signal/Channel Transmissions

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The objectives for adaptation of common signal and channel transmissions are outlined with the follows [1].

The objectives of the work item are the following: - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g., adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we discuss scenarios or options for adaptation of SSB in time domain, adaptation of PRACH in time domain and adaptation of paging occasions, and the potential impact to RAN2’s specification work.

# Discussion

## Adaptation of SSB in time domain

Furthermore, at RAN1 #120, the following agreements were made for adaptation of SSB in time-domain [3].

Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

Based on RAN1’s agreements, adaptation of SSB in time domain with SSB burst periodicity adapted is more like the OD-SSB Case #2 with AO-SSB and OD-SSB on the same frequency, e.g., the SSB burst periodicity using legacy signalling is like the AO-SSB, and the X additional SSB burst periodicity is like the OD-SSB.

Observation 1. The adaptation of SSB in time-domain with SSB burst periodicity adapted is similar to the OD-SSB Case #2 with AO-SSB and OD-SSB on the same frequency.

Given the commonality between on-demand SSB transmission and SSB adaptation with periodicity, a unified MAC CE may be used to indicate both on-demand SSB transmission and SSB transmission with periodicity adaptation.

Observation 2. The MAC CE for indication and termination of OD-SSB transmission may be applied to the adaptation of SSB in time domain with SSB burst periodicity adapted.

Based on the above observations, we propose the following proposal.

Proposal 1. RAN2 supports that the MAC CE for OD-SSB transmission is also used for SSB adaptation with periodicity.

At RAN2 #129, the following agreements were made for the L3 RRM measurement [4].

Agreements on OD-SSB

1.	RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

Agreements on SSB adaptation

- RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

RAN2 has indicated to RAN4 that it’s up to RAN4’s decision to measure both AO-SSB and OD-SSB or not. With the similarity between OD-SSB Case #2 and SSB adaptation with periodicity, the same L3 RRM measurement framework may be applied to both.

Observation 3. The SSB adaptation in time with periodicity adapted may follow the same L3 measurement framework of on-demand SSB Case #2, where the frequency locations of always-on SSB an adapted SSB are the same.

Observation 4. RAN2 has asked RAN4 to decide if AO-SSB and OD-SSB are measured when both are transmitted for OD-SSB Case #2.

Based on the above observations, we propose the follow proposal.

Proposal 2. For SSB adaptation with periodicity, RAN2 waits for RAN4’s decision on OD-SSB Case #2 before RAN2 starts the L3 RRM measurement design work.

## Adaptation of paging in time domain

At RAN2 #127bis, the following agreements were made for paging adaptation in time [2]

Agreements on paging adaptation

- Select option-b as baseline for R19 NES paging enhancement.

- R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Legacy UE is not barred.

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

At RAN2 #129, the following agreements were made for paging adaptation in time [3].

Agreements on paging adaptation

- For N, values smaller than T/32 are not supported.

- The maximum possible value for Ns is 8.

- Introduce a separate PEI configuration.

- Paging adaptations are configured semi-statically and updated via system information update notification.

- A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

With the new UE capability agreed by RAN2, a UE supporting Release 19 paging adaptation needs to indicate its capability to the network (e.g., gNB for initial connection or AMF for registration) and monitor the POs based on network’s instruction (e.g., gNB or AMF).

Observation 5. A Release 19 NES UE support supporting paging adaptation needs to indicate its capability to gNB.

Observation 6. A Release 19 NES UE support supporting paging adaptation needs to indicate its capability to AMF.

Observation 7. A Nework can configure Release 19 NES UE with Rel 19 paging parameters or legacy paging parameters.

Proposal 3. RAN2 discusses the message exchanges between UE and network nodes (gNB or AMF) for UE capability supporting Release 19 paging adaptation.

Based on RAN2’s agreement that a separate PEI configuration may be used for Release 19 paging adaptation, the following PEI parameters may be configured seperately for Release 19 paging adaptation.

- po-NumPerPEI: is the number of PO(s) associated with one PEI monitoring occasion. It is a factor of the total PO number in a paging cycle, i.e., N x Ns. The maximum number of PF associated with one PEI monitoring occasion is 2. With Ns increased to 8, the po-NumPerPEI may also be increased if max number of 2 PFs is supported.

- firstPDCCH-MonitoringOccasionOfPEI-O: is the offset in number of symbols, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O. When Release 19 paging adaptation is supported, a separate PEI-O may be needed for Release 19 paging adaptation. However, this can be RAN1’s final decision

Proposal 4. A new PEI configuration supporting Release 19 paging adaptation contains at least the following parameters

- po-NumPerPEI-r19

- firstPDCCH-MonitoringOccasionOfPEI-O-r19

# Conclusion

In this contribution, we discussed scenarios or options for adaptation of SSB in time domain, adaptation of PRACH in time domain and adaptation of paging occasions and concluded with the following observations and proposals.

Adaptation of SSB in time domain

Observation 1. The adaptation of SSB in time-domain with SSB burst periodicity adapted is similar to the OD-SSB Case #2 with AO-SSB and OD-SSB on the same frequency.

Observation 2. The MAC CE for indication and termination of OD-SSB transmission may be applied to the adaptation of SSB in time domain with SSB burst periodicity adapted.

Proposal 1. RAN2 supports that the MAC CE for OD-SSB transmission is also used for SSB adaptation with periodicity.

Observation 3. The SSB adaptation in time with periodicity adapted may follow the same L3 measurement framework of on-demand SSB Case #2, where the frequency locations of always-on SSB an adapted SSB are the same.

Observation 4. RAN2 has asked RAN4 to decide if AO-SSB and OD-SSB are measured when both are transmitted for OD-SSB Case #2.

Proposal 2. For SSB adaptation with periodicity, RAN2 waits for RAN4’s decision on OD-SSB Case #2 before RAN2 starts the L3 RRM measurement design work.

Adaptation of paging in time domain

Observation 5. A Release 19 NES UE support supporting paging adaptation needs to indicate its capability to gNB.

Observation 6. A Release 19 NES UE support supporting paging adaptation needs to indicate its capability to AMF.

Observation 7. A Nework can configure Release 19 NES UE with Rel 19 paging parameters or legacy paging parameters.

Proposal 3. RAN2 discusses the message exchanges between UE and network nodes (gNB or AMF) for UE capability supporting Release 19 paging adaptation.

Proposal 4. A new PEI configuration supporting Release 19 paging adaptation contains at least the following parameters

- po-NumPerPEI-r19

- firstPDCCH-MonitoringOccasionOfPEI-O-r19

# References

[1] RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

[2] “Report from session on V2X/SL, R18/19 MOB, and R19 NES” RAN2 #127bis, Hefei, China, October 14th – 18th, 2024.

[3] “Chair notes RAN1#120 eom0”, RAN1 #120, Athens, Greece, February 17th – 21st, 2025

[4] “R2_129_R17R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM”, RAN2 #129, Athens, Greece, February 17th – 21st, 2025