3GPP TSG-RAN2 Meeting #129bis	R2-2502586

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.2.4

Source:	Qualcomm Incorporated

Title:	Data Transmission and Other General Aspects of Ambient IoT

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

The Rel-19 Ambient IoT WI was approved in [1] and the RAN2 objectives were described as follows:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included. - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

RAN2 #129 made following agreements regarding the A-IoT data transmission and general aspects in terms of AS ID, segmentation, MAC PDU format and etc.

Agreements - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - From device perspective, it is only required to use one AS ID. - CFRA is not supported for group ID - RAN2 assumes, AS ID is needed for CFRA at least for inventory + command procedure - For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: - Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. - Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 - Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2

- Agreements on segmentation

- To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

- R2D segmentation is not supported for R19 A-IoT.

Agreements

From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”.

Agreements on MAC PDU format design - Aim to design simple MAC PDU format design - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages - At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). - FFS whether for D2R we need message type field, any length and need for padding - Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

In this paper, we will discuss the data communication and other aspects of Ambient IoT (A-IoT).

# Discussion

The traffic types of DO-DTT and DT with the focus on indoor inventory and command should be considered. The service requirements on inventory procedure and command procedure should be supported from RAN perspective.

In inventory procedure, the A-IoT application in core network would like to discover the A-IoT devices who are nearby the Reader(s) and sends the inventory request to Reader(s). The inventory request message may include filter criteria, and other upper layer information. The Reader that receives the inventory request performs the inventory procedure. The Reader forwards the inventory request to the A-IoT devices to be inventoried. The inventory request from Reader could be the group signaling which are sent to all A-IoT devices who have harvested enough energy and are able to receive.

In RAN2#125bis meeting, it has been agreed that the A-IoT access procedure is triggered by the Reader and Reader may provide the information that the A-IoT devices need to respond for the A-IoT access trigger. The A-IoT devices who are triggered by the inventory request will perform the A-IoT access towards the Reader in response to the received inventory request.

Figure 1 Ambient IoT inventory

In command procedure, the A-IoT application issues a Command Request including command container, (a list of) A-IoT device identifier(s) that the command is destined or filter criteria that identify the A-IoT devices that are supposed to act upon the command. Therefore, the trigger message sent from Reader may include the device information or filter criteria for triggering A-IoT devices to perform A-IoT access. The trigger message may also include the command container if A-IoT application provides commands to Reader in the command procedure.

## User plane functionalities

### Segmentation

With the segment indication, Reader schedules additional resource for the subsequent segments’ transmission. Reader is responsible for recording the received segments and estimates the remaining data size of the target A-IoT device. If an estimated D2R message size could be provided by CN or AIOTF, Reader considers that as the assistance information to determine the TB size for subsequent segments.

When A-IoT device segments the A-IoT upper layer data, the size of segment can be determined by the TB size from the Reader. One possible way is that the size of segment is always equal to the TB size. If the rest of the upper layer data is smaller than the scheduled TB size, padding bit(s) may be needed. Thus, the A-IoT device segments and transmits the A-IoT upper layer data based on the scheduled TB size from the Reade.

A-IoT device segments and transmits the A-IoT upper layer data based on the scheduled TB size from the Reader.

For the unsegmented D2R data (i.e., a complete D2R data), if the whole D2R data is failed to be successfully received by Reader, Reader may simply send the R2D message including the upper layer command again to trigger the A-IoT device to re-send the whole D2R data.

The A-IoT device should retransmit the unsegmented D2R data, upon receiving R2D message including the upper layer command again.

RAN2 #129 discussed 1-bit indication can indicate whether there are more data for the segmentation transmission if CN can provide an estimated D2R message size. Further, RAN2 agreed that Reader can explicitly indicate the number of bits successfully received. In that sense, we think the 1-bit indication is sufficient to indicate the status of remaining data/segmentation and no additional D2R data size indication is needed.

RAN2 confirms 1-bit indication about whether more data will be sent is sufficient for D2R data segmentation.

### AS ID

There were extensive discussions in the post email discussions [2] about the AS ID assignment in CFRA and CBRA and validity of AS ID. According to the summary from the email discussion, it seems most companies agree that the AS ID is applied for Inventory + command case.

For contention-free A-IoT access, the second message (‘A-IoT Msg2’) in response to the first message (‘A-IoT Msg1’) can assign AS ID together with the optional upper layer command to trigger the A-IoT data communication. In our companion paper [3], we have analyzed that the random ID in the first D2R message is neither needed nor sufficient for contention-free A-IoT access case and the device ID could be included in the first D2R message in contention-free A-IoT access. Thus, the second message in contention-free A-IoT access can be used for AS ID assignment.

In contention-free A-IoT access, the second message (‘A-IoT Msg2’) is used to assign AS ID if this message includes the upper layer command.

For contention-based A-IoT random access, RAN2 has agreed that it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. If random ID is sent in A-IoT Msg1 and random ID is echoed in A-IoT Msg2 as contention resolution, Reader can identify the A-IoT device successfully. The random ID can be used as AS ID if Reader would like to do so. Reader and A-IoT device can use the random ID as AS ID for the subsequent A-IoT data communication.

If the solution of AS ID assignment in contention-free A-IoT access can be agreed (proposal 4), the same R2D message used for contention-free case can be applied to the contention-based A-IoT random access, i.e., the R2D message that include the upper layer command can be used to assign AS ID for the subsequent A-IoT data communication.

In contention-based A-IoT random access, it is up to Reader to decide whether assign AS ID in A-IoT Msg2.

If proposal 4 is agreed, the R2D message carrying the upper layer command can be used to assign AS ID in contention-based A-IoT random access.

### R2D and D2R Data Message

For D2R data message, since Reader may not have the accurate information of the D2R data size, the TB size provided by Reader may not be always suitable for the D2R data transmission. Thus, the MAC padding is supported for D2R from RAN2 perspective.

The MAC padding is supported at least from D2R from RAN2 perspective.

The D2R data is generated from A-IoT upper layer which could be carried in MAC SDU when the upper layer data is submitted to the A-IoT MAC. The byte alignment is beneficial for the packet processing for the D2R data and segment. Since the size of D2R data or segment could be variable, a length field is needed for D2R data MAC SDU. The size of length field can be FFS.

The byte alignment for D2R data MAC SDU is supported.

A length field is supported for D2R data MAC SDU to support varying lengths of D2R data. The size of length field is FFS.

Besides the upper layer command carried in R2D data message, the AS ID is used for R2D data reception. Thus, the AS ID could be included in one MAC layer field in R2D data message, e.g., in MAC header or MAC payload unless RAN1 agree other option.

AS ID is included in MAC layer signalling unless RAN1 agree other option.

For the D2R and R2D data message, they target to schedule single A-IoT device or respond to one Reader. The MAC PDU payloads carrying D2R and R2D data message include the upper layer data and device ID/AS ID as well. Compared to the multiple MAC subPDU solution, one single MAC header for the MAC PDU can reduce the overhead caused by multiple MAC subheader. The length field and some optional assistance information can also be carried in the MAC header.

For R2D and D2R data message, a single MAC header associated with multiple MAC subPDUs in a A-IoT MAC PDU is supported.

### Logical and transport channels

In the A-IoT SI phase, it was agreed that legacy NR SDAP/PDCP/RLC protocol layers are not supported for A-IoT. In the WI scope [1], only MAC layer is included. For A-IoT MAC, not all legacy NR MAC functionalities are needed.

The NR logical channels are used to carry the control signaling and UP data. The A-IoT data can be carried in A-IoT MAC PDU. In A-IoT MAC, the upper layer signaling and A-IoT upper layer data can be mapped into one single logical channel and all MAC PDUs belonging to this one logical channel are concatenated in the single transport block to be delivered to the physical layer for transmission.

The concept of logical channels as in NR MAC is supported for A-IoT MAC.

Since RAN1 has agreed to support two physical layer channels, i.e., PRDCH and PDRCH, at least two transport channels mapping to physical channels should be supported in A-IoT MAC. Similar to NR DL-SCH and UL-SCH, A-IoT should support transport channels RD-SCH and DR-SCH.

At least two transport channels, i.e., RD-SCH and DR-SCH are supported for A-IoT.

## Control plane functionalities

During the SI phase, it was agreed that based on the targeted lower complexity and cost of A-IoT devices, traditional Control Plane functions like RRC connection management, RRM L3 measurement reporting, ASN.1 encoding/decoding, legacy periodical SIB/MIB, legacy paging message, legacy RRC layer, legacy DRX, tracking area update from the device etc. will not be supported by A-IoT.

RAN2#128 agreed:

=> RAN2 assumes that there is no other L2 AS layer (i.e. A-IoT MAC layer only).   There is no CP/UP protocol stack differentiation on A-IoT interfaces.

However, RAN2 still has to conclude on some of the remaining CP aspects.

### A-IoT Device Registration, Subscription and CN Awareness

Due to the target on very low complexity, cost as well as power consumption, and given that potentially a very large number of A-IoT devices may be present in a given vicinity but perform only bursty data transmissions always triggered by the Reader, one of the fundamental architectural differences between A-IoT and existing 3GPP technologies may be how the devices are exposed to the Core Network functions.

In our understanding, SA2 and SA3 would need to discuss and conclude on this. From RAN2 point of view, we think requiring such A-IoT devices to support legacy UE-registration-like procedures would be very ambitious not only because of the complexity of the procedure and protocol, but also due to the number of communication steps required. The A-IoT devices will not have enough energy source or energy storage to perform multiple steps just for the purpose of registration. Additionally, having a ‘subscription’ similar to legacy UE subscriptions per A-IoT device may be a high burden for the core network to manage.

The A-IoT device may harvest energy, accumulate it up to the required minimum amount of energy for communication, perform the communication steps and may fully deplete its energy. If the A-IoT device runs out of energy after some communication steps, it may not be able to store any configurations, i.e. it will likely ‘forget’ everything. In other words, it is plausible that every communication session starts as if the device has just been powered up from the power off state. Then, it would not make sense to have the A-IoT device perform the legacy registration procedures every time it needs to communicate.

Legacy UE-registration-like procedure is not supported by A-IoT.

### NAS functions

NAS control protocol in NR performs the functions listed in TS 23.501 such as registration, authentication, mobility management, security control etc. Supporting all these legacy NAS functionalities by the A-IoT devices may be infeasible especially given their complexity, cost, processing power and energy/power constraints.

In our view, the functionalities like security and authentication for A-IoT communication should be provided by A-IoT function (AIOTF) using AIoT NAS protocol. We understand details are being discussed in SA2.

Legacy NAS functions are not supported by A-IoT devices.

# Protocol stack

Based on the above discussion on the needed functionalities, in our view the overall AS protocol stack may look something like below. This would be applicable to both A-IoT CP and UP (i.e. no differentiation between CP and UP protocol stacks as already agreed during SI phase).

Figure 2 A-IoT protocol stack

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1:	A-IoT device segments and transmits the A-IoT upper layer data based on the scheduled TB size from the Reader.

Proposal 2:	The A-IoT device should retransmit the unsegmented D2R data, upon receiving R2D message including the upper layer command again.

Proposal 3:	RAN2 confirms 1-bit indication about whether more data will be sent is sufficient for D2R data segmentation.

Proposal 4:	In contention-free A-IoT access, the second message (‘A-IoT Msg2’) is used to assign AS ID if this message includes the upper layer command.

Proposal 5:	In contention-based A-IoT random access, it is up to Reader to decide whether assign AS ID in A-IoT Msg2.

Proposal 6:	If proposal 4 is agreed, the R2D message carrying the upper layer command can be used to assign AS ID in contention-based A-IoT random access.

Proposal 7:	The MAC padding is supported at least from D2R from RAN2 perspective.

Proposal 8:	The byte alignment for D2R data MAC SDU is supported.

Proposal 9:	A length field is supported for D2R data MAC SDU to support varying lengths of D2R data. The size of length field is FFS.

Proposal 10:	AS ID is included in MAC layer signalling unless RAN1 agree other option.

Proposal 11:	For R2D and D2R data message, a single MAC header associated with multiple MAC subPDUs in a A-IoT MAC PDU is supported.

Proposal 12:	The concept of logical channels as in NR MAC is supported for A-IoT MAC.

Proposal 13:	At least two transport channels, i.e., RD-SCH and DR-SCH are supported for A-IoT.

Proposal 14:	Legacy UE-registration-like procedure is not supported by A-IoT.

Proposal 15:	Legacy NAS functions are not supported by A-IoT devices.

# References

- RP-250796, Solutions for Ambient IoT (Internet of Things) in NR.

- R2-2501965, Report of [POST129][036][AIoT] AS ID (Xiaomi), Xiaomi.

- R2-2502585, Views on Random Access Aspects of Ambient IoT, Qualcomm.