3GPP TSG-RAN2 #129bis	R2-2502600

Wuhan, China, Apr.  7th – 11th, 2025

Agenda Item:		7.2

Source:		Ofinno

Title:	discussion on Rel-18 LTM

Document for:		Discussion and Decision

# RRC issue: UE based TA measurement

According to the section 5.3.5.18.1 and 5.3.5.18.3 of TS 38.331, a UE determine whether to configure UE based TA measurements for each LTM candidate cell only if the received LTM-Config includes the ltm-CandidateToAddModList.

Observation 1: a UE determine whether to configure UE based TA measurements for each LTM candidate cell only if the received LTM-Config includes the ltm-CandidateToAddModList.

According to the section 5.3.5.18.1 of TS 38.331, if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID: a UE updates a value of replace the ltm-ServingCellUE-MeasuredTA-ID value within VarLTM-ServingCellUE-MeasuredTA-ID with the received ltm-ServingCellUE-MeasuredTA-ID.

Observation 2: if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID: a UE updates a value of replace the ltm-ServingCellUE-MeasuredTA-ID value within VarLTM-ServingCellUE-MeasuredTA-ID with the received ltm-ServingCellUE-MeasuredTA-ID

##### 5.3.5.18.1 LTM configuration The network configures the UE with one or more LTM candidate configurations within the LTM-Config IE. … The UE shall perform the following actions based on the received LTM-Config IE: … 1> if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID: 2> if the current VarLTM-ServingCellUE-MeasuredTA-ID includes an ltm-ServingCellUE-MeasuredTA-ID: 3> replace the ltm-ServingCellUE-MeasuredTA-ID value within VarLTM-ServingCellUE-MeasuredTA-ID with the received ltm-ServingCellUE-MeasuredTA-ID; 2> else: 3> store the received ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID; 1> if the received LTM-Config includes the ltm-CandidateToReleaseList: 2> perform the LTM candidate configuration release as specified in 5.3.5.18.2; 1> if the received LTM-Config includes the ltm-CandidateToAddModList: 2> perform the LTM candidate configuration addition or modification as specified in 5.3.5.18.3; 1> reconfigure the UE according to all other fields of the received LTM-Config IE. ##### 5.3.5.18.3 LTM candidate configuration addition/modification The UE shall: 1> for each ltm-CandidateId value included in the ltm-CandidateToAddModList: 2> if the current UE configuration contains an LTM-Candidate with the ltm-CandidateId value: 3> reconfigure the corresponding LTM-Candidate in accordance with the received LTM-Candidate; 2> else: 3> add the received LTM-Candidate; 2> if the LTM-Candidate with the received ltm-CandidateId value includes ltm-UE-MeasuredTA-ID: 3> if the value of ltm-UE-MeasuredTA-ID is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID: 4> inform lower layers that the UE is configured with UE-based TA measurements for this LTM-Candidate; 3> else: 4> inform lower layers that the UE is not configured with UE-based TA measurements for this LTM-Candidate;

According to TS 38.331, if the received LTM-Config include ltm-ServingCellUE-MeasuredTA-ID, the received LTM-Config may not include the ltm-CandidateToAddModList. That is, there is no description about whether the received LTM-Config should include the ltm-CandidateToAddModList.

Observation 3: if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID, the received LTM-Config may not include the ltm-CandidateToAddModList.

If the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID but not include the ltm-CandidateToAddModList, the UE doesn’t determine whether to configure UE based TA measurements for each LTM candidate cell (see the observation 1). It causes mismatch of UE-based TA measurements between the UE and the network.

Observation 4: if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID but not include the ltm-CandidateToAddModList, mismatch of UE-based TA measurements between the UE and the network happens.

To avoid the mismatch of UE-based TA measurements between the UE and the network, the following alternatives can be considered:

- A UE determines to configure the lower layer for UE based TA measurements for each LTM candidate cell if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID but not include the ltm-CandidateToAddModList

- the ltm-CandidateToAddModList is mandatory present if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID

Proposal: To avoid the mismatch of UE-based TA measurements between the UE and the network, RAN2 agreed one of the following alternatives:

- A UE determines to configure the lower layer for UE based TA measurements for each LTM candidate cell if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID but not include the ltm-CandidateToAddModList

- the ltm-CandidateToAddModList is mandatory present if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID

# 2. Observations and Proposals

RRC issue

Observation 1: a UE determine whether to configure UE based TA measurements for each LTM candidate cell only if the received LTM-Config includes the ltm-CandidateToAddModList.

Observation 2: if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID: a UE updates a value of replace the ltm-ServingCellUE-MeasuredTA-ID value within VarLTM-ServingCellUE-MeasuredTA-ID with the received ltm-ServingCellUE-MeasuredTA-ID

Observation 3: if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID, the received LTM-Config may not include the ltm-CandidateToAddModList.

Observation 4: if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID but not include the ltm-CandidateToAddModList, mismatch of UE-based TA measurements between the UE and the network happens.

Proposal: To avoid the mismatch of UE-based TA measurements between the UE and the network, RAN2 agreed one of the following alternatives:

- A UE determines to configure the lower layer for UE based TA measurements for each LTM candidate cell if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID but not include the ltm-CandidateToAddModList

- the ltm-CandidateToAddModList is mandatory present if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID

# 3. Annex for TP

# 3.1. TP for Alt 1 of proposal

Start of Change

##### 5.3.5.18.1	LTM configuration

The network configures the UE with one or more LTM candidate configurations within the LTM-Config IE.

In NR-DC, the UE may receive two independent ltm-Config:

-	an ltm-Config associated with the MCG that is included within an RRCReconfiguration message received via SRB1; and

-	an ltm-Config associated with the SCG that is included within an RRCReconfiguration message either received via SRB3, or, alternatively, embedded in an RRCReconfiguration message received via SRB1.

In case the UE receives two independent ltm-Config:

-	the UE maintains two independent ltm-Config;

-	the UE maintains two independent VarLTM-ServingCellNoResetID, one associated with each ltm-Config;

-	the UE maintains two independent VarLTM-ServingCellUE-MeasuredTA-ID, one associated with each ltm-Config;

-	the UE independently performs all the procedures in clause 5.3.5.18 for each ltm-Config and the associated VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID unless explicitly stated otherwise.

The UE shall perform the following actions based on the received LTM-Config IE:

1>	if the received LTM-Config includes ltm-ServingCellNoResetID:

2>	if the current VarLTM-ServingCellNoResetID includes an ltm-ServingCellNoResetID:

3>	replace the ltm-ServingCellNoResetID value within VarLTM-ServingCellNoResetID with the received ltm-ServingCellNoResetID;

3>	if the received LTM-Config doesn’t include the ltm-CandidateToAddModList:

4>for each LTM-Candidate IE in ltm-Config:

5>	if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

6>	inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate;

5>	else:

6>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

2>	else:

3>	store the received ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID;

1>	if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID:

2>	if the current VarLTM-ServingCellUE-MeasuredTA-ID includes an ltm-ServingCellUE-MeasuredTA-ID:

3>	replace the ltm-ServingCellUE-MeasuredTA-ID value within VarLTM-ServingCellUE-MeasuredTA-ID with the received ltm-ServingCellUE-MeasuredTA-ID;

2>	else:

3>	store the received ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID;

1>	if the received LTM-Config includes the ltm-CandidateToReleaseList:

2>	perform the LTM candidate configuration release as specified in 5.3.5.18.2;

1>	if the received LTM-Config includes the ltm-CandidateToAddModList:

2>	perform the LTM candidate configuration addition or modification as specified in 5.3.5.18.3;

1>	reconfigure the UE according to all other fields of the received LTM-Config IE.

End of Change

  - TP for Alt 2 of proposal

Start of Change

#### –	LTM-Config

The IE LTM-Config is used to provide LTM configurations.

LTM-Config information element

-- ASN1START

-- TAG-LTM-CONFIG-START

LTM-Config-r18 ::=   SEQUENCE {

ltm-ReferenceConfiguration-r18        SetupRelease {ReferenceConfiguration-r18}                             OPTIONAL,   -- Need M

ltm-CandidateToReleaseList-r18        SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18    OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18      OPTIONAL,   -- Need N

ltm-ServingCellNoResetID-r18          INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Cond LTM-ServingCellUE-MeasuredTA-ID

ltm-CSI-ResourceConfigToAddModList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18

OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18

OPTIONAL,   -- Need N

attemptLTM-Switch-r18                 ENUMERATED {true}                                                    OPTIONAL, -- Cond LTM-MCG

ltm-ServingCellUE-MeasuredTA-ID-r18   INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

...

}

-- TAG-LTM-CONFIG-STOP

-- ASN1STOP

| Conditional Presence | Explanation |
| --- | --- |
| LTM-MCG | This field is optional present for the MCG, Need R, if the UE is configured with at least an LTM candidate configuration associated to the MCG. Otherwise, the field absent. |
| LTM-ServingCellUE-MeasuredTA-ID | The field is mandatory present when LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID. Otherwise the field is optional, need N. |

End of Change