3GPP TSG-RAN2 #129bis	R2-2502601

Wuhan, China, Apr.  7th – 11th, 2025

Agenda Item:		13.2

Source:		Ofinno

Title:	Discussion on open issues of inter-CU LTM

Document for:		Discussion and Decision

# Introduction

In this contribution, we discuss open issues on inter-CU LTM.

# MCG LTM with SCG

# 2.1. L2 reset of SCG within MCG LTM

For L3 handover (e.g., normal HO or CHO) or normal/conditional SCG change, L2 reset parameter(s) (i.e., reestablishPDCP, recoverPDCP, reestablishRLC) are used for L2 reset. However, for MCG/SCG LTM supporting subsequent cell switch, when the network transmits to a UE an LTM config, the network is not able to know that LTM cell switch will happen from which cell to which cell during the subsequent LTM cell switch. Thus, Rel-18 L2 reset IDs (i.e., ltm-NoResetID, ltm-ServingCellNoResetID) were introduced for Rel-18 intra-CU LTM. Rel-19 L2 reset IDs (i.e., ltm-NoSeurityChangeID, ltm-ServingCellNoSecurityChangeID) were introduced for Rel-19 inter-CU LTM.

Observation 1: for L2 reset, L2 reset IDs are introduced for MCG/SCG LTM supporting subsequent cell switch while L2 reset parameter(s) (i.e., reestablishPDCP, recoverPDCP, reestablishRLC) are used for L3 HO or normal/conditional SCG change.

In RAN #128 meeting, RAN2 agreed to support MCG with SCG change as shown in below.

Observation 2: MCG with SCG change is supported.

… 4. RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it. 5. RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19. 6. From RAN2 perspective, the following coexistence cases in NR-DC can be supported: - Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM - Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM 7. In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations. 8. RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message. 9. RAN2 to support intra-CU MCG LTM with SCG configuration.

Based on observation 1, for L2 reset of SCG within MCG LTM, L2 reset parameter(s) (i.e., reestablishPDCP, recoverPDCP, reestablishRLC) are used while for L2 reset of MCG LTM, L2 reset IDs are used.

Obervation 3: for L2 reset of SCG within MCG LTM, L2 reset parameter(s) (i.e., reestablishPDCP, recoverPDCP, reestablishRLC) are used

According to RAN2 agreements and 3GPP specifications, the MCG LTM is allowed to be coexisted with other SCGs (e.g., normal SCG change, conditional SCG change, subsequent SCG change, SCG LTM). For example, an inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM is allowed to be coexisted. The MCG LTM with SCG may be allowed to be coexisted with other SCGs (e.g., normal SCG change, conditional SCG change, subsequent SCG change, SCG LTM).

Obervation 4: MCG LTM with SCG can be coexisted with other SCGs (e.g., normal SCG change, conditional SCG change, subsequent SCG change, SCG LTM).

If the MCG LTM with SCG is allowed to be coexisted with other SCGs, a network may transmit to a UE an MCG LTM configuration with a first SCG configuration and transmit a second SCG configuration. The UE could apply the second SCG configuration firstly and the first SCG configuration later or conversely. When the network transmits the first SCG configuration via the MCG LTM configuration, the network may not know the order of which of the first SCG configuration and the second SCG configuration is applied.

Obervation 5: when the network transmits SCG configuration within MCG LTM configuration, the network may not know whether the SCG configuration is applied before or after other SCG configuration is applied.

It can cause a problem on L2 reset of the SCG configuration within MCG configuration. The FIG. 1 shows the example of the problem in inter-CU SCG change scenario.

When a network transmits to the UE the MCG LTM configuration comprising the first SCG configuration of a candidate PSCell A, the network may assume that a PSCell of the UE is changed from the serving PSCell of a S-SN to the candidate PSCell A of T-SN when the UE applied the first SCG configuration. Thus, the network transmits to the UE, via the first SCG configuration, the L2 reset parameter indicating to perform the L2 reset. However, the UE may perform PSCell change from the serving PSCell to a candidate PSCell B of the T-SN before applying the first SCG configuration. The PSCell change may be triggered based on normal SCG change, conditional SCG change, subsequent SCG change or SCG LTM. The candidate cell B becomes new serving PSCell. After the PSCell change, the UE may receive an LTM cell switch command for MCG. The UE may perform an LTM cell switch procedure for MCG. During the LTM cell switch procedure, the UE applies the first SCG configuration and perform L2 reset based on the L2 reset parameter. However, the network doesn’t perform L2 reset because it is intra-CU PSCell change from the candidate cell B (i.e., new serving cell) to the candidate cell A. It causes mismatch about L2 reset between the UE and the network.

Obervation 6: it causes a problem for L2 reset, of SCG within MCG LTM configuration, based on L2 reset parameter (i.e., reestablishPDCP, recoverPDCP, reestablishRLC) when a UE performs the MCG LTM after PSCell change based on other SCG configuration.

Thus, to avoid the problem, L2 reset IDs needs to be used for L2 reset of SCG within MCG LTM.

Proposal 1: RAN2 agree to use L2 reset IDs for L2 reset of SCG within MCG LTM configuration instead of L2 reset parameter(s) (i.e., reestablishPDCP, recoverPDCP, reestablishRLC).

Figure 1. example of L2 reset problem in inter-CU SCG change

2.2. LTM fast recovery with MCG LTM with SCG

According to the section 5.3.7.3 of TS 38.331, for LTM fast recovery, a UE is allowed to use a configuration for MCG LTM with SCG. RAN2 needs to discuss whether a configuration for MCG LTM with SCG is considered for a LTM fast recovery.

#### 5.3.7.3 Actions following cell selection while T311 is running Upon selecting a suitable NR cell, the UE shall: … 1> if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and 1> if attemptLTM-Switch is configured; and 1> if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG: 2> perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

Proposal 2. RAN2 discuss whether a configuration for MCG LTM with SCG is considered for LTM fast recovery.

2.3. Eary TA acquisition of SCG within MCG LTM

For a candidate PSCell of SCG LTM, an early TA acquisition is supported. A UE may skip a random access procedure toward the candidate PSCell using a TA acquired by the early TA acquisition procedure. It reduces a switching time of a SCG LTM cell switch procedure for the candidate PSCell. However, when a UE applied a SCG configuration within MCG LTM configuration. the UE need to perform a random access procedure toward a candidate PSCell of SCG within MCG LTM.

Observation 7: early TA acquisition is supported for candidate PSCell of SCG LTM while not supported for a candidate PSCell of SCG within MCG LTM.

For example, the candidate PSCell of SCG within MCG LTM may be of SCG LTM. The UE may acquire a TA value of the candidate PSCell based on UE based TA measurements. The UE may use the TA value of the candidate PSCell to access to the candidate PSCell. It is beneficial to use the TA of the candidate PSCell for access to the candidate PSCell.

Proposal 3: RAN2 discuss whether to support early TA acquisition for a candidate PSCell of SCG within MCG LTM.

# LTM fast recovery

According to SA3 LS, S3-251124, for security handling for inter-CU LTM in non-DC cases, SA3 agreed to select the option, MAC CE indicating NCC in plaintext.

Observation 8: for security handling for inter-CU LTM in non-DC cases, SA3 agreed to select the option, MAC CE indicating NCC in plaintext.

SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

If NCC is transmitted via MAC CE, a UE is not able to have NCC of other LTM candidate cells in advance. Thus, the UE is not able to perform the LTM fast recovery for an LTM candidate cell if the UE does not have NCC associated with the LTM candidate cell. Thus, the UE is not able to perform the LTM fast recovery between candidate cells of different CUs.

Observation 9: if NCC is provided via MAC CE, a UE is not able to have NCC of other LTM candidate cells in advance.

Observation 10: a UE is not able to perform the LTM fast recovery for an LTM candidate cell if the UE does not have NCC associated with the LTM candidate cell.

Observation 11: the UE is not able to perform the LTM fast recovery between candidate cells of different CUs.

Thus, the UE is only able to perform the LTM fast recovery between candidate cells of the same CU. The UE is able to identify whether the candidate cell is of the same CU based on Rel-19 L2 IDs (i.e., ltm-NoSeurityChangeID, ltm-ServingCellNoSecurityChangeID).

Proposal 4: LTM fast recovery for a candidate cell is supported if value of Rel-19 L2 ID of serving cell (i.e., ltm-ServingCellNoSecurityChangeID) is equal to value of Rel-19 L2 ID of the candidate cell (i.e., ltm-NoSeurityChangeID).

# SCG LTM

Agreements on RRC issues: 1. For inter-CU SCG LTM, introducing the SK counter list in LTM-ConfigSCG-r19, each list associated with one ltm-NoSecurityChangeID

In RAN2 #129 meeting, RAN2 agreed to introduce SK counter list in SCG LTM configuration. In Rel-18, SK counter list was introduced for SCPAC. A SK counter can be used for SCG within MCG LTM, normal PSCell change/addition and conditional PSCell change/addition. If a value of one SK counter is overlapped with a value of the other SK counter, it can cause security problem.

Proposal 5: RAN2 confirms a value of SK count list for SCG LTM is not overlapped with a value of other SK counters (e.g., SK counter list for SPCAC, SK counter for SCG within MCG LTM, SK counter for normal PSCell change/addition or SK counter for conditional PSCell change/addition).

# CG resource for LTM

A network may postpone allocating a RACH resource for LTM until transmitting an LTM cell switch command indicating the RACH resource. However, the network may allocate a configured grant (CG) resource when transmitting to a UE an LTM configuration. The network may need to keep allocating the CG resource of a candidate cell until the UE execute an LTM cell switch procedure for the candidate cell. The UE may not execute the LTM cell switch procedure for the candidate cell. The CG resource is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

Observation 12: a CG resource of a candidate cell for LTM is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

The network may need to release or update the CG resource (e.g., to avoid the waste of the CG resource). For releasing or updating the CG resource, the network needs to transmit an RRC reconfiguration message comprising an LTM configuration. It causes signal overheads and is not desired for subsequent LTM without the RRC reconfiguration.

Observation 13: it causes signal overheads and is not desired for subsequent LTM without the RRC reconfiguration for an RRC reconfiguration message to release or update a CG resource of the candidate cell.

Proposal 6: RAN2 discuss dynamic management of CG resource of candidate cell for LTM.

# Observations and Proposals

MCG LTM with SCG

- L2 reset of SCG within MCG LTM

  - Observation 1: for L2 reset, L2 reset IDs are introduced for MCG/SCG LTM supporting subsequent cell switch while L2 reset parameter(s) (i.e., reestablishPDCP, recoverPDCP, reestablishRLC) are used for L3 HO or normal/conditional SCG change.

  - Observation 2: MCG with SCG change is supported.

  - Obervation 3: for L2 reset of SCG within MCG LTM, L2 reset parameter(s) (i.e., reestablishPDCP, recoverPDCP, reestablishRLC) are used

  - Obervation 4: MCG LTM with SCG can be coexisted with other SCGs (e.g., normal SCG change, conditional SCG change, subsequent SCG change, SCG LTM).

  - Obervation 5: when the network transmits SCG configuration within MCG LTM configuration, the network may not know whether the SCG configuration is applied before or after other SCG configuration is applied.

  - Obervation 6. it causes a problem for L2 reset, of SCG within MCG LTM configuration, based on L2 reset parameter (i.e., reestablishPDCP, recoverPDCP, reestablishRLC).

  - Proposal 1. RAN2 agree to use L2 reset IDs for L2 reset of SCG within MCG LTM configuration instead of L2 reset parameter(s) (i.e., reestablishPDCP, recoverPDCP, reestablishRLC).

- LTM fast recovery with MCG LTM with SCG

  - Proposal 2. RAN2 discuss whether a configuration for MCG LTM with SCG is considered for LTM fast recovery.

- Eary TA acquisition of SCG within MCG LTM

  - Observation 7: early TA acquisition is supported for candidate PSCell of SCG LTM while not supported for candidate PSCell of SCG within MCG LTM.

  - Proposal 3: RAN2 discuss whether to support early TA acquisition for candidate PSCell of SCG within MCG LTM.

LTM fast recovery

- Observation 8: for security handling for inter-CU LTM in non-DC cases, SA3 agreed to select the option, MAC CE indicating NCC in plaintext.

- Observation 9: if NCC is provided via MAC CE, a UE is not able to have NCC of other LTM candidate cells in advance.

- Observation 10: a UE is not able to perform the LTM fast recovery for an LTM candidate cell if the UE does not have NCC associated with the LTM candidate cell.

- Observation 11: the UE is not able to perform the LTM fast recovery between candidate cells of different CUs.

- Proposal 4: LTM fast recovery for a candidate cell is supported if value of Rel-19 L2 ID of serving cell (i.e., ltm-ServingCellNoSecurityChangeID) is equal to value of Rel-19 L2 ID of the candidate cell (i.e., ltm-NoSeurityChangeID).

SCG LTM

- Proposal 5: RAN2 confirms a value of SK count list for SCG LTM is not overlapped with a value of other SK counters (e.g., SK counter list for SPCAC, SK counter for SCG within MCG LTM, SK counter for normal PSCell change/addition or SK counter for conditional PSCell change/addition).

CG resource for LTM

- Observation 12: a CG resource of a candidate cell for LTM is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

- Observation 13: it causes signal overheads and is not desired for subsequent LTM without the RRC reconfiguration for an RRC reconfiguration message to release or update a CG resource of the candidate cell.

- Proposal 6: RAN2 to discuss dynamic management of CG resource of candidate cell for LTM.