3GPP TSG-RAN-WG2 Meeting #129bis	                                   R2-2502636

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.1.2.2

Source: 	Sharp

Title:  	On LCM for UE-sided Model for Beam Management Use Case

Document for: 	Discussion

# Introduction

In RAN2#129, following agreements were made [1]:

Agreements

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

- Agreements applicability reporting and management

Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.   FFS if we report explicit cause

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

The provided periodic CSI configuration should be consistent with reported UE capabilities

FFS option B

Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

Semi-persistent reporting, activated by MAC CE/DCI

Aperiodic CSI reporting, activated by DCI

This tdoc examines the FFS topics outlined in the above RAN2 agreements, with a focus on the reporting of functionality applicability, taking into account the LS from RAN1. It addresses key considerations for reporting functionality applicability, particularly when applicability changes, including the provision of feedback or cause for inapplicability. Additionally, it touches on certain aspects related to functionality (de) activation. This tdoc also discuss challenges arising from frequent changes or updates in functionality applicability, which could adversely affect signaling overhead, system performance, and the network’s capacity to efficiently manage AI/ML functionalities.

# Discussion on Applicable Functionality Reporting

RAN 1 made the following agreements related to the Questions/LS from RAN 2:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

RAN 1 has identified two options for applicability-related reporting: Option A (Full Inference Configuration) and Option B (Set(s) of Inference-Related Parameters). The applicability report (Functionality Applicability) is based on these two options:

- Option A: Requires the configuration of CSI-ReportConfig to support the full inference configuration.

- Option B: Requires the provision of inference-related parameters, which must be appropriately selected.

At the RAN2 #129 meeting, it was discussed whether both Option A and Option B should be considered and supported or if the focus should initially be on Option A, with Option B addressed later if necessary. In our view, it is premature to narrow down the options at this stage, particularly without prior discussion or coordination with RAN1. Given that RAN1 is currently assessing both options, RAN2 should discuss and develope signaling procedures for both Option A and Option B accordingly, ensuring a comprehensive approach until further clarity is provided.

Proposal 1: RAN 2 to support both Option A and Option B for applicability reporting.

At the RAN2 #129 it was also discussed whether to use "OtherConfig" to configure option B to the UE and "CSI-ReportConfig" for configuring option A to the UE for the AI/ML Beam Management use case.

A notable support by many companies was observed for “OtherConfig” for configuring Option B, largely due to its ability to reduce signaling overhead and its flexibility for future use cases. However, some companies raised concerns about tailoring the design to specific use cases or the need for separate procedures for options A and B, with questions arising about the necessity of distinct procedures. Some companies also suggested to configure both options A and B within OtherConfig, though there was no consensus on this. For Option A, most companies were in favor of using CSI-ReportConfig.

However, concerns were raised by some companies due to its complexity and the inclusion of lengthy, unnecessary information elements (IEs). The possibility of configuring the UE with both options was also brought up. Overall, while Otherconfig enjoys broad support, the discussion underscores unresolved issues related to implementation and compatibility between the two options. At offline [AT129][026] [AI Phy] Signaling for BM discussion [2] several options were discussed for configuring Option A and Option B using CSI-ReportConfig and OtherConfig. respectively. We think that since Option B consists of a partial inference configuration with mostly PHY layer parameters, it should be indicated in the CSI-ReportConfig or in CSI-MeasConfig format following legacy reporting procedure as baseline.

Proposal 2: RAN 2 to support configuring Option A for applicability reporting in CSI-ReportConfig.

Porposal 3: RAN 2 to support configuring Option B also using CSI-ReportConfig. format or in CSI-MeasConfig.

# Discussion on Reporting of functionality applicability with feedback or cause

In this section, the reporting of the changes of the applicability of a functionality is discussed based on the following agreements in RAN2 #129.

- Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.   FFS if we report explicit cause

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

In RAN2 #129 [1], it was discussed and agreed to support the explicit reporting of applicability or inapplicability in the initial report while any subsequent reports will be sent only when there is a change in applicability (e.g., via UAI). However, whether to report the explicit cause for inapplicability is still FFS.

However, if one or more configurations provided by the gNB are inapplicable due to UE-side conditions, such as hardware/battery or compute resource limitations, environmental factors, capability restrictions, or model unavailability/obsolescence, and the cause of inapplicability is not reported by the UE, it leads to several challenges. First, the gNB may attempt to configure the UE with a new configuration while waiting for the status update on the reported inapplicable configuration.

The network may also repeatedly attempt to (re)configure the UE, assuming the inapplicability is temporary or resolvable (e.g., by providing an updated or a new configuration), which results in multiple RRC Reconfiguration messages. This not only creates unnecessary signaling overhead but also causes inefficient use of radio resources, wasting valuable network capacity.

Additionally, repeated reconfiguration attempts can delay valid reconfiguration procedures, increasing overall latency. The frequent attempts also put a strain on UE power consumption, affecting UE battery life. Moreover, the gNB might misallocate resources, assuming that the UE can eventually support the requested configuration or one of the configurations provided to the UE will be applicable anytime soon, which could lead to suboptimal resource distribution and network inefficiencies.

Persistent reconfiguration attempts may also cause an unstable RRC state at the UE, complicating network management. This issue stems from the lack of detailed feedback regarding the cause of inapplicability, which prevents the gNB from understanding whether the problem is temporary or permanent, or if it is due to UE-side conditions or other factors. As a result, the gNB may misinterpret the inapplicability as a transient issue, continuing to send reconfiguration requests, which exacerbates signaling overhead and other network inefficiencies.

Observation 1: Lack of feedback regarding the cause of inapplicability from the UE can lead to significant inefficiencies in signaling, resource usage and allocation, increased latency, and higher power consumption at the UE-side.

Proposal 4: RAN2 to agree that cause for inapplicability can be reported by the UE.

The cause of inapplicability may also assist the network in better planning and managing the data collection process, particularly in terms of resource allocation and timing.

Proposal 5: RAN2 to discuss signaling procedure to report the cause for inapplicability from the UE to the network.

The cause for inapplicability may not need to be reported every time; therefore, the network can either request the inapplicability cause when needed or configure the UE to report it only when necessary.

Proposal 6: The network may request the cause for inapplicability from the UE when needed.

Another issue is how to manage and avoid frequent activation-deactivation of functionality or configurations when they become applicable or inapplicable respectively. When a functionality becomes inapplicable after activation, the UE is required to report it to the network. Frequent changes in applicability/inapplicability may lead to increased signaling from the UE to the network. If due to fluctuating conditions, the applicability of a functionality changes frequently, it may lead to the "ping-pong" phenomenon, where the functionality repeatedly activate and deactivate. This situation not only creates confusion and inefficiencies within UE but also negatively impacts the overall user experience. Furthermore, it can result in unnecessary signaling (e.g., frequent UAI updates from the UE) between the UE and the network, increasing the signaling overhead. Frequent changes in functionality applicability may further exacerbate this issue, introducing additional overhead, affecting system performance, and complicating the management of AI/ML-based functionalities by the network.

The frequent changes between different functionality statuses can significantly impact functionality LCM. These frequent transitions may require more resources for performance monitoring and may lead to repeated activation and deactivation, which results in higher signaling and resource overhead.

Observation 2: Applicable functionalities are dynamic in nature and may fluctuate due to variations in the UE's environment, network-side additional conditions, UE-side additional conditions, or the availability of the AI/ML model.

From the network's perspective, it is preferable to choose functionalities for inference that are likely to remain applicable for an extended period and are not prone to frequent status changes—particularly those affected by dynamic UE-side conditions. To assist the network in selecting the appropriate configurations, it would be beneficial for the UE to provide more than just the basic applicability information.

Specifically, the UE could send an indication called "applicable functionality stability index". The applicable functionality stability index may help the UE and the network to evaluate and determine the stability of an applicable functionality for example, the probability of how long a functionality may remain applicable under given conditions. If the UE anticipates that a certain inference operation might for example, exceed its processing capacity or that a model is only applicable under specific conditions (such as the current UE speed), this could be reflected in applicable functionality stability index and indicated to the network.

The UE may compute the applicable functionality index using for example, UE side internal conditions, environmental factors and other related factors. With the help of applicable functionality stability index, the UE determines probability (estimation) of how long a functionality maybe ‘applicable’ (ready to be activated) if not activated and how long it can stay activated once it is activated.

This " applicability stability index " indication may not necessarily need to explain the explicit reasons for potential changes or instability in functionality status. Instead, the UE could simply communicate the duration or probability of how long the functionalities may remain applicable. With this information, the network can make a more informed decision, balancing signaling/resource overhead and system performance guarantees. If the functionality’s applicability is expected to remain stable for a relatively long period, fewer resources would be needed for monitoring. On the other hand, if the applicability is uncertain or expected to change quickly, the network may decide not to activate the functionality or may choose to activate it with additional resources dedicated to performance monitoring.

By incorporating the applicable functionality stability index, the network can optimize functionality management, reduce unnecessary signaling, and improve the overall performance of the system.

Observation 3: Frequent functionality applicability changes or updates could negatively affect signaling overhead, system performance, and the network’s ability to manage AIML functionality effectively.

Proposal 7: RAN2 should explore methods to minimize the necessity for frequent applicability status updates facilitating the stability of inference operation once activated.

Proposal 8: The UE could provide a “applicable functionality stability index” indication for each applicable functionality, among other potential approaches, to enhance operational consistency and reduce signaling demands.

# Discussion on Functionality Activation

In this section, the activation/deactivation of a functionality or configuration is discussed based on the following agreements in RAN2 #129 [1].

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

The provided periodic CSI configuration should be consistent with reported UE capabilities

FFS option B

Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

Semi-persistent reporting, activated by MAC CE/DCI

Aperiodic CSI reporting, activated by DCI

Fig. 1 depicts the signalling framework used as baseline in RAN2 discussion for beam management use case for UE-sided model:

Figure 1: Signaling flow for applicable functionality reporting

The final applicability of a functionality may be determined either by the UE or the network, depending on whether the UE has sufficient information to evaluate the functionality's applicability. If the UE has adequate information, such as full CSI-ReportConfig (inference configurations), associated ID(s) or additional conditions, it may autonomously determine the applicability.

The potential for multiple functionality/inference configurations (e.g., CSI-ReportConfig.) provided to the UE, as well as the possibility of simultaneously activating multiple functionalities, are critical factors that must be incorporated to enhance flexibility and ensure optimal performance. In a beam management use case, the network can leverage AI/ML inference configurations to optimize beam prediction, selection and signal quality. These inference configurations or parameters can be communicated to the UE through AI/ML CSI-ReportConfigs.

As per the RAN 1 agreements [3], for beam management, multiple CSI-ReportConfigs for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. This implies that each UE may be assigned multiple CSI-ReportConfig instances, tailored for different reporting types, such as periodic or aperiodic or semi-persistent reporting.

In Step 4 (Fig. 1), the UE reports the applicability and inapplicability of one or more CSI-ReportConfigs and/or sets of inference-related parameters. It is still to be determined whether any additional information, apart from the applicability, needs to be included in the applicability report. One or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI-ReportConfig. as a working assumption) is configured in Step 3, the applicable aperiodic and semi-persistent CSI reports can be activated or triggered by the network after receiving the applicability report from the UE using MAC CE or DCI indication [1]. For applicable periodic CSI reports, they are activated by the UE only if the applicability of the corresponding CSI-ReportConfig is reported in the RRCReconfigurationComplete message to the network (as per RAN2 # 129 agreement [1]). However, it is not clear what should be the UE behaviour when an applicable periodic CSI-ReportConfig. becomes inapplicable. In this case, it should be discussed if the UE can autonomously deactivate or de-configure the functionality or wait for the network to de-activate it.

Proposal 9: RAN2 to discuss and define UE behaviour when an applicable periodic configuration/functionality (CSI-ReportConfig) becomes inapplicable.

In Step 5, the network can optionally configure the CSI-ReportConfig for inference configuration through the RRCReconfiguration message. The Step 5 may be optional if the UE has already been configured with the CSI-ReportConfig in Step 3 and may only be applicable to semi-persistent and aperiodic CSI reporting case.

This allows flexibility in measurement and reporting of applicable functionality based on specific conditions or requirements. However, with multiple configurations present, both the network and UE may need to decide on a primary report or configuration to use at any given time.

To facilitate this selection process, the network can designate a default or preferred CSI-ReportConfig during configuration (e.g., in step 3). This default setting helps ensure that the UE has a clear starting point for reporting in scenarios where multiple options exist, simplifying the decision-making process.

Proposal 10: The network may provide a default or preferred configuration in functionality configuration for e.g., a default or preferred CSI-ReportConfig either in step 3 or 5 respectively.

As per RAN 1 agreements [3], In step 4, the UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters. It is FFS on whether/what other information along with the applicability is needed. Thus, in step 4 the UE may also indicate to the network its preferred configuration. The network may consider UE’s preference when deciding which configurations/functionality to activate or deactivate.

Proposal 11: The UE may indicate preferred configuration (s) to the network (e.g., preferred CSI-ReportConfig) in step 4.

# Conclusions

Based on the discussion above, we have the following observations and proposals:

Observation 1: Lack of feedback regarding the cause of inapplicability from the UE can lead to significant inefficiencies in signaling, resource usage and allocation, increased latency, and higher power consumption at the UE-side.

Observation 2: Applicable functionalities are dynamic in nature and may fluctuate due to variations in the UE's environment, network-side additional conditions, UE-side additional conditions, or the availability of the AI/ML model.

Observation 3: Frequent functionality applicability changes or updates could negatively affect signaling overhead, system performance, and the network’s ability to manage AIML functionality effectively.

Proposal 1: RAN 2 to support both Option A and Option B for applicability reporting.

Proposal 2: RAN 2 to support configuring Option A for applicability reporting in CSI-ReportConfig.

Proposal 3: RAN 2 to support configuring Option B also using CSI-ReportConfig. format or in CSI-MeasConfig.

Proposal 4: RAN2 to agree that cause for inapplicability can be reported by the UE.

Proposal 5: RAN2 to discuss signaling procedure to report the cause for inapplicability from the UE to the network.

Proposal 6: The network may request the cause for inapplicability from the UE when needed.

Proposal 7: RAN2 should explore methods to minimize the necessity for frequent applicability status updates facilitating the stability of inference operation once activated.

Proposal 8: The UE could provide a “applicable functionality stability index” indication for each applicable functionality, among other potential approaches, to enhance operational consistency and reduce signaling demands.

Proposal 9: RAN2 to discuss and define UE behaviour when an applicable periodic configuration/functionality (CSI-ReportConfig) becomes inapplicable.

Proposal 10: The network may provide a default or preferred configuration in functionality configuration for e.g., a default or preferred CSI-ReportConfig either in step 3 or 5 respectively.

Proposal 11: The UE may indicate preferred configuration (s) to the network (e.g., preferred CSI-ReportConfig) in step 4.

# References

[1] Chair notes, RAN2 129 3GPP TSG-RAN WG2 Meeting #129, Athens, Greece, 2025

[2] R2-2501517, Summary of offline discussion [AT129][026] [AI Phy] Signaling for BM

[3] R2-2406381 Report of [POST126][032] [AI/ML PHY] LCM (Intel/Samsung) Phase 2.