3GPP TSG-RAN WG2 Meeting #129bis	R2-2502668

Wuhan, China, Apr. 7th – 11st, 2025

Agenda item:	8.8.4

Source:	Samsung

Title:	Discussion on Broadcast service area

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

For Rel-19 NTN, one objective is to support MBS broadcast service for smaller areas in an NTN cell. The detailed justification and objective in WID are provided as follows.

MBS feature provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. Terrestrial MBS features are equally available for NR NTN in the 5G specifications, but for some cases the intended service area is expected to be smaller than the coverage of a Uu cell, some enhancements need to be done to notify the service area of a Broadcast service. Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In this contribution, we continue to share our views on the MBS broadcast intended service area, and discuss service area for ETWS.

# Discussion

## MBS intended service area

### UE skipping MCCH acquisition

RAN2 has made several agreements regarding the signaling of MBS intended service area (ISA). As a summary,

- a new SIB, denoted SIBxx, provides a list of intended service areas, an ID is used to identify each MBS intended service area;

- in MCCH, each MBS session is associated to intended service area ID(s). It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH).

For the current serving cell, if UE is within an intended service area indicated in SIBxx, UE acquires MCCH and receives the associated MBS session by receiving MTCH. UE can skip acquiring MCCH if UE is not in any intended areas of MBS broadcast service indicated in SIBxx. Note NW should ensure to indicate the intended service areas of all MBS broadcast sessions of the current cell in SIBxx so that UE can skip MCCH if it is not in any intended service area.

Proposal 1: Intended service areas of all MBS broadcast services of the current serving cell are included in the new SIBxx.

Proposal 2: If UE is not in any intended service areas indicated in SIBxx, UE does not need to acquire MCCH.

In RAN2#128, we have agreed:

A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

One scenario is that the intended service area of an MBS broadcast service covers the whole current serving cell. In this case, for a UE connecting to the current serving cell, it does not need to know the exact geographic area of the current cell; from NW perspective, NW may not be willing to reveal the exact geographic area of the whole cell either. The UE only needs to know there are certain MBS broadcast service intended for the whole cell by reading the new SIBxx, and no matter where the UE locate in the cell it acquires MCCH. Hence, the intended service area does not need to be signaled with the exact geographic area of the whole current serving cell, but can be indicated by a one-bit flag.

Observation 1: In case the intended service area of an MBS broadcast service covers the whole current serving cell, UE does not need to know the exact geographic area covered by the current cell, but only needs to know there are certain MBS broadcast service intended for the whole cell by reading the new SIBxx, and no matter where the UE locates in the cell, the UE acquires MCCH.

Proposal 3: In case the intended service area of an MBS broadcast service covers the whole current serving cell, discuss how to signal in SIBxx.

- Option 1: signal the geographic area of the whole serving cell.

- Option 2: indicate the corresponding intended service area by a one-bit flag.

### Service continuity

For MBS broadcast service, SIB21 contains information related to service continuity for MBS broadcast reception. As specified in stage-2, to avoid the need to read MBS broadcast related system information and potentially MCCH on neighbour frequencies, the UE is made aware of which frequency is providing which MBS broadcast services via PTM, through User Service Description, as defined in TS 26.517, or the combination of User Service Description (USD) and Frequency Selection Area Identities (FSAI) in SIB21. Based on the mapping between frequency and MBS broadcast service, for cell reselection UE can prioritize a frequency if it can only receive this MBS broadcast service(s) by camping on that frequency, or deprioritize a frequency if it cannot receive the MBS broadcast service on that frequency.

Now given ISAs for MBS broadcast services, UE is made aware of which frequency is providing which MBS broadcast services in which geographic area. For cell reselection, UE can take into account the ISA to prioritize or deprioritize a frequency. If UE can only receive an MBS broadcast service(s) by camping on a frequency and UE is in the ISA of that MBS broadcast service, UE may prioritize that frequency; if UE cannot receive an MBS broadcast service on a frequency or if UE not in the ISA of an MBS broadcast service which is provided on a frequency, UE can deprioritize that frequency.

Proposal 4: The new SIBxx includes ISAs of MBS broadcast services that are provided not only on the intra frequency but also on inter frequencies.

Proposal 5: For cell reselection, if UE can only receive an MBS broadcast service(s) by camping on a frequency and UE is in the ISA of that MBS broadcast service, UE may prioritize that frequency.

Proposal 6:  For cell reselection, if UE cannot receive an MBS broadcast service on a frequency or if UE not in the ISA of an MBS broadcast service which is provided on a frequency, UE may deprioritize that frequency.

RAN2 has agreed that both (quasi)-earth-fixed and earth-moving cells are supported for MBS broadcast service. We understand the intended service area represents a fixed geographic area on the ground for both cases. However, due to the fast movement of quasi-earth-fixed or earth-moving cells, the cell may provide an MBS broadcast service to an intended area for a quite short duration, which will lead to frequent updates of MBS broadcast service and the associate intended area and, thus, UE frequent acquisitions of SIB or MCCH. This is different from unicast service which is UE specific and does not need to provide intended service area. To avoid this issue, some additional information (e.g., duration, etc.) can be provided for an MBS broadcast service and its intended area for quasi-earth-fixed or earth-moving cells.

Observation 2: In quasi-earth-fixed or earth-moving cells, the MBS broadcast service to an intended area is for a quite short duration due to satellite movement, which will lead to frequent updates of MBS broadcast service and the associate ISA and, thus, UE frequent acquisitions of SIB or MCCH.

For example, a quasi-earth-fixed or earth-moving cell can signal intended service areas for MBS broadcast services that are to be provided in the future by the upcoming satellite(s) and indicate the time interval during which the service is going to provide. In this way, the intended service areas and the associated MBS broadcast services are announced in advance, and UE only considers to receive a service when the service provision time comes. If NW can plan in advance the MBS broadcast service in quasi-earth-fixed or earth-moving cells (e.g., based on the trajectory and constellation of satellites), this approach can reduce frequent updates/acquisition of the intended service areas and the associated MBS broadcast services in SIB and MCCH.

Proposal 7: For a quasi-earth-fixed or earth-moving cell, NW can provide ISA for MBS broadcast services that are to be provided by the upcoming satellite(s) and indicate the time interval during which the service is going to broadcast.

## ETWS service area

In the last meeting, regarding the warning area of ETWS primary/secondary notification, it is agreed to introduce “warning area coordinates” similar as CMAS and SIB6 for ETWS primary notification is not segmented.  In NR NTN session:

- Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented)

and in IoT NTN session:

- Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB

For CMAS, the field warningAreaCoordinatesSegment, copied as below, is included in the SIB, which is an octet string to indicate Warning Area Coordinates IE defined in and encoded according to TS 23.041.

warningAreaCoordinatesSegment   OCTET STRING                                    OPTIONAL,   -- Need R

warningAreaCoordinatesSegment If present, carries a segment, with one or more octets, of the geographical area where the CMAS warning message is valid as defined in [28]. The first octet of the first warningAreaCoordinatesSegment is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 [29] and so on.

Similarly, for ETWS primary/secondary notification SIB6/7, the same field warningAreaCoordinatesSegment should be used, and there should be no segmentation for SIB6, which can be ensured by NW implementation.

The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits. The size of field warningAreaCoordinatesSegment should fit into the SIB6 message which has up to 2976 bits. According to TS 23.041 and the reference therein [2], warningAreaCoordinate IE has an upper limit of 1022 octets. Currently, SIB6 has 34 bits, so it still can include up to 367 octets, which is sufficient to signal a warning area to some accuracy extend. If a more accuracy warning area is needed, the ETWS secondary notification SIB7 can provide a warning area with segments.

Observation 3: ETWS primary notification SIB6 without segmentation can still include up to 367 octets for warning area coordinates, which is sufficient to signal a warning area to some accuracy extend. If needed, a more accurate warning area can be further signaled in ETWS secondary notification SIB7 with segmentation.

Proposal 8: The field warningAreaCoordinatesSegment is included in SIB6/SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

Proposal 9: The size of the included field warningAreaCoordinatesSegment should be ensured to fit into SIB6 message which has up to 2976 bits.

# Conclusion

In this contribution we discussed the following proposals.

Proposal 1: Intended service areas of all MBS broadcast services of the current serving cell are included in the new SIBxx.

Proposal 2: If UE is not in any intended service areas indicated in SIBxx, UE does not need to acquire MCCH.

Observation 1: In case the intended service area of an MBS broadcast service covers the whole current serving cell, UE does not need to know the exact geographic area covered by the current cell, but only needs to know there are certain MBS broadcast service intended for the whole cell by reading the new SIBxx, and no matter where the UE locates in the cell, the UE acquires MCCH.

Proposal 3: In case the intended service area of an MBS broadcast service covers the whole current serving cell, discuss how to signal in SIBxx.

- Option 1: signal the geographic area of the whole serving cell.

- Option 2: indicate the corresponding intended service area by a one-bit flag.

Proposal 4: The new SIBxx includes ISAs of MBS broadcast services that are provided not only on the intra frequency but also on inter frequencies.

Proposal 5: For cell reselection, if UE can only receive an MBS broadcast service(s) by camping on a frequency and UE is in the ISA of that MBS broadcast service, UE may prioritize that frequency.

Proposal 6:  For cell reselection, if UE cannot receive an MBS broadcast service on a frequency or if UE not in the ISA of an MBS broadcast service which is provided on a frequency, UE may deprioritize that frequency.

Observation 2: In quasi-earth-fixed or earth-moving cells, the MBS broadcast service to an intended area is for a quite short duration due to satellite movement, which will lead to frequent updates of MBS broadcast service and the associate ISA and, thus, UE frequent acquisitions of SIB or MCCH.

Proposal 7: For a quasi-earth-fixed or earth-moving cell, NW can provide ISA for MBS broadcast services that are to be provided by the upcoming satellite(s) and indicate the time interval during which the service is going to broadcast.

Observation 3: ETWS primary notification SIB6 without segmentation can still include up to 367 octets for warning area coordinates, which is sufficient to signal a warning area to some accuracy extend. If needed, a more accurate warning area can be further signaled in ETWS secondary notification SIB7 with segmentation.

Proposal 8: The field warningAreaCoordinatesSegment is included in SIB6/SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

Proposal 9: The size of the included field warningAreaCoordinatesSegment should be ensured to fit into SIB6 message which has up to 2976 bits.

# Reference

RP-240775, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

ATIS-0700041: "WEA 3.0: Device-Based Geo-Fencing".