3GPP TSG-RAN WG2 Meeting #129bis	R2-2502688

Wuhan, China, 7th – 11th April, 2025

Agenda item:	8.9.2

Source:	Samsung

Title:	Further discussion on Store and Forward operation

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#107 the latest WID for IoT NTN Phase 3 was agreed [1], where S&F objective is the following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In this contribution, we provide some discussions related Store and Forward.

# Discussion

- In the last meeting (RAN2#129), the following was agreed:

Agreements:

- RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change

- The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

- Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

- A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

## On satellite IDs for S&F

RAN2 has in this meeting received a reply LS to the LS sent in RAN2#128 [2] on Satellite IDs for Store-and-Forward operation.

RAN2 first asked the following:

- RAN2 understanding: The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

In the SA2 response in [3], SA2 pointed towards clause 4.13.9 in 23.401. The relevant parts are as follows:

--------------------------- 23.401 V19.2.0 ---------------------------

For a UE which indicates support of Store and Forward Satellite operation and when an MME is operating in S&F Mode:

-	If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, the MME shall include a reject cause indicating the NAS rejection is due to Store and Forward Satellite operation.

-	If the UE is rejected with a reject cause indicating it is due to S&F operation, the UE's EMM state shall remain unchanged.

-	The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure.

NOTE 4:	How the MME determines the S&F Wait Timer and S&F Monitoring List is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc.

-	When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause.

NOTE 5:	When the S&F Wait Timer is running, the power consumption optimization behaviours, if any, are left for UE implementation e.g. whether to listen to paging or deactivate its Access Stratum functions.

The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from.

-	The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s).

NOTE 6:	The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode.

NOTE 7:	How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

--------------------------- 23.401 V19.2.0 ---------------------------

- In other words, the UE is not prevented from performing any type of action. Instead the UE can use the list in order to find satellites where it can perform NAS procedures and expect MT data. From the point of view of AS, the important parts of how a UE operates can be found in the Notes 5 and 7. Both Note 5 and 7 state that it is mostly up to UE implementation how to operate when receiving the S&F Wait timer and S&F Monitoring List. Thus we think that the AS layer has to somewhat mimic this behaviour in order to stay consistent.

- In the AS layer, there is already similar behaviour: discontinuous coverage. In discontinuous coverage, the UE is provided with a set of TLE Ephemeris elements and other assistance information that describe how a satellite and its cells move. This is then used by the UE to determine when to wake up in order to camp or to perform MO or MT access. When the UE has determined that there is no coverage, the UE may not perform the AS procedures related to NTN.

- Thus we believe that the S&F Monitoring List signalled over NAS shall be used by the AS layer to determine when to wake up for NAS and MT access, i.e for the UE to perform discontinuous coverage. As an example, the MME may provide an S&F Monitoring List indicating satellite IDs 3, 4, 7, 10. In the current cell that the UE is connected to, the cell may broadcast SIB32 where the satellite IDs 1, 3, 4 and 6 are indicated. Then the UE may perform discontinuous coverage to wake up to camp on satellite with satellite ID 3 and 4, but may also wake up for satellite with satellite ID 1 and 6, for instance to receive Long Term Ephemeris for more satellites. An example of this can be seen in Figure 1. In line with this, we propose:

Proposal 1: In accordance with SA2 23.401, clause 4.13.9, the monitoring list is used in AS for UE to perform discontinuous coverage operation on the satellites indicated in the monitoring list.

Figure 1. Using the S&F Monitoring list with discontinuous coverage.

- For the second question:

Question: If a satellite is included in the satellite list signalled by the MME to a UE, does it mean one of the below options, or possibly other option(s):

- the satellite has the UE context but does not necessarily support store-and-forward operation; or

- the satellite has the UE context and must support store-and-forward operation (but may be operating in normal IoT NTN mode or in store-and-forward mode when the satellite serves the area where the UE is located)?

- For this, SA2 indicated the second option. We believe that for the second question, this may not potentially have any impact on RAN2.

## Timing information on S&F

- In the last meeting it was agreed to introduce a broadcasted indication of when the current satellite operation mode will transit from “S&F operation” to normal mode. There were several FFSes left in the agreements since then. The main motivation of the timing indication is that NAS procedures can be delayed. Since whether NAS will delay its procedures or not would be decided and specified by CT1, we think that RAN2 needs to send an LS to CT1 to notify them of our agreement and reply if they have any concerns.

Proposal 2: Send LS to CT1 regarding RAN2 decision on introducing timing information to delay NAS procedures.

- It has also been discussed whether a network should signal the opposite direction, i.e that S&F operation will start, but for this case the use of the indication is not clear. Since NAS normally does not delay any operation, then such an indication would for instance not be able to “speed up” any NAS operation to conclude before S&F operation begins. Therefore we do not see a use for the indication.

Proposal 3: RAN2 does not introduce an indication for when the S&F operation will start.

## Uplink buffer full indication

- In addition to the barring operation, there has been discussions on a scenario where for instance uplink storage of a satellite becomes full, but the S&F cell may still have downlink data to deliver to UEs. Thus some companies state that they may want to limit access for UEs with uplink data, but not for other UEs. However, this can already be achieved via network implementation. For instance, a network may reject the RRC connection setup for UEs with specific establishment causes, such as mo-Signalling or mo-Data when the uplink buffer of an eNB is full in S&F mode. Similarly, the network may allow establishment causes such as mt-Access even if the uplink buffer is full.

Observation 1: In case of full uplink buffer for S&F cell, the eNB can reject all connection attempts with establishment causes which indicates MO-data by network implementation.

Proposal 4: Network implementation can take care of the case where the uplink buffer is full in S&F scenario.

## New release cause

- In the last meeting there were discussions regarding a new release cause. The supposed use of this release cause is to release a UE because the feeder link becomes unavailable. It is not exactly clear what the UE behaviour should be when receiving such an indication. Such a release cause would only be supported by a Release 19 S&F-capable UE, and in this case the UE should be capable of operating in an S&F network. So it is not clear why the UE needs to be released and why a specific indication is needed for it. Therefore we do not see the need for a new release cause, unless SA2/RAN3 indicates the need.

Proposal 5: RAN2 does not introduced a new release cause when the network wants to release a Release 19 UE to RRC idle because the feeder link becomes unavailable, unless SA2/RAN3 indicates the need.

# Conclusion

In this contribution, we discussed issues related to S&F network:

Observation 1: In case of full uplink buffer for S&F cell, the eNB can reject all connection attempts with establishment causes which indicates MO-data by network implementation.

Proposal 1: In accordance with SA2 23.401, clause 4.13.9, the monitoring list is used in AS for UE to perform discontinuous coverage operation on the satellites indicated in the monitoring list.

Proposal 2: Send LS to CT1 regarding RAN2 decision on introducing timing information to delay NAS procedures.

Proposal 3: RAN2 does not introduce an indication for when the S&F operation will start.

Proposal 4: Network implementation can take care of the case where the uplink buffer is full in S&F scenario.

Proposal 5: RAN2 does not introduced a new release cause when the network wants to release a Release 19 UE to RRC idle because the feeder link becomes unavailable, unless SA2/RAN3 indicates the need.

# Reference

RP-250472, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#107, Incheon, ROK, March 2025.

R2-2411197, LS on Satellite IDs for store-and-forward operation, RAN2, RAN2#128, Orlando, USA, November 2024.

R2-2501753, Reply LS on Satellite IDs for store-and-forward operation, SA2, RAN2#129bis, Wuhan, China, April 2025.