3GPP TSG-RAN WG2 Meeting #129bis	R2-2502690

Wuhan, China, 7th – 11th April, 2025

Agenda item:	8.8.6

Source:	Samsung

Title:	Remaining issues on E-UTRAN to NR NTN mobility

WID/SID:	LTE_TN_NR_NTN_mob-Core

Document for:	Discussion and Decision

# Introduction

In RAN#103 the LTE-part of the WID NR NTN Phase in Rel-19 was agreed [1]:

The work item aims at specifying support for the following objectives: - idle mode mobility based on cell reselection from E-UTRA TN to NR NTN, where E-UTRA TN provides satellite information for NR NTN neighbor cells in a System Information Block. [RAN2]

In this contribution, we provide our views on the last remaining issues on E-UTRAN to NR NTN mobility.

# Discussion

## Redirection

In the last meeting the following was agreed [2]:

Agreements:

- RAN2 confirms that redirection from LTE TN to NR NTN is not supported before Rel-19.

- Redirection from LTE TN to NR NTN is supported in Rel-19 introducing a new capability signaling for LTE TN to NR NTN redirection purpose and also a satellite ID(s) in RedirectedCarrierInfo.

This was further implemented in [3] in the following manner:

RRCConnectionRelease-v1650-IEs ::=	SEQUENCE {

mpsPriorityIndication-r16				ENUMERATED {true}	OPTIONAL, -- Cond Redirection2

nonCriticalExtension					RRCConnectionRelease-v19xy-IEs		OPTIONAL

}

RRCConnectionRelease-v19xy-IEs ::=	SEQUENCE {

ntn-RedirectedCarrierInfoNR-r19		NTN-RedirectedCarrierInfoNR-r19	OPTIONAL,	-- Need ON

nonCriticalExtension				SEQUENCE {}		OPTIONAL

}

NTN-RedirectedCarrierInfoNR-r19 ::=	SEQUENCE {

satAssistanceInfoList-r19			SEQUENCE (SIZE(1..maxSat-r17)) OF SatelliteId-r18	OPTIONAL,	-- Need ON

redirectedCarrierInfo-r19			CarrierInfoNR-r15

}

ReleaseCause ::=				ENUMERATED {loadBalancingTAUrequired,

other, cs-FallbackHighPriority-v1020, rrc-Suspend-v1320}

RedirectedCarrierInfo ::=			CHOICE {

eutra								ARFCN-ValueEUTRA,

geran								CarrierFreqsGERAN,

utra-FDD							ARFCN-ValueUTRA,

utra-TDD							ARFCN-ValueUTRA,

cdma2000-HRPD						CarrierFreqCDMA2000,

cdma2000-1xRTT						CarrierFreqCDMA2000,

...,

utra-TDD-r10						CarrierFreqListUTRA-TDD-r10,

nr-r15								CarrierInfoNR-r15,

nr-r17								CarrierInfoNR-r17

}

One of the issues of implementing it in this manner is that there may be need to be clarifications on several features. Some of legacy features and enhancements related to redirection to NR are:

Enhanced security for redirection to NR. With Release 15, security enhancement was introduced whereby a UE must be configured with AS security in order to be redirected to NR. This can be seen below:

1>	if AS security has not been activated:

2>	ignore the content of redirectedCarrierInfo, if included and indicating redirection to nr;

2>	ignore the content of idleModeMobilityControlInfo, if included and including freqPriorityListNR;

2>	ignore the altFreqPriorities and T323, if included;

2>	if the UE ignores the content of redirectedCarrierInfo or of idleModeMobilityControlInfo, or of altFreqPriorities and T323:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other', upon which the procedure ends;

MPS redirection. This was introduced in Rel-16, where a UE can be redirected to another frequency or RAT, where the UE can access the selected cell with a high priority establishment cause. MPS redirection to NR is supported, where the UE can establish using the MPS priority establishment cause. The MPS redirection uses the redirectedCarrierInfo information element.

mpsPriorityIndication Indicates the UE can set the establishment cause to highPriorityAccess for a new connection following a redirect to E-UTRA or set the resume cause to highPriorityAccess for a resume following a redirect to E-UTRA. If the target RAT is NR, see TS 38.331 [82]. The eNB/ng-eNB sets the indication only for UEs authorized to receive MPS treatment as indicated by ARP and/or QoS characteristics at the eNB/ng-eNB, and it is applicable only for this instance of release with redirection to carrier/RAT included in the redirectedCarrierInfo field in the RRCConnectionRelease message.

Therefore we propose for it to be implemented as a new element in the CHOICE structure of the IE RedirectedCarrierInfo. This allows all of the existing procedure to function well with previously introduced features.

RRCConnectionRelease ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

rrcConnectionRelease-r8				RRCConnectionRelease-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionRelease-r8-IEs ::=		SEQUENCE {

releaseCause						ReleaseCause,

redirectedCarrierInfo				RedirectedCarrierInfo				OPTIONAL,	-- Need ON

idleModeMobilityControlInfo			IdleModeMobilityControlInfo			OPTIONAL,	-- Need OP

nonCriticalExtension				RRCConnectionRelease-v890-IEs		OPTIONAL

}

RRCConnectionRelease-v890-IEs ::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING RRCConnectionRelease-v9e0-IEs)	OPTIONAL,

nonCriticalExtension				RRCConnectionRelease-v920-IEs		OPTIONAL

}

. . .

RedirectedCarrierInfo ::=			CHOICE {

eutra								ARFCN-ValueEUTRA,

geran								CarrierFreqsGERAN,

utra-FDD							ARFCN-ValueUTRA,

utra-TDD							ARFCN-ValueUTRA,

cdma2000-HRPD						CarrierFreqCDMA2000,

cdma2000-1xRTT						CarrierFreqCDMA2000,

...,

utra-TDD-r10						CarrierFreqListUTRA-TDD-r10,

nr-r15								CarrierInfoNR-r15,

nr-r17								CarrierInfoNR-r17,

nr-r19								CarrierInfoNTN-r19

}

RedirectedCarrierInfo-v9e0 ::=			SEQUENCE {

eutra-v9e0								ARFCN-ValueEUTRA-v9e0

}

. . .

CarrierInfoNR-r15	::= SEQUENCE {

carrierFreq-r15					ARFCN-ValueNR-r15,

subcarrierSpacingSSB-r15			ENUMERATED {kHz15, kHz30, kHz120, kHz240},

smtc-r15							MTC-SSB-NR-r15				OPTIONAL		-- Need OP

}

CarrierInfoNR-r17	::= SEQUENCE {

carrierFreq-r17					ARFCN-ValueNR-r15,

subcarrierSpacingSSB-r17			ENUMERATED {kHz15, kHz30, kHz120, kHz240, kHz480, spare1},

smtc-r17							MTC-SSB-NR-r15				OPTIONAL		-- Need OP

}

CarrierInfoNTN-r19	::= SEQUENCE {

carrierFreq-r19						ARFCN-ValueNR-r15,

subcarrierSpacingSSB-r19			ENUMERATED {kHz15, kHz30, kHz120, kHz240, kHz480, spare1},

smtc-r19							MTC-SSB-NR-r15				OPTIONAL,		-- Need OP

satAssistanceInfoList-r19			SEQUENCE (SIZE(1..maxSat-r17)) OF SatelliteId-r18

}

. . .

-- ASN1STOP

Therefore we propose that this RRC structure is pursued and that RAN2 confirms that legacy features are applicable to redirection to NR NTN.

Proposal 1: The redirection to NR NTN information element is enclosed in an NTN specific CarrierInfoNR, which is introduced in the choice structure as nr-19.

Proposal 2: RAN2 confirms that the following aspects apply to redirection to NR NTN (without specification impact):

- AS security must be activated to redirect to NR

- MPS redirection to NR NTN

# Conclusion

In this contribution we discussed issues related to E-UTRAN TN to NR NTN.

Proposal 1: The redirection to NR NTN information element is enclosed in an NTN specific CarrierInfoNR, which is introduced in the choice structure as nr-19.

Proposal 2: RAN2 confirms that the following aspects apply to redirection to NR NTN (without specification impact):

- AS security must be activated to redirect to NR

- MPS redirection to NR NTN

# Reference

RP-240846, New WID: Inter RAT mobility support from E-UTRAN TN to NR-NTN, Thales, CATT, RAN#103, Maastrict, Netherlands, March 2024

R2-2501333, Report from session on NR NTN and IoT NTN, Session chair (ZTE), RAN2#129, Athens, Greece, February 2025.

R2-2501418, Introduction of LTE TN or NR NTN IDLE mode mobility, CATT, RAN2#129, Athens, Greece, February 2025.