3GPP TSG RAN WG2 Meeting #129bis	R2-2502700

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.8.4

Source:	CMCC

Title:	Considerations on broadcast service for NR NTN

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of broadcast service and some agreements were achieved as follows [1]:

Agreements:

- In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list

- It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH)

- Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented)

In this contribution, we would like to provide some further considerations on the remaining issues for broadcast service in NTN system.

Discussion

UE behavior on MCCH (re)acquisition

- Per the current TS 38.331, it is up to UE implementation to use the cell/tracking area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service. Similarly, for MCCH (re)acquisition in NTN, it could be also up to UE implementation. Based on the ISA information included in the new SIB and UE its own location information, UE is able to determine whether it is in a service area that can provide the broadcast service that the UE is interested in. If UE is in or close to any ISA with interested broadcast service, it could monitor the MCCH, otherwise skip corresponding MCCH (re)acquisition.

- Observation 1: Based on the ISA information included in the new SIB and UE its own location information, UE is able to determine whether it is within a service area that can provide the broadcast service the UE is interested in.

- Proposal 1: It could be up to UE implementation to monitor or skip MCCH (re)acquisition based on the ISA included in the new SIB and its own location information.

Service continuity

In RAN2 #127bis meeting, we have discussed on service continuity as following:

Proposal 4: Service continuity can be enhanced in two places by considering intended service area. • In MBSBroadcastConfiguration, the intended service area within each neighbor cell is provided for each MBS broadcast service session. • In SIB21, the intended service area is provided for each MBS broadcast service session, or for each FSAI (depending on common understanding selected). - Ericsson thinks we should focus on the scenario where the quasi-Earth-fixed cell is replaced due to satellite movement - ZTE thinks we currently support both and then supports p4 - QC also thinks both scenarios should be supported. - HW thinks this covers the case of UE movement. For the satellite movement there would be no difference wrt previous releases. HW is ok to reflect the second part of p4 - Ericsson disagrees with the proposal as there is no need for service continuity for this. Apple thinks this is needed for cell reselection. - CATT wonders whether we really need to provide the service area for service continuity, in a per neighbour cell way. - Ericsson thinks we need to clarify the UE behavior when receiving this information Come back in the next meeting

From our perspective, providing service area information for each MBS broadcast session of neighbour cells has some benefit for reselection and handover, it could help UE to select a more suitable cell to continue receiving the same broadcast services. As known, MCCH may also contain a list of neighbour cells providing the same broadcast MBS service(s) as provided in the current cell. In order to guarantee service continuity better, providing service area information per neighbour cell in the MBSBroadcastConfiguration could help UE to reselect or handover to a neighbour cell including service areas association with the MBS session UE interested in or providing same broadcast service which UE is receiving currently.

Then SIB21 provides information related to service continuity, which contains the mapping between the current and/or neighbouring carrier frequencies and MBS Frequency Selection Area Identities (FSAI). However, considering the different understanding about the relation between FSA and MBS session (i.e. one to one, one to multiple), to enhance service continuity, we could just consider to introduce intended service area for each MBS broadcast service session in SIB21.

Proposal 2: It is proposed to provide intended service area within each neighbor cell for each MBS broadcast service session in MBSBroadcastConfiguration.

Proposal 3: It is proposed to provide intended service area for each MBS broadcast service session in SIB21.

MBS interest indication

We discussed that whether a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area without convergence.

As current specification, the MBS interest indication procedure is to inform the network that the UE in RRC_CONNECTED is receiving or is interested to receive MBS broadcast service(s) and/or to inform the network about the priority of MBS broadcast versus unicast and multicast MRB reception. MBS Interest Indication can only be sent after AS security activation. And the trigger condition is described in section 5.9.4.2 of TS 38.331.

- 5.9.4.2 Initiation An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell. If the UE does not have the CFR information and subcarrier spacing for MBS broadcast reception on a non-serving cell at the time it sends the MBS Interest Indication, the UE sends another MBS Interest Indication after it has acquired this information from the non-serving cell.

In the contribution [2], it is mentioned that initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area could help NW to choose a suitable cell providing the broadcast service for this UE. However, per the exiting mechanism, UE already supports to initiate the MII procedure upon handover as highlighted above. And in NTN, UE is able to determine whether in the intended service area based its own GNSS and service area information provided by network. Further, network may have no acknowledge of the UE location, i.e., UE is in which service area may not be clear at network side. Then whether initiate the MII procedure could be just up to UE itself. Therefore, further enhancement for MBS Interest Indication initiation is not needed.

Observation 2: UE is already able to initiate the MBS Interest Indication procedure in several cases as in existing specification mentioned above.

Proposal 4: For the MBS Interest Indication initiation, we could just follow the legacy mechanism, further enhancement is not needed.

ETWS

- For the signaling details of “warning area coordinates”, since we agreed not to segment SIB6 containing an ETWS primary notification, that is to say, for SIB6, we need to include a new IE(e.g. warningAreaCoordinates) to provide a full warning area coordinates information. Then for SIB7, we could just reuse warningAreaCoordinatesSegment in SIB8 directly.

- Proposal 5: For SIB6, introduce a new IE(e.g. warningAreaCoordinates) to provide a full warning area coordinates information.

Proposal 6: For SIB7, reuse warningAreaCoordinatesSegment in SIB8 directly.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on MBS broadcast enhancements for NTN and have the following observations and proposals:

- Observation 1: Based on the ISA information included in the new SIB and UE its own location information, UE is able to determine whether it is within a service area that can provide the broadcast service the UE is interested in.

- Proposal 1: It could be up to UE implementation to monitor or skip MCCH (re)acquisition based on the ISA included in the new SIB and its own location information.

Proposal 2: It is proposed to provide intended service area within each neighbor cell for each MBS broadcast service session in MBSBroadcastConfiguration.

Proposal 3: It is proposed to provide intended service area for each MBS broadcast service session in SIB21.

Observation 2: UE is already able to initiate the MBS Interest Indication procedure in several cases as in existing specification mentioned above.

Proposal 4: For the MBS Interest Indication initiation, we could just follow the legacy mechanism, further enhancement is not needed.

- Proposal 5: For SIB6, introduce a new IE(e.g. warningAreaCoordinates) to provide a full warning area coordinates information.

Proposal 6: For SIB7, reuse warningAreaCoordinatesSegment in SIB8 directly.

Reference

- R2-129 NR-NTN-IoT-NTN (Sergio)_ EOM

- R2-2408016	Further Discussion on MBS Broadcast Provision in NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core