3GPP TSG-RAN WG2 Meeting #129bis                           	                                R2-2502710

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.7.5

Source: 	CMCC

Title:  	Discussion on the left issue of RLC enhancements

WID/SID:	          NR_XR_Ph3-Core

Document for:	Discussion and Decision

Introduction

In the last RAN2 #129 meeting, the RLC enhancements has been discussed and the following related agreements have been approved.

Autonomous retransmissions and polling enhancements: - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

In this contribution, we provide our views on the RLC enhancements.

Discussion

Timely RLC Retransmission

Autonomous Retransmission

In the last meeting, one FFS left for discussion is that if remaining time is determined based on discardTimer at PDCP or new timer at RLC.

In the autonomous retransmission mechanism, the transmitter could retransmit the PDUs without waiting for the NACKs indicated in STATUS report from receiving side. Indeed the autonomous retransmission may fasten the retransmission procedure by reducing the report waiting time. It is agreed that the PDUs with short remaining time could be selected to be retransmitted autonomously. If the remaining time based on the discardTimer from PDCP is below a threshold, RLC should be indicated by PDCP to trigger the autonomous transmission which may need the interaction between these two layer. While it is more flexible for RLC to introduce a new timer associated with a particular RLC SDU to trigger the autonomous retransmission. The new timer could start when the RLC SDU is received from PDCP or the RLC PDU containing the RLC SDU or any segment of RLC SDU is submitted to lower layer for the first time.

Observation 1: If the remaining time based on the discardTimer from PDCP is below a threshold, RLC should be indicated by PDCP to trigger the autonomous transmission which may need the interaction between these two layer.

Proposal 1: Introduce a new RLC timer associated with a RLC SDU to trigger the autonomous retransmission. The new timer could start when the RLC SDU is received from PDCP or the RLC PDU containing the RLC SDU or any segment of RLC SDU is submitted to lower layer for the first time.

In the last meeting, it is agreed that only a single autonomous retransmission will be triggered per RLC SDU.  Therefore one shot autonomous retransmission is considered as the last attempt to retransmit the RLC SDU. The round time for the next RLC SDU retransmission and its status report after one shot autonomous retransmission may exceed the PDB of this SDU. After one shot autonomous retransmission is submitted to lower layer, the RLC may receive the discard indication for this RLC SDU from PDCP or the ACK/NACK for single autonomous retransmission in the STATUS reporting. If the RLC receives the NACK for single autonomous retransmission in the STATUS reporting, the retranmsission couter RETX_COUNT could be incremented by one. If the RETX_COUNT reaches maxRetxThreshold and RLC indicates to upper layers that max retransmission has been reached as legacy.

For the capacity consideration, it is suggested to use a new timer illustrated in Figure 1 in the transmitting side of an AM RLC entity for autonomours retransmission. A timer for RLC SDU could be introduced to avoid excessive retransmissions and STATUS reporting for pervious retransmission before the autonomous retransmission. It could start after the one shot autonomous retransmission. When the timer is running, no retransmission is allowed and NACKed in STATUS reporting is considered as for pervious retransmission.

Figure 1 new timer for single autonomours retransmission

Proposal 2: If the RLC receives the NACK for the single autonomous retransmission in the STATUS reporting, the retranmsission couter RETX_COUNT could be incremented by one. If the RETX_COUNT reaches maxRetxThreshold and RLC indicates to upper layers that max retransmission has been reached as legacy.

Proposal 3: A timer for RLC SDU could be introduced to avoid excessive retransmissions and STATUS reporting for pervious retransmission before the autonomous retransmission. It could start after the one shot autonomous retransmission. When the timer is running, no retransmission is allowed and NACKed in STATUS reporting is considered as for pervious retransmission.

Polling Enhancement

In order to reduce the polling delay for the RLC SDUs with small remaining time value, the remaining time could be considered in the polling trigger mechanism. Indeed, when the remaining time based on the PDCP timer discardTimer is below a threshold for a PDU, which means this PDU turns to be a delay-critical data, the polling may be triggered.

Proposal 4: When the remaining time based on the PDCP timer discardTimer is below a threshold for a RLC SDU, which means this SDU turns to be a delay-critical data, the polling may be triggered.

Several solutions for polling enhancement was proposed in previous meetings.

One solution is to include the polling bit in the PDU containing delay-critical data. But this solution may result lots of pollings or STATUS reports if consecutive delay-critical data come when the congestion occurs. The network could use t-PollRetransmit timer to control the frequency of sending the pollings. The value below 5ms for the timer t-PollRetransmit should be added to reduce the delay of retransmitting the polling. The additional values could be introduced for short PBD of XR traffic, including [0,1,2,3,4] in milliseconds value.

Proposal 5: The additional values of the timer t-PollRetransmit could be introduced for reducing the delay of retransmitting the polling, including [0,1,2,3,4] in milliseconds value.

Another solution is to configure a dedicated value pollPDU or pollByte for delay-critical data in the RLC entity. Two options listed below could be considered to apply a new dedicated value pollPDU or pollByte for delay-critical data.

- Option 1: If the any PDU containing the delay-critical data is found, and when the total number of PDUs or the total data volume of PDUs is accumulated larger than a new dedicated value pollPDU or  pollByte, the polling could be included in the PDU to request the status report from receiving side.

- Option 2: When the total number of PDUs or the total data volume of PDUs containing the delay-critical data is accumulated larger than a new dedicated value pollPDU or  pollByte, the polling could be included in the PDU to request the status report from receiving side.

- Compared to option1, this option 2 could fasten the polling trigger procedure by additional polling for delay-critical data and not impact the legacy polling procedure.

- Proposal 6: The following mechanism could be considered to trigger the polling enhancement:

- Option 1:  If the any PDU containing the delay-critical data is found, and when the total number of PDUs or the total data volume of PDUs is accumulated larger than a new dedicated value pollPDU or  pollByte, the polling could be included in the PDU to request the status report from receiving side.

- Option 2: When the total number of PDUs or the total data volume of PDUs containing the delay-critical data is accumulated larger than a new dedicated value pollPDU or  pollByte, the polling could be included in the PDU to request the status report from receiving side. The legacy polling procedure is not impacted.

Avoidance of Unnecessary Retransmission

In the last RAN2 meeting,  the details of combined approach have not reach a consensus in last meeting and still need to be discussed.

The RLC failure is indicated from RLC to upper layer when max retransmission has been reached, which may lead Radio Link Failure (RLF) and bring the interruption. If the RLC AM entity discards an obsolete RLC SDU and the maximum number of retransmissions is reached, the indication from RLC to upper layer, which indicates max retransmission has been reached as RLC failure, shall not be triggered due to the retransmissions discarding.

Proposal 7: The indication for RLC failure from RLC to upper layer shall not be triggered due to the retransmissions discarding.

Conclusion

In this contribution, we discussed the RLC enhancements related issues. We propose:

Observation 1: If the remaining time based on the discardTimer from PDCP is below a threshold, RLC should be indicated by PDCP to trigger the autonomous transmission which may need the interaction between these two layer.

Proposal 1: Introduce a new RLC timer associated with a RLC SDU to trigger the autonomous retransmission. The new timer could start when the RLC SDU is received from PDCP or the RLC PDU containing the RLC SDU or any segment of RLC SDU is submitted to lower layer for the first time.

Proposal 2: If the RLC receives the NACK for the single autonomous retransmission in the STATUS reporting, the retranmsission couter RETX_COUNT could be incremented by one. If the RETX_COUNT reaches maxRetxThreshold and RLC indicates to upper layers that max retransmission has been reached as legacy.

Proposal 3: A timer for RLC SDU could be introduced to avoid excessive retransmissions and STATUS reporting for pervious retransmission before the autonomous retransmission. It could start after the one shot autonomous retransmission. When the timer is running, no retransmission is allowed and NACKed in STATUS reporting is considered as for pervious retransmission.

Proposal 4: When the remaining time based on the PDCP timer discardTimer is below a threshold for a RLC SDU, which means this SDU turns to be a delay-critical data, the polling may be triggered.

Proposal 5: The additional values of the timer t-PollRetransmit could be introduced for reducing the delay of retransmitting the polling, including [0,1,2,3,4] in milliseconds value.

- Proposal 6: The following mechanism could be considered to trigger the polling enhancement:

- Option 1:  If the any PDU containing the delay-critical data is found, and when the total number of PDUs or the total data volume of PDUs is accumulated larger than a new dedicated value pollPDU or  pollByte, the polling could be included in the PDU to request the status report from receiving side.

- Option 2: When the total number of PDUs or the total data volume of PDUs containing the delay-critical data is accumulated larger than a new dedicated value pollPDU or  pollByte, the polling could be included in the PDU to request the status report from receiving side. The legacy polling procedure is not impacted.

Proposal 7: The indication for RLC failure from RLC to upper layer shall not be triggered due to the retransmissions discarding.

Reference

- 3GPP TS 38.323 V18.4.0 (2024-12), Packet Data Convergence Protocol (PDCP) specification(Release 18)

- 3GPP TS 38.322 V18.2.0 (2024-12), Radio Link Control (RLC) protocol specification (Release 18).