3GPP TSG-WG2 Meeting #129bis			R2-2502719

Wuhan, China, Apr. 7th-11th, 2025

Agenda item:	8.7.4.2

Title:	Further consideration on DSR enhancement for XR

Source:	CMCC

Document for:	Discussion

Introduction

The last meeting [1] discusses some critical issues on DSR enhancement for XR and leaves the following agreements and FFS:

Agreements on DSR enhancements - One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. - FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. - We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. - Different LCGs may be configured with different number of reporting thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. - Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). - Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

This contribution discusses further details on DSR enhancement for XR, especially on data volume calculation.

Discussion

  - Co-existence of legacy and new DSR

The enhancement of DSR introduces a flexible framework to support multiple reporting thresholds per LCG, which is critical for XR traffic requiring differentiated delay-status reporting granularity.

However, ambiguities remain in aligning legacy and new DSR configurations. It was observed that configuring both legacy and new DSR MAC CEs simultaneously introduces unnecessary complexity, as the new DSR MAC CE inherently subsumes the functionality of the legacy format. Furthermore, the use of an extension bit (e.g., redefining the reserved R bit) ensures backward compatibility while efficiently indicating the presence of additional reporting pairs.

As DSR configuration is per LCG, with a proper MAC CE format design, it’s highly possible that LCH configured with one or multiple reporting threshold can be reported in the New DSR MAC CE.

Proposal 1: The New DSR MAC CE should be backward compatible, i.e., supporting LCG configured with one or more reporting threshold(s).

Proposal 2: If P1a is agreeable, the New DSR MAC CE will always be used in case at least one LCG is configured with multiple reporting threshold.

The following is a proposed New DSR MAC CE format where the R field is replaced by the M field which stands for Multiple reporting thresholds. The final terminology can be further discussed.

The proposed refined DSR MAC CE can format support both LCGs configured with one or more reporting threshold(s). For example, M=0 stands for that the corresponding pair of reporting values is a new LCG, M=1 stands for otherwise.

Figure 1: Proposed New DSR MAC CE

The following is an example of description for the introduced M field:

- M: This field is present only if the corresponding LCG is configured with multipleReportingThresholds and the buffer size indicated by the corresponding reporting threshold of the corresponding LCG is not zero; otherwise, this field is reserved and set to 0. If present, the T field set to 1 indicates that the pair of remaining time and buffer size is the first or the only one of the corresponding LCG, while the BT field set to 0 indicates otherwise;

Another threshold related issue is the maximum number of reporting thresholds that can be configured for an LCG. Since the portion with a buffer size of zero no longer needs to be reported in the DSR, the DSR length can be effectively shortened. As a result, the limitation on the maximum number of reporting thresholds can be appropriately relaxed.

Based on the contribution of RAN2-127bis, there are proposals supporting 2, 3 and 4 reporting thresholds per LCG. As kind of a majority view, it is proposed that there will be at most 5 reporting thresholds for each LCG. Although the proposed DSR MAC CE can support a lot of reporting thresholds per LCG, configuring too many thresholds would be inappropriate due to the overhead of the DSR itself. RAN2 can further discuss to reduce the maximum number of reporting threshold per LCG.

Proposal 3: As a start, RAN2 agrees that there should be no more than 5 reporting thresholds per LCG. RAN2 can further discuss whether to reduce the maximum number of reporting thresholds.

From our perspective, 3 reporting threshold is a good compromise.

  - Data volume Calculation for DSR

The last meeting has a discussion up in the air about the data volume calculation for DSR and leaving several options to be discussed:

Option 1. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about the issue of LCH only having the control PDU and retransmitted data at the next meeting. Option 2. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold. Option 3. PDCP considers Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about how RLC considers Control PDU and retransmitted data at the next meeting.

For our perspective, the triggering and shortest configured reporting threshold of the DSR can be regarded as how much data in LCG are sensitive to delay.

The control PDU and data to be retransmitted are important but they should not be equal to delay-critical data. Especially for data to be retransmitted, whether they consist of delay-critical data is uncertain. Therefore, at least they should not be considered to be reported with the shortest configured reporting threshold. UE may be aware of whether the retransmitted data is delay-critical or not, but it would be rather complicated for implementation.

Observation 1a: Data to be retransmitted may consist of non-delay-critical data only.

Observation 1b: UE may be aware of whether data to be retransmitted include delay-critical data or not, but it’s rather complicated for implementation.

As companies has already agreed that the control PDU and data to be retransmitted should be associated with the shortest reporting threshold included in DSR. Reporting them in the shortest reporting threshold is an acceptable approach.

For LCG with LCHs that only have control PDU and data to be retransmitted, no DSR will be triggered, hence there will be no specification impact.

Observation 2: For LCG whose LCHs only have control PDU and data to be retransmitted, no DSR will be triggered.

Proposal 4: When DSR with multiple reporting threshold is triggered, LCH that only have control PDU and/or data to be retransmitted should be considered into the shortest reporting threshold only if the corresponding LCG have delay critical data in other LCH(s).

As an instance, if LCG mixed with LCHs that have delay-critical data and LCHs that only have control PDU and data to be retransmitted, as legacy, control PDU and data to be retransmitted can be considered into the shortest reporting threshold in the DSR. But if all of the LCH(s) in the LCG only have control PDU and/or data to be retransmitted, none of them should be indicated in DSR.

Conclusion

This contribution offers the following observations and proposals:

DSR details:

Proposal 1: The New DSR MAC CE should be backward compatible, i.e., supporting LCG configured with one or more reporting threshold(s).

Proposal 2: If P1a is agreeable, the New DSR MAC CE will always be used in case at least one LCG is configured with multiple reporting threshold.

Proposal 3: As a start, RAN2 agrees that there should be no more than 5 reporting thresholds per LCG. RAN2 can further discuss whether to reduce the maximum number of reporting thresholds.

Data volume calculations:

Observation 1a: Data to be retransmitted may consist of non-delay-critical data only.

Observation 1b: UE may be aware of whether data to be retransmitted include delay-critical data or not, but it’s rather complicated for implementation.

Observation 2: For LCG whose LCHs only have control PDU and data to be retransmitted, no DSR will be triggered.

Proposal 4: When DSR with multiple reporting threshold is triggered, LCH that only have control PDU and/or data to be retransmitted should be considered into the shortest reporting threshold only if the corresponding LCG have delay-critical data in other LCH(s).

References

[1]	R2_129 R18 MBS, R18 QoE and R19 XR session notes (Dawid)_EOM