# 3GPP TSG-RAN WG2 Meeting #129bis	                                                        R2-2502725

# Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.5.3

Source:	CMCC

Title:	Discussion on On-demand SIB1

Document for:	Discussion

Introduction

In RAN#102 meeting, a new WID on enhancements of network energy saving has been approved [1], one of the objectives is on-demand SIB1 for UEs in idle/inactive mode, the scope of this objective has been discussed and revised in RAN#105 [2]. RAN2#129 meeting is the first meeting of the normative phase, following are the related agreements [3]:

Agreements on OD-SIB1

- There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- SIB-x can be cell specific or area specific, as legacy.

- Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

In this contribution, we provide our views on the open issues related OD-SIB1.

Discussion

RRC_CONNECTED UE handling

In last meeting, RAN2 discussed how to handling RRC_CONNETED UE considering OD_SIB1, and reached consensus on RLF case and non-RLF case without searchspaceSIB1 configured as following:

1. Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running: - When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid. - The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition. 2. For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

However, the case that searchSpaceSIB1 is configured but SIB1 is not broadcasting is still FSS:

Proposal 5: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, rely on the NW implementation to deliver SIB1 as baseline. [Vivo]: NW cannot know when the UE wants to request OSI, then proposal 5 cannot work in the case. [Ericsson]: Want to have more time to think of two options. We will make conclusion on P5 next meeting.

Following are descriptions on MIB and SIB1 acquisition in TS 38.331 [4]:

- 5.2.2.3 Acquisition of System Information - 5.2.2.3.1 Acquisition of MIB and SIB1 The UE shall: 1> apply the specified BCCH configuration defined in 9.1.1.1; 1> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 1> if the UE is in RRC_CONNECTED while T311 is running: 2> acquire the MIB, which is scheduled as specified in TS 38.213 [13]; 2> if the UE is unable to acquire the MIB; 3> perform the actions as specified in clause 5.2.2.5; 2> else: 3> perform the actions specified in clause 5.2.2.4.1. 1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or 1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, UE has not acquired SIB1 in current modification period; or 1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1, and, the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, si-BroadcastStatus for the required SIB(s) or posSI-BroadcastStatus for the required posSIB(s) is set to notBroadcasting in acquired SIB1 in current modification period; or 1> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 1> if the UE is in RRC_CONNECTED while T311 is running: 2> if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE: 3> acquire the SIB1, which is scheduled as specified in TS 38.213 [13]; 3> if the UE is unable to acquire the SIB1: 4> perform the actions as specified in clause 5.2.2.5; 3> else: 4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2. 2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> perform the actions as specified in clause 5.2.2.5. NOTE 1: The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 and MBS broadcast if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e., the broadcast and unicast/MBS multicast beams are quasi co-located. The UE in RRC_INACTIVE state while T319a is running, is only required to acquire broadcasted SIB1 and MIB if the UE can acquire them without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located. NOTE 2: UE in RRC_INACTIVE that does not support inactiveStateNTN-r17 enters RRC_IDLE upon cell reselection between TN cell and NTN cell, and initiates the NAS signalling connection recovery (see TS 24.501 [23]).

Currently, for an RRC_CONNECTED UE may acquire SIB1 due to the changing system information, or  acquiring OSI. And the UE may consider the cell is barred based or not on ssb-SubcarrierOffset.When it  comes to OD-SSB case, these requirements still exist, therefore, it should be assured that UE can acquire SIB. Since searchspaceSIB1 is configured, we prefer that UE triggers OD-SIB1 procedure as in RRC_ILE/INACTIVE state, using the WUS configuration acquired from Cell A or NES Cell. Noted that in TS 38.331 that The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 and MBS broadcast if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e., the broadcast and unicast/MBS multicast beams are quasi co-located. Therefore, it’s network implementation on how to deliver SIB1, e.g. via broadcasting or dedicated RRC signalling.

Proposal 1: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting:

- The NES UE triggers OD-SIB1 procedure as in RRC_IDLE/INACTIVE state, based on the stored WUS configuration acquired from Cell A or NES cell;

- It’s network implementation on how to deliver SIB1 to the RRC_CONNECTED NES UE, e.g., via broadcasting or dedicated RRC signalling.

Besides, RAN2 made the following agreement that UE needs to check SIB1 is being broadcasted or not before acquiring OD-SIB1[5]:

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Based on this agreement, RAN1 listed the following options to achieve the purpose for the UE to check SIB1 broadcasted or not in RAN1#120 meeting [6]:

Agreement If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, down select from the following: - Alt. 1: Only if K_SSB=30 for FR1 and K_SSB=14 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission; Otherwise, UE doesn’t monitor Type 0 PDCCH - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified. - Alt. 2: UE monitors Type 0 PDCCH for SIB1 transmission - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified. - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission - Alt. 4: UE doesn’t monitor Type 0 PDCCH for SIB1 transmission. - Alt. 5: UE monitors Type 0 PDCCH for SIB1 based on some repurposed parameters in MIB or PBCH Note: If the UE detects a SSB where K_SSB<=23 for FR1 or K_SSB<=12 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission as legacy. Note: From Ericsson point view, Alt 3 and Alt 4 may be inconsistent with existing RAN2 agreements Note: The above cases are for SSB on the sync raster.

In our opinion, Alt. 3 and Alt. 4 will result in obvious energy waste for both gNB and UE side, and it is also inconsistent with existing RAN2 agreements. Thus, these alternatives should not be selected. Alt.1 and Alt.5 can achieve the similar effect. Moreover, compared Alt. 1/5 with Alt.2, Alt. 1/5 requires one detection on MIB/PBCH while Alt. 2 requires one blind detection on PDCCH. Therefore, we think there’s no need to introduce explicit indication to indicate SIB1 broadcasting status, while UE can perform blind detection on SIB1 to check whether SIB1 is broadcasted or not, e.g., Alt 2 can be supported.

Proposal 2: In case a UE has SIB1 request configuration of a cell and before transmitting UL WUS and detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, the UE can perform blind detection on SIB1 to confirm whether SIB1 is broadcasted or not before triggering OD-SIB1 request, and there’s no need to introduce explicit indication for OD-SIB1 broadcasting status.

WUS configuration

In last meeting, RAN2 agreed to use a new SIB for WUS configuration delivery, but it’s not clear the  maximum number of WUS configurations. Considering that Cell A is more possibility to be deployed on lower frequency for better coverage, while NES cell is deployed on higher frequency as a hotspot. Therefore, it is likely that Cell A and NES cell are on the different carrier frequency. Based on this, the maximum number of configured inter frequency cells can be considered as the maximum  number of WUS configuration, i.e., 16.

Proposal 3: The maximum number of configured inter frequency cells can be considered as the maximum  number of WUS configuration, i.e., 16.

In last meeting, RAN2 agreed that SIB-x can be cell specific or area specific, as legacy. But it is not clear whether WUS configuration is applied to one NES cell or not. Considering the WUS configuration is mainly configuration for Msg1 transmission, therefore, we think it’s difficult to share the same configuration. Therefore, it is proposed that WUS configuration is cell specific.

Proposal 4: WUS configuration is cell specific.

On-demand SIB1 acquisition procedure

Based on the agreements made in RAN2, the procedure for a UE to acquire on-demand SIB1 can be described as following:

- The UE camps on a cell (Cell A) and get NES cells’ WUS configuration provided in the new SIB.

- The UE reselects to a cell using legacy cell reselection criterion and identifies whether it’s a NES cell. If it’s a NES cell, and it has its WUS configuration, the UE will initiate on-demand SIB1 acquisition procedure using the WUS configuration. Else, if the UE doesn’t have the related WUS configuration, it will treat the cell as barred.

One issue should be discussed is the validity of the WUS configuration, since the UE may moves among cells, and not all of the cells are anchor cells and even for an anchor cell it may not broadcast the new SIB to provide WUS configuration of NES cell(s). From our point of view, it’s reasonable that the WUS configuration is semi-static, therefore, the UE can treat the WUS configuration as valid unless the network indicated a system information updating.

Proposal 5: The UE treats the WUS configuration as valid during its mobility unless indicated by the network for system information updating.

Conclusions

In this contribution, we analyse the OD-SIB1 related open issues , following are the proposals and observations:

Proposal 1: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting:

- The NES UE triggers OD-SIB1 procedure as in RRC_IDLE/INACTIVE state, based on the stored WUS configuration acquired from Cell A or NES cell;

- It’s network implementation on how to deliver SIB1 to the RRC_CONNECTED NES UE, e.g., via broadcasting or dedicated RRC signalling.

Proposal 2: In case a UE has SIB1 request configuration of a cell and before transmitting UL WUS and detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, the UE can perform blind detection on SIB1 to confirm whether SIB1 is broadcasted or not before triggering OD-SIB1 request, and there’s no need to introduce explicit indication for OD-SIB1 broadcasting status.

Proposal 3: The maximum number of configured inter frequency cells can be considered as the maximum  number of WUS configuration, i.e., 16.

Proposal 4: WUS configuration is cell specific.

Proposal 5: The UE treats the WUS configuration as valid during its mobility unless indicated by the network for system information updating.

References

- RP-234065, New WID: Enhancements of network energy savings for NR, 3GPP TSG RAN Meeting #102;

- RP-242354, Revised WID: Enhancements of network energy savings for NR, 3GPP TSG RAN Meeting #105;

- Report of 3GPP TSG RAN WG2 meeting #129;

- TS 38.331;

- Report of 3GPP TSG RAN WG2 meeting #127bis;

- Draft Report of 3GPP TSG RAN WG1 #120 v0.3.0	(Athens, Greece, February 17th – 21st, 2025).