3GPP TSG-RAN WG2 Meeting #129bis	R2-2502750

Wuhan, China, 7 – 11 April, 2025

Agenda item:	8.7.6

Source:	Samsung

Title:	Discussion on UL rate control for Rel-19 XR

Document for:	Discussion & Decision

Introduction

In the last RAN2 meeting, we agreed that:

Agreements on XR rate control - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). - Companies to check with their SA4 colleagues whether there are any issues with this

In this paper, we provide our view for the remaining issues.

Discussion

QoS Flow Level Bit Rate Signalling

It is still FFS for RAN2 on which approach should be adopted between 1) MAC CE based approach, and 2) RRC/MAC CE combined approach, to implement QoS flow level signalling.

Since RAN3 has confirmed in their LS reply that both approaches entail impact on the F1 interface, the argument that the combined RRC/MAC CE approach is more compatible with the current CU/DU split architecture is no longer compelling.

Moreover, RRC/MAC CE combined approach introduces more residual issues, e.g., how to let UE know each respective portion of bit rate for each QoS flow if multiple QoS flows are mapped to a common DRB. Also, it is still challenging for RRC/MAC CE approach how the DU can decide target DRBs to be throttled and the corresponding bit rates, respectively, without knowing the QoS flow level characteristics.

Therefore, we can make our preference between the two approaches clear, from a RAN2 perspective.

Proposal 1: RAN2 to confirm that, from RAN2 perspective, the MAC CE based approach is preferable, over RRC/MAC CE combined approach.

QoS Flow Identification

If a new MAC CE is introduced to indicate QoS flow level bit rate signalling, how to identify the QoS flow should be discussed first. For this purpose, considering that QoS Flow ID is unique within a certain PDU session/DRB/Logical Channel, we can consider the following three alternatives for this purpose:

- Alt. 1: PDU session ID + QoS Flow ID;

- Alt. 2: DRB ID + QoS Flow ID;

- Alt. 3: LCID + QoS Flow ID.

Proposal 2: RAN2 to discuss how QoS flow is identified in the new MAC CE design, considering the following alternatives:

- Alt. 1: PDU session ID + QoS Flow ID;

- Alt. 2: DRB ID + QoS Flow ID;

- Alt. 3: LCID + QoS Flow ID.

Rate Query

From the responsibility point of view, it is gNB who is supposed to be responsible for enforcing the QoS requirement for each QoS flow, based on the corresponding QoS profile. While, the application residing in UE can have its own preference, e.g., based on its own optimization policy. In order to let gNB make final decision considering both QoS enforcement and application preference, it is desirable to construct a closed loop system embracing a UL MAC CE for UE rate query.

Proposal 3: RAN2 to confirm the working assumption that rate query MAC CE is supported with the same design as rate indication MAC CE for QoS flow level rate query.

Multi-QoS Flow Extension

Since an XR service normally entails multiple QoS flows, and among the multiple QoS flows, it is likely that more than one flow are supposed to be throttled simultaneously, when congestion happens. With this in mind, we see the benefit by featuring the new MAC CE with multi-QoS flow support, to reduce the signalling overhead (otherwise multiple MAC CEs are needed) and the associated latency.

Proposal 4: The new MAC CE is designed to support the signalling for multiple QoS flows.

Conclusion

Based on the discussion above, we have the following observations and proposals:

Proposal 1: RAN2 to confirm that, from RAN2 perspective, the MAC CE based approach is preferable, over RRC/MAC CE combined approach.

Proposal 2: RAN2 to discuss how QoS flow is identified in the new MAC CE design, considering the following alternatives:

- Alt. 1: PDU session ID + QoS Flow ID;

- Alt. 2: DRB ID + QoS Flow ID;

- Alt. 3: LCID + QoS Flow ID.

Proposal 3: RAN2 to confirm the working assumption that rate query MAC CE is supported with the same design as rate indication MAC CE for QoS flow level rate query.

Proposal 4: The new MAC CE is designed to support the signalling for multiple QoS flows.