3GPP TSG-RAN WG2 Meeting #129bis                                                                      R2-2502759

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.2.2

Source:	China Telecom

Title:	Further discussions on Ambient IoT Paging

Document for:	Discussion

- Introduction

During the RAN#106 meeting, a new work item (RP-243326) focused on Ambient IoT (Internet of Things) within NR was approved. One of the objectives is to specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling mechanisms to facilitate DO-DTT and DT data transfer. RAN2 aims to design a paging message format that accommodates multiple identifiers within a single paging message to ensure forward compatibility.

During the RAN2#129 meeting, the following agreements were made.

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/“filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

- The A-IoT paging message can include a number of msg1 resources

- From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later:

Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

- The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

Based on the above progress, this paper mainly analyses the details of subsequent A-IoT paging and paging contents.

- Discussion

  - Parallel Service from Different Readers

During the last RAN2 meeting, it was concluded that parallel service requests from the same reader can be avoided through network implementation. The device is expected to perform only one procedure at a time and cannot handle multiple parallel service requests. However, restricting the A-IoT reader to send a single service request at a time does not completely avoid parallel service requests from different readers. The scenario in which multiple readers page the same device for the same service request remains an open topic for discussion. To ensure coverage, overlapping coverage areas are unavoidable. Multiple readers may be deployed to serve an area, and their coverage may overlap. Whether the parallel service issue can be avoided through network implementation depend on RAN3.

When CN generates a paging request for inventory, it may send the same request to multiple readers, as it does not have precise information about which reader covers the a-IoT device.  Without TDM coordination among readers, it is quite likely that multiple readers will simultaneously page the device.

In our view, this is primarily a network deployment-related issue and it is up to RAN3 to evaluate whether and how to resolve this issue through network deployment. If BS/CN implementation can avoid the parallel services from the neighbour readers, there are no impact on air interface.

Proposal 1：It is up to RAN3 to evaluate whether and how to resolve parallel service from different readers with network deployment.

Based on the above situation, devices located in overlapping areas covered by different readers may receive R2D messages from different readers simultaneously. or this case, the decision from SA2/RAN3 is required on whether to categorize these paging requests as the same transaction or distinct transactions. If categorized as separate transactions, the allocation/generation of transaction ID among neighboring readers must be determined by RAN3 and/or SA2. Furthermore, whether the device should respond to each request from different readers requires further discussion based on this decision.

Additionally, some companies have proposed that in positioning scenarios, devices must respond to multiple readers. Allowing an A-IoT device to respond to multiple A-IoT paging messages from different readers for the same service request could improve localization accuracy. Currently, according to TR 38.769, the inclusion of multiple Reader IDs by the A-IoT RAN node in the Inventory Report/Command Response message remains an open discussion. At present, other working groups, such as RAN3, have yet to reach a conclusion on this matter. Therefore, it is proposed that RAN2 postpone the discussion on this topic until further progress is made by other working groups.

Proposal 2:  For the A-IoT paging messages initiated by different readers, RAN2 send an LS to RAN3 to ask:

- whether coordination between readers is expected,

- whether or not parallel service from different readers can be avoid by a network-side solution,

- whether or not same service request from different readers shall be regarded as the same transaction or different transactions,

- whether or not multiple Reader IDs can be included by the A-IOT RAN node in the Inventory Report/Command Response message.

  - Transaction ID

RAN2 has agreed that the transaction ID can be generated by the reader based on CN correlation ID. AIoT devices can distinguish different service requests based on the transaction ID, thereby avoiding duplicate responses to paging messages associated with the same service request. However, the details of how to generate the transaction ID and other aspects have not yet been concluded. In our view, the specific details of the transaction ID’s generation will be further discussed and decided by SA2/RAN3.

Proposal 3： Wait for SA2/RAN3 progress before further discussing on how the reader generate transaction ID.

It is assumed that the device does not need to retain multiple transaction IDs simultaneously. However, if the transaction ID is too short, a wrap-around issue may still occur. This means that the transaction ID may be reused too quickly, potentially causing confusion on the device side. Therefore, if the transaction ID is only 2 bits long, the likelihood of such errors is significantly increased. We proposed to design a transaction ID of sufficient length, preferably 3–6 bits, to mitigate this risk.

Proposal 4：To design 3-6 bit “Transaction ID” in AIoT paging message.

  - Paging Round

AIoT devices may operate in the following states and can transition between states depending on their energy harvesting status:

- ON state: transmission, reception, and a clock running

- OFF state: energy harvesting only

- SLEEP sate: energy harvesting, monitoring, maintaining memory, and a rough clock running

The issue on whether Device 1 can support sleep state in addition to the ON and OFF state was still discussed in RAN1. However, it is evident that Device 2b can support the ON, OFF, and SLEEP states.

During RAN1#118 meeting, RAN1 reached the following conclusions on energy harvesting/device unavailability:

Agreement For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection.

Regarding Direction 2, the reader can transmit information in the R2D message, allowing the device to select its availability for energy harvesting. When the device is not required to respond to any access rounds during that paging cycle, it may switch to the OFF or SLEEP state for energy harvesting until the end of that paging round. Limited by capacity, it may be difficult for the device to make predictions about the duration of a paging round based on other information. Therefore, the reader should explicitly include time indication information in the A-IoT paging message, which indicates the duration of this paging round to help the device determine their respective available/unavailable times.

- Proposal 5	To include time indication information in the A-IoT paging message which indicates the duration of each access round in the device's current paging round, so that the device can easily determine their respective available/unavailable times.

  - Multiple/Subsequent A-IoT Paging

In general, the current agreements presume that paging necessitates a response in all cases. This assumption is particularly valid for paging scenarios where the reader must conduct an inventory of one or more devices, retrieve data from devices, or establish communication for data exchange. However, a single round of paging may not always reach all target devices within the coverage area.  This may occur because some devices are insufficiently charged, preventing them from responding to paging requests. Alternatively, a high density of devices may lead to contention for uplink resources when multiple devices attempt to respond simultaneously. Consequently, for each CN service request for paging, the reader can initiate multiple paging messages.

A paging round refers to the duration of a sequence of paging messages triggered by the same CN request. Each paging message may constitute a paging round. To minimize complexity, subsequent paging messages within the same paging round are identical to the initial trigger message. The method for initiating multiple or subsequent A-IoT paging still need further discussion. Three potential implementation options are as follows:

- Upon receiving the CN request, the reader configures the timing for subsequent A-IoT paging and determines the number of paging repetitions.

- The CN service request includes a periodic A-IoT paging configuration (e.g., paging round duration and the number of repetitions). Based on this configuration, the reader schedules subsequent A-IoT paging and defines the number of repetitions.

- The CN triggers each instance of multiple or subsequent A-IoT paging independently, and the reader executes the transmission upon receiving the paging trigger from the upstream node.

Option 1 may not be feasible, as the reader typically lacks information regarding devices within the coverage area. Compared to Option 3, Option 2 may reduce CN requests and associated overhead, as it allows the reader to conduct A-IoT paging at regular intervals. However, according to conclusion from SA2, reader-triggered periodic paging requires a termination request from the CN to halt paging repetitions, and any reconfiguration necessitates an additional CN request. Therefore, to simplify the A-IoT protocol, it is recommended that the CN initiate A-IoT paging on demand rather than configuring periodic paging.

- Proposal 6	It is recommended that the CN initiate A-IoT paging on demand rather than configuring periodic paging.

  - Multi-Devices CFRA Paging Scenario

To initiate the multi-device CFRA procedure, the A-IoT paging message must specify the mapping between each device ID and the corresponding access occasion index, ensuring that each device can accurately determine its designated access occasion. However, explicitly specifying a one-to-one mapping in the paging message introduces significant overhead. An alternative approach involves using a group or mask ID. When multiple A-IoT devices are activated for the CFRA procedure through group or mask IDs, their respective access timing indices are determined by their positions relative to the assigned group or mask ID. However, if the target devices are assigned to distinct groups, individual device IDs must be explicitly included in the A-IoT paging message. A third approach involves using a PF/PO-like mechanism to allocate access timing to each device ID. In this method, an initial access timing is assigned to the target A-IoT device based on a computation of its device ID, which is modelled as a parameter defining the access timing size within a cycle. This strategy confines conflicts to A-IoT devices sharing the same collision outcome. Each device is required to determine its ranking within the group of devices sharing this outcome and adjust its computation accordingly to identify its accurate access timing. Using a PF/PO-like access index assignment method, an A-IoT device only needs to include in its paging message a list of device IDs sharing the same collision outcome and the number of accesses per cycle.

- Proposal 7	 To identify their appropriate access occasions in the A-IoT CFRA procedure, the following potential options need to be further evaluated:

- One-to-one mapping of the device ID to the access occasion

- PF/PO-like access index allocation approach

- Mask ID

- Conclusions

Based on the discussion in the previous sections, we propose the following:

Proposal 1：It is up to RAN3 to evaluate whether and how to resolve parallel service from different readers with network deployment.

Proposal 2:  For the A-IoT paging messages initiated by different readers, RAN2 send an LS to RAN3 to ask:

  - whether coordination between readers is expected,

  - whether or not parallel service from different readers can be avoid by a network-side solution,

  - whether or not same service request from different readers shall be regarded as the same transaction or different transactions,

  - whether or not multiple Reader IDs can be included by the A-IOT RAN node in the Inventory Report/Command Response message.

Proposal 3： Wait for SA2/RAN3 progress before further discussing on how the reader generate transaction ID.

Proposal 4：To design 3-6 bit “Transaction ID” in AIoT paging message.

- Proposal 5	To include time indication information in the A-IoT paging message which indicates the duration of each access round in the device's current paging round, so that the device can easily determine their respective available/unavailable times.

- Proposal 6	It is recommended that the CN initiate A-IoT paging on demand rather than configuring periodic paging.

- Proposal 7	 To identify their appropriate access occasions in the A-IoT CFRA procedure, the following potential options need to be further evaluated:

- One-to-one mapping of the device ID to the access occasion

- PF/PO-like access index allocation approach

- Mask ID

- References

- RP-250796, Revised WID: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile	USA

- R2-25xxxxx, Draft Report of 3GPP TSG RAN WG2 meeting #129,

- TR 23.700-13	Study on Architecture support of Ambient power-enabled Internet of Things