3GPP TSG-RAN WG2 Meeting #129bis	R2-2502774

Wuhan, China, Apr. 7th- 11th, 2025

Agenda item:	8.6.2

Source:	Samsung

Title:	Further Considerations to Support Inter-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

In RAN2#129 meeting, RAN2 reached the following agreements for inter-CU LTM procedure and some details:

- For inter-CU SCG LTM, introducing the SK counter list in LTM-ConfigSCG-r19, each list associated with one ltm-NoSecurityChangeID.

- Reuse the selectedSK-Counter-r18 to indicate the selected SK counter upon the execution of inter-CU SCG LTM.

- The reference configuration related to inter-SN LTM is included within the new ltm-ConfigSCG field.

- UE does not combine reference configurations from ltm-Config and ltm-ConfigSCG.

- FFS whether only one indication is needed to indicate LTM configuration as complete configuration.

- The existing field mrdc-ReleaseAndAdd is always included when SCG configuration is included during an MCG LTM execution. Can be revisited if there is any issue.

- The release of the LTM configuration (MCG or SCG) follows the Rel-18 principles.

- A maximum number of 8 candidate configurations (LTM plus CLTM candidate configurations) can be configured.

- The LTM-Candidate IE is re-used to model an CLTM candidate cell configuration.

- Re-use existing L1 events for CLTM.

- For field ltm3-Offset-r19, the granularity is 0.5 dB.

- Add extension mark “…” to LTM-ConfigSCG-r19, adopt the principle that fields only applicable to inter-CU SCG change should be defined in the root of LTM-ConfigSCG-r19, not LTM-Config-r18.

- Using the term “LTM” to represent “intra-CU LTM” and “inter-CU LTM” in MAC specification, and introduce separate term for “conditional intra-CU LTM” in the specification(s).

- Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

- Same as Rel-18 intra-CU LTM, CFRA resource configuration could be included in LTM cell switch command MAC CE for Rel-19 inter-CU LTM.

- For inter-CU MCG LTM, when Rel-19 ID of candidate cell is different from serving cell, the UE performs PDCP re-establishment for SRB1/SRB2.

- For inter-CU MCG LTM, when the Rel-19 ID of candidate cell is the same with serving cell, the UE performs PDCP SDU discard for SRB1/SRB2.

- How to support shared RACH resources for inter-CU LTM is left to RAN3.

In this contribution, we will further address inter-CU LTM in details.

Discussion (non-DC part)

Issue 1: Security for inter-CU LTM

SA3 provided us the reply LS for security as below:

Reply LS from SA3 (S3-251124) SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

The reply LS indicates that NCC over MAC CE is an acceptable solution for SA3. Although some companies in SA3 express the concern on this compromise, RAN2 can follow SA3 decision to continue the work.

Proposal 1-1: RAN2 is kindly asked to agree that the MAC CE can be used to indicate the NCC for inter-CU LTM.

If the above proposal is not agreeable, RAN2 needn’t to reopen the discussion between RRC-based solution (i.e., UE is provided via RRC signalling with the NCC value after every inter-CU LTM cell switch) and MAC CE based solution since this has been thoroughly discussed in SA3. To make progress, some compromised options can be considered to resolve the concern on NCC in plaintext:

- Option 1: contain NCC index rather than NCC in MAC CE

In this option, the gNB can assign different index to different NCC via RRC message. When the inter-CU LTM cell switch is triggered, the gNB can send the NCC index to the UE so that the UE can derive the real NCC value accordingly. The similar method has been used to indicate the LTM target cell by containing the candidate configuration ID instead of the real cell ID, and the intention of such design is also about the security.

- Option 2: contain the delta value of the in-use NCC in MAC CE

In this option, the gNB can indicate the delta value of new NCC compared to the in-use NCC. Since the in-use NCC is well protected, the attacker cannot derive the new NCC based on the delta value in MAC CE.

Actually, the above option 1 and option 2 can solve the concern on the MAC CE based solution. Meanwhile, the RRC signalling between two cell switches can be avoided.

Proposal 1-2: if NCC in plaintext over MAC CE is not agreeable, RAN2 is kindly asked to discuss the following options and make decisions:

- Option 1: contain NCC index rather than NCC in MAC CE

- Option 2: contain the delta value of the in-use NCC in MAC CE

Regardless of which way to go, RAN2 has to define the new signalling to help UE derive the new NCC for security key update. Such new signalling can be designed by considering the two options below:

- Option 1: the new signalling is used for both vertical security key change and horizontal security key change.

In this option, such new signalling is always needed, and the UE can determine the security key change based on such new signalling. Then, if the UE doesn’t receive the new signalling, the security key update is not needed. However, such design results in that the new Rel-19 ID becomes useless for inter-CU MCG LTM.

- Option 2: the new signalling is used only for the vertical security key change, while the horizontal security key change is realized by new Rel19 ID

This option can save the signalling overhead since only vertical security key change requires the new signalling. For the inter-CU LTM with horizontal security update, the UE can determine it based on ltm-ServingCellNoSecurityChangeID and ltm-NoSecurityChangeID.

Proposal 1-3: RAN2 is kindly asked to determine whether the new Rel19 ID is still used for the determination of security key update during inter-CU MCG LTM.

Issue 2: Failure handling

For failure cases due to the expiry of T304, the following four cases can be considered (as shown in Fig. 1):

- Case 1: intra-CU cell switch failure with selected intra-CU candidate cell w.r.t. the source cell

- Case 2: inter-CU cell switch failure with selected intra-CU candidate cell w.r.t. the target cell

- Case 3: intra-CU cell switch failure with selected inter-CU candidate cell w.r.t. the source cell

- Case 4: inter-CU cell switch failure with selected intra-CU candidate cell w.r.t. the source cell

Fig. 1 failure cases due to the expiry of T304

According to the current specification, for all cases, the UE will revert back to the configuration of source PCell, including the security key, after the cell switch failure. If the selected cell is an intra-CU candidate cell w.r.t. the source PCell (i.e., Case 1 and Case 4), the UE can perform the LTM cell switch procedure without security key change for fast recovery.

Proposal 1-4: After inter-CU/intra-CU LTM cell switch failure (due to the expiry of T304), RAN2 is kindly asked to agree that the UE can perform the fast recovery if the selected LTM candidate cell is the intra-CU cell w.r.t. to the source cell (Case 1 & 4).

For Case 3, the selected cell is an inter-CU candidate cell so that the UE has to perform the security key update. Thus, the fast recovery cannot be carried out, and the UE has to perform the normal RRC reestablishment procedure.

Proposal 1-5: After intra-CU LTM cell switch failure (due to the expiry of T304), RAN2 is kindly asked to agree that the UE performs RRC reestablishment procedure (not fast recovery) if the selected LTM candidate cell is the inter-CU candidate cell w.r.t. to the source cell (Case 3) .

For Case 2, to carry out fast recovery, the UE cannot revert back to the configuration of source PCell after the failure. The operation of “revert back” depends on the selected cell:

- If the selected cell is the intra-CU candidate cell w.r.t. source cell, the UE performs the “revert back” operation, and it can use the security key of the source cell to perform the fast recovery

- Otherwise(as indicated in Case 2), the UE needn’t perform “revert back”, and it can perform LTM cell switch directly at the selected cell.

Proposal 1-6: For inter-CU LTM cell switch failure (due to expiry of T304), RAN2 is kindly asked to discuss whether to support the fast failure recovery if the selected cell is intra-CU LTM candidate cell w.r.t. the target cell (Case 2).

For failure cases due to RLF shortly after LTM cell switch, the following four cases can be considered (as shown in Fig. 2):

- Case 1: RLF after intra-CU cell switch and select an intra-CU candidate cell w.r.t. the target cell

- Case 2: RLF after inter-CU cell switch and select an intra-CU candidate cell w.r.t. the target cell

- Case 3: RLF after intra-CU cell switch and select an inter-CU candidate cell w.r.t. the target cell

- Case 4: RLF after inter-CU cell switch and select an inter-CU candidate cell w.r.t. the target cell

Fig. 2 failure cases due to RLF shortly after LTM cell switch

Among the above cases, the fast recovery can be performed if the selected LTM candidate cell is the intra-CU cell w.r.t. the target cell (Case 1 & 2) since the security key update is not needed. For Case 3&4, the UE has to perform the re-establishment procedure since the security key update is needed.

Proposal 1-7: For RLF after the LTM cell switch, RAN2 is kindly asked to agree that the UE can perform the fast recovery if the selected LTM candidate cell is the intra-CU cell w.r.t. the target cell (Case 1 & 2).

Issue 3: Fast RRC processing

In last RAN2 meeting, the fast RRC processing was discussed for Rel-18 and reached the following agreements.

R2-2500989 Discussion on the fast RRC processing for LTM Ericsson, MediaTek Inc. discussion Rel-18 NR_Mob_enh2-Core Proposal 4: A new field is added within the LTM-Candidate IE to indicate the number of serving cells configured for the LTM candidate configuration. Proposal 5: RAN2 to agree on the CR provided in the Annex. A new field within the LTM-Candidate IE to indicate the number of serving cells configured for the LTM candidate configuration is not needed.

In order to support the early ASN.1 decoding, RAN4’s reply LS (R2-2420218) indicates that the number of serving cells configured for the LTM candidate configuration is beneficial for the UE. However, since this feature was raised lately after Rel18 frozen, companies were not convinced to have it in Rel18. In Rel19, the same issue is applicable, i.e., the UE needs to determine whether to perform the fast ASN.1 decoding.

Proposal 1-8: RAN2 is kindly asked to agree that the number of serving cells configured for each LTM candidate configuration can be notified to the UE.

Moreover, the knowledge on the number of cells configured in each inter-CU LTM candidate configuration is also beneficial for the gNB side. For example, with this information, the source gNB can be used to determine whether the UE performs the early ASN.1 decoding or not for the target cell. Specifically, when the source gNB sends the LTM cell switch command to the UE, if all the configured LTM candidate configurations do not exceed the capability of fast RRC processing, the gNB can assume the UE performs the early ASN.1 decoding, and it should carry out the network operations (e.g., send the cell switch notification to the target gNB) quickly.

Proposal 1-9: RAN2 is kindly asked to agree that it is beneficial to let the source gNB know the number of serving cells configured for each LTM candidate configuration.

Discussion (DC part)

Inter-CU MCG LTM

According to agreements in the past, the following inter-CU MCG LTM cases are supported:

- Inter-CU MCG LTM with intra-SN PSCell change

- Inter-CU MCG LTM with SCG addition

- Inter-CU MCG LTM with SCG reconfiguration

- Inter-CU MCG LTM + intra-CU MCG LTM

- Inter-CU MCG LTM +intra-CU SCG LTM

Issue 1: reference configuration for inter MCG LTM

Similar to no-DC case, the reference configuration including MCG part can be configured to the UE. The question is whether the reference configuration can include SCG part or not. For inter-CU MCG LTM, the SCG configuration can be added/modified, as agreed in the past. Thus, the SCG reference configuration is beneficial to reduce the configuration overhead at the UE side. Moreover, since the SN is unchanged, when preparing the SCG part, the SN can evaluate whether the reference configuration for SCG part can save signalling or not. If it is needed, SN can provide the reference configuration to MN directly (without request from MN), and the MN can configure the reference configuration including both MCG and SCG part.

Proposal 2-1: For inter-CU MCG LTM with unchanged SN or SCG addition, RAN2 is kindly asked to agree the followings:

- The reference configuration can include the SCG part

- During inter-CU MCG LTM preparation, the SN can determine whether to provide the reference configuration of SCG part to the MN or not.

Conclusion

- Non-DC

Security for inter-CU LTM:

Proposal 1-1: RAN2 is kindly asked to agree that the MAC CE can be used to indicate the NCC for inter-CU LTM.

Proposal 1-2: if NCC in plaintext over MAC CE is not agreeable, RAN2 is kindly asked to discuss the following options and make decisions:

- Option 1: contain NCC index rather than NCC in MAC CE

- Option 2: contain the delta value of the in-use NCC in MAC CE

Proposal 1-3: RAN2 is kindly asked to determine whether the new Rel19 ID is still used for the determination of security key update during inter-CU MCG LTM.

Failure handling:

Proposal 1-4: After inter-CU/intra-CU LTM cell switch failure (due to the expiry of T304), RAN2 is kindly asked to agree that the UE can perform the fast recovery if the selected LTM candidate cell is the intra-CU cell w.r.t. to the source cell (Case 1 & 4).

Proposal 1-5: After intra-CU LTM cell switch failure (due to the expiry of T304), RAN2 is kindly asked to agree that the UE performs RRC reestablishment procedure (not fast recovery) if the selected LTM candidate cell is the inter-CU candidate cell w.r.t. to the source cell (Case 3) .

Proposal 1-6: For inter-CU LTM cell switch failure (due to expiry of T304), RAN2 is kindly asked to discuss whether to support the fast failure recovery if the selected cell is intra-CU LTM candidate cell w.r.t. the target cell (Case 2).

Proposal 1-7: For RLF after the LTM cell switch, RAN2 is kindly asked to agree that the UE can perform the fast recovery if the selected LTM candidate cell is the intra-CU cell w.r.t. the target cell (Case 1 & 2).

Fast RRC processing

Proposal 1-8: RAN2 is kindly asked to agree that the number of serving cells configured for each LTM candidate configuration can be notified to the UE.

Proposal 1-9: RAN2 is kindly asked to agree that it is beneficial to let the source gNB know the number of serving cells configured for each LTM candidate configuration.

- DC

Inter-CU MCG LTM

Proposal 2-1: For inter-CU MCG LTM with unchanged SN or SCG addition, RAN2 is kindly asked to agree the followings:

- The reference configuration can include the SCG part

- During inter-CU MCG LTM preparation, the SN can determine whether to provide the reference configuration of SCG part to the MN or not.

References