3GPP TSG-RAN WG2 Meeting #129bis                                 R2-2502775

Wuhan, China, Apr. 7th- 11th, 2025

Agenda item:		8.2.3

Source:		Samsung

Title:			Discussion on A-IoT paging

Document for:	Discussion & Decision

- 1 Introduction

In RAN2#129, the AIoT paging was extensively discussed with the following agreements:

- Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

Agreements on paging ID

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

- Agreements - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

In this contribution, we will share our further views on the Paging design.

- 2 Discussion

Issue 1: Resource configuration information

The resource configuration is used by the device(s) to determine the resource used to transmit the Msg1 of random access procedure. In last meeting, RAN2 has reach the following agreements:

- the A-IoT paging message can include a number of msg1 resources

Since CFRA and CBRA can be supported, the resource configuration information in AIoT paging message should be indicated differently:

- CFRA case

In this case, the AIoT paging message should indicate the dedicated resource for Msg1 transmission of AIoT device, as shown in the following figure:

  - Option 1: immediately follow AIoT Paging

In this option, the dedicated resource always immediately follows the AIoT paging message, i.e., once the device receives the AIoT paging message with dedicated resource for such device, it can transmit the Msg1 immediately.

  - Option 2: resource with dedicated index

In this option, the AIoT paging message can indicate the assigned resource with a dedicated index. For example, given the R2D message contains the index of the resource for Msg1 transmission, if the index of the assigned dedicated resource for the device is same as the one in the R2D message, the device can start the Msg1 transmission.

Fig. 1 Dedicated resource indication for CFRA

- CBRA

In this case, the AIoT paging message should indicate a resource range so that each device can randomly select one of resource for Msg1 transmission. In RFID design, Q value is provided to the device so that the device can derive the range via 2^Q. Such design can be applied for AIoT. In addition, in order to have a flexible resource range indication, the AIoT paging message can also indicate the starting point of the allocated resource range.

To have a unified design for both case, the resource indication field in the AIoT paging message can have two field: 1) starting point, and 2) number of Msg1 resource (e.g., Q value), as shown in the following figure. The length field is optional, and it is present only for CBRA.

Fig. 2 Resource configuration information in AIoT paging message

Proposal 1-1: RAN2 is kindly asked to agree that the resource configuration in AIoT paging message contains the starting point of assigned resource, and optionally, the number of Msg1 resource.

Issue 2: Device behaviour with new serving request

- Support of the case

The post-email discussion addressed the case that the device may receive the new service request while one procedure is still ongoing. We need discuss whether such case can happen or not. Such case mainly happens for multi-reader scenario, where multiple AIoT services may be triggered by CN at different readers. In our understanding, such case is possible, and we don’t need to restrict CN behavior (i.e., don’t need to restrict that the CN only trigger one service at one reader at a time). However, RAN2 cannot make the decision on whether such case is supported, and SA2’s involvement is needed.

Proposal 2-1: RAN2 is kindly asked to consult SA2 on whether to support the case where multiple AIoT services are triggered at different readers at the same time (RAN2 has agreed that the parallel service requests by the same reader is not supported)

- Device behavior

In case of supporting this case, we need discuss the behavior when the device receives a new service request when one procedure is still ongoing, and the following options are given:

- Option 1: ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure

- Option 2: terminate the ongoing procedure and respond to the latest request

Regardless of which option to go, the device can only perform the procedure of one service request at a time.

Then, before making a choice between two options, we may need have a clear understanding on “the ongoing procedure”. The following behaviors can be considered the device is during a procedure.

  - The device recevies the AIoT paging message triggering the device’s response

  - The device is performing the CBRA, e.g., just transmit Msg 1 (not resolve the contention)

  - The device is perform the CFRA, e.g., transmit Msg1

  - The device completes RA procedure

  - The device is performing command procedure.

If the device just receives the AIoT paging messsage, the devices has the flexibility to prioritize any service. However, if the devices is performing command procedure, there is no need to drop the ongoing one. So, the clear understanding on the “ongoing procedure” can help determine whether the new request should be respected or not.

Proposal 2-2: RAN2 is kindly asked to agree that the device can only perform the procedure of one service request at a time.

Proposal 2-3: RAN2 is kindly asked to clarify the “ongoing procedure” first before making a choice on whether continuing the ongoing procedure when receiving a new request.

- Determination of ongoing procedure

The ongoing procedure is associated with one service request. The device can identify such service request based on the transaction ID in the AIoT paging message. Meanwhile, RAN2 agreed to have a R2D message (either new message or reusing paging message) to determine the Msg1 resource. In order to identify the R2D message associated with one service request, one possible way is to include the transaction ID in R2D message. Another way is to allow different readers indicate different Msg1 resource. For example, as shown in the following figure,

  - Reader 1 indicates the resource range for the Msg1 transmission is in (0, 127), while Reader 2 indicates the resource range for the Msg 1 transmission is in (128, 255);

  - Each R2D message includes the index of the access occasion. However, the index of access occasion from Reader 1 is in (0, 127), while the index of access occasion from Reader 2 is in (128, 255)

  - The device responsing to Reader 1 only choose the access occasions with index within (0, 127)

  - The device responsing to Reader 2 only choose the access occasions with index within (128, 255)

Fig. 3 Use the index of resource to identify the associated service

Proposal 2-4: RAN2 is kindly asked to discuss how to enhance the R2D message (used for the determination of Msg1 transmission) to assist device determine whether the received R2D message is associated with the ongoing procedure.

Issue 3: Device behaviour with same service request

This issue is related to the case where different readers send the Paging message for the same service request. An intuitive assumption is that the same service request result in the same transaction ID in the AIoT paging message. However, during the discussion, some companies mentioned a special service, i.e., location, where the same service requests the device to response to different readers. In order to support this case, we prefer to have unified device behavior, i.e., the device only response the same service request once. Then, the NW side (either CN or reader) can assign different transaction ID for the same service request.

Proposal 3-1: RAN2 is kindly asked to agree that the device only response the same service request once, and how to ensure the responses to different readers for the same service request is up to NW side decision (e.g., assign different transaction ID at different readers for the same service request).

On the other hand, the device may encounter another case that it receives different resource assignment from different readers for the same service request. For example, for the same service request, due to different resource status, one reader may assign CFRA resource for the device, while another reader may assign CBRA resource for the same device. At device side, it can prioritize the CFRA resource and provide the response

Proposal 3-2: RAN2 is kindly asked to discuss the device behavior if the device receives CFRA resource from one reader while it receives CBRA resource from another reader, w.r.t. the same service request.

Issue 4: Paging message update

Among the AIoT paging messages associated with the same CN request, the configuration infomration in each message may be changed. Fig. 1 shows one example on the resource configuration change for different AIoT paging messages. Specifically, the CN request is to inventory 100 devices. At the beginning, the AIoT reader may need assign a long period (i.e., period 1) to collect responses since large number of devices cause more collisions. After Period 1, 80 devices have been inventoried. Thus, the AIoT devices can assign a short period (i.e., period 2) for inventory considering the small number of remaining devices.

Fig. 4 Configuration information change among AIoT paging messages

Proposal 4-1: for the same service, RAN2 is kindly asked to agree that the A-IoT paging message may contain different configuration information (e.g., resource configuration).

- 3 Conclusion

Based on the discussion above, we have the following observations and proposals:

Resource configuration information

Proposal 1-1: RAN2 is kindly asked to agree that the resource configuration in AIoT paging message contains the starting point of assigned resource, and optionally, the number of Msg1 resource.

Device behaviour with new serving request

Proposal 2-1: RAN2 is kindly asked to consult SA2 on whether to support the case where multiple AIoT services are triggered at different readers at the same time (RAN2 has agreed that the parallel service requests by the same reader is not supported).

Proposal 2-2: RAN2 is kindly asked to agree that the device can only perform the procedure of one service request at a time.

Proposal 2-3: RAN2 is kindly asked to clarify the “ongoing procedure” first before making a choice on whether continuing the ongoing procedure when receiving a new request.

Proposal 2-4: RAN2 is kindly asked to discuss how to enhance the R2D message (used for the determination of Msg1 transmission) to assist device determine whether the received R2D message is associated with the ongoing procedure.

Device behaviour with same service request

Proposal 3-1: RAN2 is kindly asked to agree that the device only response the same service request once, and how to ensure the responses to different readers for the same service request is up to NW side decision (e.g., assign different transaction ID at different readers for the same service request).

Proposal 3-2: RAN2 is kindly asked to discuss the device behavior if the device receives CFRA resource from one reader while it receives CBRA resource from another reader, w.r.t. the same service request.

Paging message update

Proposal 4-1: for the same service, RAN2 is kindly asked to agree that the A-IoT paging message may contain different configuration information (e.g., resource configuration).