3GPP TSG-RAN WG2 Meeting #129bis					R2-2502788

Wuhan, China, 7 - 11 April, 2025

Title: 	Open issue list for running 36.331 CR for R19 SONMDT

Source: 	Huawei, HiSilicon

Agenda item:	8.10.1

Document for:	Discussion/Decision

- Introduction

After RAN2#129 [1], a post-meeting email discussion was agreed to progress on the running CR [2]. During the email discussion, some comments were made by companies.

This contribution is to list open issues for running 36.331 CR.

- Discussion

Per RA info list

At RAN2#127bis meeting, the following agreements were made.

- Add reporting of the following parameters for SCG failure report in EN-DC scenario:

⁻	For failedPSCellId and previousPSCellId: frequency and the PCI of the PSCell;

⁻	For timeSCGFailure: value range 0-1023;

⁻	For failureType: Reuse the legacy field.

⁻	perRA-InfoList

Regarding perRA-InfoList, we reference TS 38.331 definitions. In TS 38.331 v18.4.0, there are PerRAInfoList-r16, PerRAInfoList-v1660 (introduced in [3]), and PerRAInfoList-v1800 (introduced in [4]) defined in SCGFailureInformation. Similarly, it is suggested to also include 3 containers for them.

An example is shown as below:

****************************************************

- –	SCGFailureInformationNR

The SCGFailureInformationNR message is used to provide information regarding NR SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

SCGFailureInformationNR message

-- ASN1START

SCGFailureInformationNR-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

scgFailureInformationNR-r15			SCGFailureInformationNR-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCGFailureInformationNR-r15-IEs ::=	SEQUENCE {

failureReportSCG-NR-r15				FailureReportSCG-NR-r15				OPTIONAL,

nonCriticalExtension					SCGFailureInformationNR-v1590-IEs	OPTIONAL

}

SCGFailureInformationNR-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension					OCTET STRING					OPTIONAL,

nonCriticalExtension						SEQUENCE {}					OPTIONAL

}

FailureReportSCG-NR-r15 ::=		SEQUENCE {

failureType-r15						ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-reconfigFailure,

srb3-IntegrityFailure, dummy},

measResultFreqListNR-r15				MeasResultFreqListFailNR-r15		OPTIONAL,

measResultSCG-r15						OCTET STRING						OPTIONAL,

...,

[[	locationInfo-r16				LocationInfo-r10						OPTIONAL,

logMeasResultListBT-r16			LogMeasResultListBT-r15					OPTIONAL,

logMeasResultListWLAN-r16		LogMeasResultListWLAN-r15				OPTIONAL,

failureType-v1610				ENUMERATED {t312-Expiry, scg-lbtFailure,

beamFailureRecoveryFailure, bh-RLF-r16,

beamFailure-r17,

spare3, spare2, spare1}	OPTIONAL

]],

[[

previousPSCellId-r19			SEQUENCE {

physCellId-r19                       PhysCellIdNR-r15,

carrierFreq-r19                      ARFCN-ValueNR-r15

}                                                    		OPTIONAL,

failedPSCellId-r19				SEQUENCE {

physCellId-r19                       PhysCellIdNR-r15,

carrierFreq-r19                      ARFCN-ValueNR-r15

}                                                    		OPTIONAL,

timeSCGFailure-r19                   INTEGER (0..1023)        OPTIONAL,

perRAInfoListNR-r19				SEQUENCE {

perRAInfoListNR				OCTET STRING			OPTIONAL,

perRAInfoList-v1660-NR			OCTET STRING		OPTIONAL,

perRAInfoList-v1800-NR			OCTET STRING		OPTIONAL

}

]]

}

MeasResultFreqListFailNR-r15 ::=	SEQUENCE (SIZE (1..maxFreqNR-r15)) OF MeasResultFreqFailNR-r15

MeasResultFreqFailNR-r15 ::=		SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

measResultCellList-r15				MeasResultCellListNR-r15			OPTIONAL,

...

}

-- ASN1STOP

perRAInfoListNR

This field is used to indicate per RA information for NR RACH. The perRAInfoListNR IE includes PerRAInfoList-r16, and the perRAInfoList-v1660-NR IE includes PerRAInfoList-v1660, and the perRAInfoList-v1800-NR includes PerRAInfoList-v1800, which are specified in TS 38.331 [82].

****************************************************

Proposal 1: For ASN.1 definitions in TS 36.331, it is proposed to introduce the perRAInfoListNR IE, and it includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 (which are specified in TS 38.331).

For RRCReconfiguration condition

During the email discussion, the following text was discussed.

2>	else:

3>	set the failedPSCellId to the physical cell identity and carrier frequency of the PSCell in which the SCG failure was declared;

3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared:

4>	set the timeSCGFailure to the elapsed time since the last execution of RRCReconfiguration message including the reconfigurationWithSync for the SCG until declaring the SCG failure;

4>	set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last received RRCReconfiguration message including reconfigurationWithSync for the SCG;

For the above highlighted part, one company would like to further discuss it.

Samsung (Aby): Woundn’t this caluse in 3> will be always true? For both SCG addition and change, RRCReconfiguration message will include reconfigurationWithSync for the SCG. It looks to me that this part seems to be intended only PSCellChange. First it needs to be clarified, if timeSinceSCGFailure needs to be set for PSCellAddition for these cases. If yes, this 3> can be removed and 4 should be 4>set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last received RRCReconfiguration message including reconfigurationWithSync, if available; If no, We may need to update 3> as below: 3> if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared from another PSCell: I think it will need some discussion.

For the above changes, they are following the definitions from TS 38.331, and NR introduced this text since Rel-17, i.e. R2-2204209. Furthermore, the changes were made based on the RAN3 requirement in the LS R3-211332, and the applicable use case is PScell change failure for MRO.

The CR rapporteur thinks that the condition "RRCReconfiguration message including the reconfigurationWithSync for the SCG" may not always happen before SCG failure happens, e.g. after the UE accesses the network, the MCG and SCG are configured by the network, and then an SCG failure occurs. So this condition is important and it should be kept.

Proposal 2: It is proposed to keep the condition for UE setting timeSCGFailure and previousPSCellId, i.e. 3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared.

- Conclusion

In this paper, we provide our analysis on two open issues identified when discussing running 36.331 CR, and it is proposed:

Proposal 1: For ASN.1 definitions in TS 36.331, it is proposed to introduce the perRAInfoListNR IE, and it includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 (which are specified in TS 38.331).

Proposal 2: It is proposed to keep the condition for UE setting timeSCGFailure and previousPSCellId, i.e. 3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared.

- References

Draft_RAN2_129_Meeting_Report_v1

R2-2502787, Running 36.331 CR for R19 SONMDT, Huawei, HiSilicon

R2-2108968, SON-MDT Changes agreed in RAN2#115 meeting, Ericsson, Huawei

R2-2313855, CR to 38331 for introducing SON/MDT features in Rel-18, Ericsson, Huawei, ZTE