3GPP TSG-RAN WG2 Meeting #129bis		R2-2502808

Wuhan, China, Apr 7 – 11, 2025	Revision of R2-2501214

Agenda Item:	8.10.4

Source: 	LG Electronics

Title:  	NTN Logging for Unchanged PCI Mobility

Document for:	Discussion

Introduction

In Rel-18 NTN, there is a mobility procedure from a source satellite to a target satellite that the PCI does not change. In this case, the UE considers the source satellite to be changed to the target satellite at the time (i.e. tService), based on the timing configuration provided by SIB 19 for this unchanged PCI mobility.

In this contribution, we discuss logging for unchanged PCI mobility in NTN. We would like to consider the failure scenario and near failure scenario related to the unchanged PCI mobility procedure.

Discussion

When initiating this unchanged PCI mobility procedure, the UE just (re-)synchronizes the Layer 1 DL/UL synchronization while maintaining the current cell configuration. The time information (i.e. tService) is provided to the UE via SIB19, and if another time information (i.e. tServiceStart) is included, the UE can softly re-synchronize with the target satellite. The softly re-synchronizing involves performing DL sync with the target satellite between the tServiceStart time and the tService time (i.e. early DL synchronization with the target satellite). The detailed procedure is specified below as like in 5.7.19 (Satellite switch with resynchronization) of [1].

5.7.19 Satellite switch with resynchronization A UE capable of hard satellite switch with resynchronization in RRC_CONNECTED initiates the procedure when SatSwitchWithReSync and t-Service are included in SIB19. Upon initiating the procedure, the UE shall: 1> if t-ServiceStart is included in SIB19 and the UE supports soft satellite switch with resynchronization: 2> start acquiring DL synchronization with the SpCell served by the satellite indicated by ntn-Config in SatSwitchWithReSync between the time indicated by t-ServiceStart and the time indicated by t-Service for the serving cell; 1> upon the time indicated by t-Service: 2> stop timer T430 if running; 2> inform lower layers that UL synchronisation is lost due to satellite switch with resynchronization; 2> synchronise to the DL of the SpCell served by the satellite indicated by ntn-Config in SatSwitchWithReSync, if the UE has not previously synchronized to the DL of the SpCell; 2> start timer T430 with the timer value set to ntn-UlSyncValidityDuration from the subframe indicated by epochTime in ntn-Config in SatSwitchWithReSync; 2> inform lower layers when UL synchronisation is obtained. NOTE: UE should attempt to re-acquire SIB19 after satellite switch with resynchronization. The exact time is left to UE implementation.

First, let's consider the failure scenario of the unchanged PCI mobility.

If the UE initiates the L1 synchronization procedure at tServiceStart or tService time and the L1 synchronization fails, out-of-sync indications will be sent from UE L1 to UE L3. This procedure does not require a mobility validity timer like T304, but the out-of-sync indication makes the UE to declare a radio link failure. Thus, if the unchanged PCI mobility fails, the UE can use the RLF report to log and report the results related to this mobility. Adding unchanged PCI mobility failure as a new RLF cause would be a simple addition for this scenario.

Proposal 1: For unchanged PCI satellite mobility, the UE uses the RLF report, including a new cause to indicate a failure of the unchanged PCI satellite mobility if RLF is declared due to this mobility.

Next, let’s consider the near failure scenario of the unchanged PCI mobility.

The scenario is one where the UE receives some out-of-sync indications from UE L1 while performing the L1 synchronization procedure, but the UE completes the L1 synchronization procedure successfully. In other words, the UE detects the out-of-sync indications but does not declare a radio link failure, and the L1 synchronization procedure is completed somewhat unstably.

The problem in this case is that the UE cannot include this unstable information in any of the existing reports to the network, so the network remains unaware of this fact.

The out-of-sync indication can occur for various reasons. However, if the time information is incorrectly configured due to mis-coordination between the source satellite and the target satellite, there may be a case where the UE succeeded the L1 synchronization with some out-of-sync indications. Due to this reason, it would be beneficial for the network to optimize the time configuration for the mobility procedure from a source satellite to a target satellite that the PCI does not change by the UE reporting the out-of-sync indication even in the success case of the unchanged PCI mobility.

Based on the above reason, we propose the followings:

Proposal 2: For unchanged PCI satellite mobility, the UE reports the near failure information that out-of-sync indication(s) has been detected during the mobility if the UE detects the out-of-sync indications during the unchanged PCI satellite mobility but successfully complete this mobility.

If RAN2 agrees with Proposal 2, it is essential for RAN2 to consider the appropriate granularity for declaring a near failure.

Similar to how the existing T304 timer uses a threshold-based delay to determine near failure, it is crucial not to declare a near failure every time an out-of-sync indication is received from the lower layer. Instead, the UE should consider the near failure only when the UE RRC receives a significant number of out-of-sync indications to almost declare an RLF. This approach will be more helpful for the network to prevent potential failures and perform appropriate optimizations. If the UE reports a near failure every time an out-of-sync indication occurs, the network will receive numerous information for the optimization even where an RLF is clearly not declared. This could lead to inappropriate optimizations by the network.

Therefore, we propose that a new logging condition is needed for this near failure case, and logging should be performed when a significant number of out-of-sync indications are received to be considered a near failure.

Proposal 3: To determine a proper near failure of the unchanged PCI satellite mobility, a new logging condition is considered based on reception of a significant number of out-of-sync indications during the mobility.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1: For unchanged PCI satellite mobility, the UE uses the RLF report, including a new cause to indicate a failure of the unchanged PCI satellite mobility if RLF is declared due to this mobility.

Proposal 2: For unchanged PCI satellite mobility, the UE reports the near failure information that out-of-sync indication(s) has been detected during the mobility if the UE detects the out-of-sync indications during the unchanged PCI satellite mobility but successfully complete this mobility.

Proposal 3: To determine a proper near failure of the unchanged PCI satellite mobility, a new logging condition is considered based on reception of a significant number of out-of-sync indications during the mobility.

References

- 3GPP TS 38.331 V18.2.0 (2024-06)