3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502809

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.19

Source:	Huawei, HiSilicon

Title:	RAN2 impacts for 7 MHz Channel Bandwidth

Document for: Discussion and Decision

1	Introduction

In RAN#106, a new work item to introduce 7MHz channel bandwidth for band n5 and n26 was approved [1] with the following RAN2 related objectives:

- Enable, if feasible, optional UE support of 7 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4] - No impact to 38.307 is expected. - Once RAN4 has introduced the new channel bandwidth, RAN2 will introduce the necessary signalling for this channel bandwidth. [RAN2] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN2 will be informed by RAN4 via LS. No additional RAN2 TUs will be requested for this objective.

In the last meeting, RAN4 sent an LS to RAN2 about 7MHz Channel Bandwidth:

1. Overall Description: RAN4 would like to inform RAN2 on the progress made on the topic of release independence for the 7 MHz channel bandwidth. RAN4 has noted that TS 38.307 says that “NR channel bandwidths, all unless otherwise stated” are release independent from Release 15. RAN4 understands that there were some unique characteristics of 3 MHz channel bandwidths including new RAN1 requirements and signalling related to support for the punctured SSB that prevented the 3 MHz channel bandwidth from being release independent from Rel-15. RAN4 has discussed release independence of the 7 MHz channel bandwidth and have decided that there is no reason from a RAN4 perspective that 7 MHz cannot be release independent from Release 15 for a band with a minimum channel bandwidth of 3MHz or 5MHz. RAN4 also notes that 35 and 45 MHz channel bandwidths were introduced in a Release 17 Work Item, but are release independent from Release 15, including the use of spare bits in 38.306 Release 15. RAN4 believes that adding 7 MHz should be similar to adding 35 and 45 MHz from signaling design aspect. RAN4 understands that maximum and minimum channel bandwidths can be reported for band combinations. For band combinations that include a band or bands that support 7 MHz, we think it is important to note that 7 MHz is unlikely to be the minimum or maximum channel bandwidth that a UE supports for a band in a given band combination. We think it would be fine to add support for 7 MHz as a possible maximum or minimum supported bandwidth in a release later than Rel-15, as 35 and 45 MHz were added in SupportedBandwidth-v1700, while the bandwidths themselves are release independent from Rel-15. RAN4 respectfully requests that RAN2 consider the signalling aspects related to adding the 7 MHz channel bandwidth, and to inform RAN4 if RAN2 sees any reason from a signalling perspective why the 7 MHz channel bandwidth cannot be release independent from Release 15. 2. Actions: To RAN2 ACTION: RAN4 respectfully requests that RAN2 consider the signalling aspects related to adding the 7 MHz channel bandwidth, and to inform RAN4 if RAN2 sees any reason from a signalling perspective why the 7 MHz channel bandwidth cannot be release independent from Release 15.

In this contribution, we will analyse the RAN2 impacts to support 7 MHz Channel Bandwidth, and discuss whether the 7MHz Channel Bandwidth can be release independent from Release 15.

2	Discussion

## 2.1 RAN2 signalling to support 7MHz Channel Bandwidth

The 7MHz channel bandwidth, which is commonly called "irregular channel bandwidth”, is introduced to facilitate more efficient utilization of licensed spectrum that does not conform to the existing NR channel bandwidths. From signalling perspective, we think RAN2 should make sure that the 7MHz channel bandwidth can be configured by NW and UE can report whether 7MHz channel bandwidth is supported.

Configuration for 7MHz Channel Bandwidth

Currently, the carrierBandwidth IE is used to configure the channel bandwidth of the cell, which is encoded in number of PRBs. And the maximum number of PRBs that can be indicated is 275. Since 7MHz Channel Bandwidth can be configured as 35 PRBs, it is straightforward to reuse carrierBandwidth in SIB1 for configuration of 7MHz channel bandwidth .

SCS-SpecificCarrier information element

-- ASN1START

-- TAG-SCS-SPECIFICCARRIER-START

SCS-SpecificCarrier ::=             SEQUENCE {

offsetToCarrier                     INTEGER (0..2199),

subcarrierSpacing                   SubcarrierSpacing,

carrierBandwidth                    INTEGER (1..maxNrofPhysicalResourceBlocks),

...,

[[

txDirectCurrentLocation         INTEGER (0..4095)                                       OPTIONAL -- Need S

]]

}

-- TAG-SCS-SPECIFICCARRIER-STOP

-- ASN1STOP

SCS-SpecificCarrier field descriptions

carrierBandwidth Width of this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier) (see TS 38.211 [16], clause 4.4.2).

Proposal 1: RAN2 confirms that the current carrierBandwidth in SIB1 can be reused to configure the 7MHz Channel Bandwidth as 35 PRBs.

UE capabilities for 7MHz Channel Bandwidth

In Rel-16 and Rel-17, when introducing the support of 35MHz, 45MHz and 100MHz, RAN2 agreed to use the spare bits in channelBWs-DL and channelBWs-UL reflect this:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| channelBWs-DL … For FR1, the leading/leftmost bit in channelBWs-DL-v1590 indicates 70MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz, the fourth leftmost bit indicates 100MHz and all the remaining bits in channelBWs-DL-v1590 shall be set to 0. … | Band | No | N/A | N/A |
| channelBWs-UL … For FR1, the leading/leftmost bit in channelBWs-UL-v1590 indicates 70 MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz, the fourth leftmost bit indicates 100MHz and all the remaining bits in channelBWs-UL-v1590 shall be set to 0. … | Band | No | N/A | N/A |

We think the support of 7MHz may follow the same principle and one of the spare bits can be used to indicate 7MHz in channelBWs-DL and channelBWs-UL.

Proposal 2: The spare bits in channelBWs-DL IE and channelBWs-UL IE may be used to indicate UE support of 7MHz channel bandwidth.

In addition, RAN4 LS also mentioned the capability in CA for 7MHz:

RAN4 understands that maximum and minimum channel bandwidths can be reported for band combinations. For band combinations that include a band or bands that support 7 MHz, we think it is important to note that 7 MHz is unlikely to be the minimum or maximum channel bandwidth that a UE supports for a band in a given band combination. We think it would be fine to add support for 7 MHz as a possible maximum or minimum supported bandwidth in a release later than Rel-15, as 35 and 45 MHz were added in SupportedBandwidth-v1700, while the bandwidths themselves are release independent from Rel-15.

We understand this capability is similar to what was added for 35 and 45 MHz in SupportedBandwidth-v1700 , which is FSPC level. And we should design two Rel-19 IEs for this capability, e.g., supportedBandwidthUL-v1900 and supportedBandwidthDL-v1900.

Proposal 3: New Rel-19 IEs may be needed to support the 7MHz as one “minimum or maximum channel bandwidth” in CA for the UE to report.

Regarding the RAN4 understanding that there is no reason that 7 MHz cannot be release independent from Release 15 for a band with a minimum channel bandwidth of 3MHz or 5MHz, we noticed there were some discussions on whether this is release independent for all cases, e.g., for a band not with a minimum channel bandwidth of 3MHz or 5MHz. Since the WI is supposed to be finished in August, 2025, to avoid any unnecessary inter-WGs interactions in case there is any different understandings, we’d better wait for more RAN4 progress to finalize the CRs.

Proposal 4: Wait for further RAN4 progress before finalizing the signaling design in RAN2.

# 3	Conclusion

Based on the above discussion we have the following proposals:

Configuration

Proposal 1: RAN2 confirms that the current carrierBandwidth in SIB1 can be reused to configure the 7MHz Channel Bandwidth as 35 PRBs.

Capability

Proposal 2: The spare bits in channelBWs-DL IE and channelBWs-UL IE may be used to indicate UE support of 7MHz channel bandwidth.

Proposal 3: New Rel-19 IEs may be needed to support the 7MHz as one “minimum or maximum channel bandwidth” in CA for the UE to report.

Proposal 4: Wait for further RAN4 progress before finalizing the signaling design in RAN2.

4	Reference

- RP-243323, “New WID on 7 MHz Channel Bandwidth for n26 and n5”, China Telecom, T-Mobile USA

- R4-2503017, LS on Signalling for 7 MHz Channel Bandwidth, from RAN4.