3GPP TSG-RAN WG2 Meeting #129bis		       R2-2502818

Wuhan, China: 7th April – 11th April 2025

Agenda Item:	8.1.4

Source:	ASUSTeK

Title:	Discussion on LCM for UE-sided data collection

Document for:	Discussion and Decision

- Introduction

The WI NR_AIML_air agreed to work on the signalling mechanism of data collection for UE-sided models. We would like to address some of the issues related to the data collection for UE-sided models in this document.

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models

- Discussion

Agreements (RAN2 #128) 1. For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration. 2. For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains. 3. The network can provide the data collection configuration (at any point in time), with or without UE request. 4. The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection. 5. FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

In the previous meetings, RAN2 agreed that the UE can send a request for data collection. We would like to discuss what the request contains.

In our opinion, there are two aspects where the UE may request for data collection.

- Aspect 1: an AI/ML capable UE does not have any AI/ML model (e.g., new UE), but is ready to train and perform inference.

- Aspect 2: an AI/ML capable UE received some configurations, and decide to finetune an already available model or train a new model based on the received configurations.

For example, the UE may receive an inference configuration from the NW, but is inapplicable due to lack of AI/ML model. The UE may want to collect data and train a model based on the received inference configuration.

On the other hand, without the NW providing configurations, the UE may not be aware of several cell-specific features (e.g., the RS indices). In this case, the UE may provide additional assistance information such as periodicity, number of RS resources to the NW.

Proposal 1: The UE can provide additional assistance information in the request (e.g., periodicity, number of RS resources).

Proposal 2: The UE can indicate a configuration (e.g., inference configuration) in the request.

Another functionality for the request is to start/stop the data collection. When there are multiple ongoing data collections, the UE should be able to inform the NW about the completion of the collection, without affecting other ongoing data collections.

Proposal 3: The granularity for requesting start/stop of data collection is at least per configuration.

- Conclusion

Proposal 1: The UE can provide additional assistance information in the request (e.g., periodicity, number of RS resources).

Proposal 2: The UE can indicate a configuration (e.g., inference configuration) in the request.

Proposal 3: The granularity for requesting start/stop of data collection is at least per configuration.

References

[1] RP-240774, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[2] Chair Notes for RAN2 #128

Annex